3GPP TSG-RAN4 Meeting #101bis-e		R4-2202010

# Electronic Meeting, January 17-25, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurement requirements for Pre-MG in TS 38.133 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-Core |  |  |  |  |  | Date: |  |  | 2022-01-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Specify rules for pre-MG status |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defines rules to determine the pre-MG status autonomously or based on RRC indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE measurement performed using Pre-MG patterns cannot be guanranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 9.1.2A, 9.1.2A.1, 9.1.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [11] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [11].

AoA	Angle of Arrival

AoD	Angle of Departure

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BLER	Block Error Rate

BM-RS	Beam Management Reference Signal

BWP	Bandwidth Part

CA	Carrier Aggregation

CBD	Candidate Beam Detection

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

CSI_RP	Received (linear) average power of the resource elements that carry NR CSI-RS signals and channels, measured at the UE antenna connector

DBT	Discovery Burst Transmission

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA-NR Dual Connectivity

FDD	Frequency Division Duplex

FR	Frequency Range

HARQ	Hybrid Automatic Repeat Request

HO	Handover

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

L1 SL-RSRP	Layer 1 Sidelink RSRP which corresponds to PSCCH-RSRP and/or PSSCH-RSRP

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR-DC	NR-NR Dual Connectivity

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

Pre-MG	Pre-configured Measurenent Gap pattern

PRP	PRS Received Power

PRS	Positioning Reference Signal

PRS-RSRP	Positioning Reference Signal based Reference Signal Received Power

PSBCH	Physical Sidelink Broadcast Channel

PSBCH-RSRP	Physical Sidelink Broadcast Channel DMRS based Reference Signal Received Power

PSCCH	Physical Sidelink Control Channel

PSCCH-RSRP	Physical Sidelink Control Channel DMRS based Reference Signal Received Power

PSCell	Primary SCell

PSS	Primary Synchronization Signal

PSSCH	Physical Sidelink Shared Channel

PSSCH-RSRP	Physical Sidelink Shared Channel DMRS based Reference Signal Received Power

pTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRM	Radio Resource Management

RSSI	Received Signal Strength Indicator

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

S-SSB	Sidelink Synchronization Signal Block

S-SSB_RP	Received (linear) average power of the resource elements that carry NR S-SSB signals and channels, measured at the UE antenna connector

SA	Standalone operation mode

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SL-RSSI	Sidelink Received Signal Strength Indicator

SLSS	Sidelink Synchronization Signal

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TDD	Time Division Duplex

TDOA	Time Difference Of Arrival

TRP	Transmission-Reception Point

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

----------------------NEXT CHANGE----------------------------

### 9.1.2A	Pre-configured measurement gap

#### 9.1.2A.1	Introduction

The UE capable of Pre-configured measurement gap (Pre-MG) pattern can be configured with a Pre-MG pattern via RRC signalling [2].

#### 9.1.2A.2	Requirements

The requirements for Pre-MG pattern are applicable provided that the UE is configured with:

- PCell or

- PCell and one or more SCells

Any of the measurement gap pattern defined in Table 9.1.2-1 can be configured as Pre-MG pattern.

The requirements for Pre-MG with activation/deactivation when configured with CA is only applicable for active BWP switching on a single CC.

The UE capable of autonomous activation/deactivation mechanism [1] can autonomously change the Pre-MG status from activation to deactivation or vice versa based on any of the following triggering conditions:

- DCI or timer based active BWP switching,

- Adding/removing any measurement object(s): FFS

- Adding/releasing/changing SCell(s): FFS

- Activating/de-activating any Scell(s): FFS

- LPP positioning request: FFS

The UE shall autonomously deactivate the activated Pre-MG pattern or shall assume the Pre-MG status as deactivated immediately after the configuration of the Pre-MG pattern provided any of the following conditions is met:

- The UE is configured with only PCell and with only SSB based measurements, and the measured SSB is completely contained in the active BWP.

- The UE is configured with per UE-Pre-MG, PCell, one or more SCells and only SSB based measurements, and the measured SSBs of all the CCs are completely contained in their respective active BWPs.

- The UE is configured with per FR-Pre-MG, PCell, one or more SCells and only SSB based measurements, and the measured SSBs of all the CCs in the same FR are completely contained in their respective active BWP.

The UE shall autonomously activate the deactivated Pre-MG pattern or shall assume the Pre-MG status as activated immediately after the configuration of the Pre-MG pattern provided any of the following conditions is met:

- The UE is configured with only PCell and with only SSB based measurements, and the measured SSB is not fully contained in the active BWP.

- The UE is configured with per UE-Pre-MG, PCell, one or more SCells and only SSB based measurements, and the measured SSB of at least one CC is not fully contained in its respective active BWP.

- The UE is configured with per FR-Pre-MG, PCell, one or more SCells and only SSB based measurements, and the measured SSB of at least one CC in the same FR is not fully contained in its respective active BWP.

- The UE is configured with at least:

  - CSI-RS based measurements, or

  - NR PRS-based positioning measurements, or

  - E-UTRA Inter-RAT measurements, or

  - E-UTRA Inter-RAT RSTD and E-CID measurements, or

  - UTRA Inter-RAT measurements, or

  - GSM Inter-RAT measurements.

The UE shall autonomously activate or deactivate the Pre-MG pattern within the Pre-MG activation/deactivation delay (Tstatus_change) starting from the moment the active BWP switch causing the Pre-MG status change is triggered:

Tstatus_change = TBWP_switch + T

Where:

- TBWP_switch is the DCI or timer based active BWP switch delay for single CC defined in clause 8.6.2 or for multiple CCs defined in clause 8.6.2A for DCI based active BWP switch and in clause 8.6.2B for timer based active BWP switch.

- T is the additional transition time. T can be up to FFS.

The UE capable of RRC based activation/deactivation mechanism can change the Pre-MG status from activation to deactivation or vice versa based on the received RRC indication [1].

The UE shall deactivate the Pre-MG pattern based on RRC indication when any of the following condition is met:

- The UE is configured with only PCell and the RRC indication is ‘deactivation’.

- FFS: The UE is configured with with per UE-Pre-MG, PCell, one or more SCells, and the RRC indication is FFS.

- FFS: The UE is configured with with per FR-Pre-MG, PCell, one or more SCells, and the RRC indication is FFS.

The UE shall activate the Pre-MG pattern based on RRC indication when any of the following condition is met:

- The UE is configured with only PCell and the RRC indication is ‘activation’.

- FFS: The UE is configured with with per UE-Pre-MG, PCell, one or more SCells, and the RRC indication is FFS.

- FFS: The UE is configured with with per FR-Pre-MG, PCell, one or more SCells, and the RRC indication is FFS.

----------------------END OF CHANGES----------------------------