[]3GPP TSG-RAN WG4 Meeting # 101-bis-e 	R4-2202771

# Electronic Meeting, Jan. 17-25, 2022

Title:	[DRAFT] ReplyLS on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap

Response to:	R1-2112762

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact Person:

Name:	Yanliang SUN

E-mail Address:	 yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #101-bis-e meetings, RAN4 has discussed RAN1 LS R1-2112762 on L1-RSRP measurement behaviour when SSBs associated with different PCIs overlap. Based on discussion RAN4 would like to provide the following feedback.

RAN4 will specify RRM requirements based on UE measurement behavior assumptions for L1-RSRP when SSBs associated with serving cell and cells different PCIs overlap as:

In FR1 –

- If SSBs from cells with different PCIs are measured outside SMTCs, RRM requirements are only applicable to all UEs for the case when

  - timing difference among serving cell and cells with different PCIs is less than CP

- FFS: If SSBs from cells with different PCIs are measured within SMTCs

In FR2 –

- RAN4 has agreed to assume one active UE panel for the RRM requirements

- RAN4 at least assumes the number of non-serving cell Nmax = 1 based on RAN1 agreement.

  - FFS : Nmax  >  1

- In case the SSBs from cells with different PCI are fully overlapped with SMTC:

  - FFS: UE measurement behavior assumptions as that for legacy L3 measurements requirements, including Rx beam assumptions, are re-used

- In case the SSBs from cells with different PCI are not fully overlapped with SMTC:

  - UE behavior assumptions for this case is still FFS

For FR2, RAN4 continues discussion on simultaneous reception and measurement on L1-RSRP over the overlapping SSBs based on two active UE panels.

- FFS: RAN4 assumes to reuse simultaneousReceptionDiffTypeD-r16

2. To RAN WG1 group.

ACTION: 	RAN4 respectfully asks RAN1 to take RAN4 conclusions in consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #102e	Feb. 21 - Mar. 03	Online