3GPP TSG-RAN WG4 Meeting # 101-bis-e                                          R4-2203019

# Electronic Meeting, January 17-25, 2022

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on timing enhancement for eIAB

Agenda Item:	6.17.1

Document for:	Approval

# Background

Previous RAN4#101-e meeting agreed the WF [R4-2120679] that included:

- RF requirement impact due to TX power imbalance between IAB-MT and IAB-DU

on timing error between own MT TX and DU TX:

- Case#6 timing: Timing error between own MT TX and DU TX

This Way forward contains the agreement and issues for further discussion on RF impact due to Rel-17 IAB enhancement based on 1st round summary in [2].

# Way forward

Timing error between intra-node MT TX and DU TX for case#6

Candidate options:

- Option 1: To specify TAE between IAB-MT and IAB-DU in timing case #6

    - The requirement value is min (3us , 4.69 / (SCS/15 kHz) µs).

- Option 2: No TAE between IAB-MT and IAB-DU

Agreement:

- RAN4 will make final decision on February RAN4 meeting.

Timing error between parent IAB DU and Child IAB for case#6

Agreement:

- This is out of RF scope. No specific requirements will be specified from RF requirements aspect.

# Reference

[1] R4-2120679, WF on specification impact for Rel-17 eIAB, Samsung

[2] R4-2202974, Email discussion summary for [101-bis-e][310] NR_eIAB