3GPP TSG-RAN WG4 Meeting #101-e	R4-2117185

Electronic Meeting, November 01-12, 2021

Source:	China Telecom

Title:	On phase continuity and power consistency tolerance

Agenda Item:	8.18.2.2

Document for:	Discussion

- Introduction

In RAN4 #100e meeting, one more reply LS on PUCCH and PUSCH transmission was sent to RAN1 [1], and the WF on phase continuity and power consistency was approved in [2].

This contribution provides our views on phase continuity and power consistency tolerance.

- Discussion

2.1  Phase continuity tolerance

Firstly, we’d like to discuss the model of phase variation. Two options were listed in the WF:

Model of phase variation

- Down select Model of explicit phase offset between option 1 and option 2

  - Option 1: Gaussian, std deviation

  - Option 2: uniform distribution

- Definition of the offset

  - For each individual slot (0…n) within the bundle, an independent offset is generated and applied.

For the model of phase offset, we support the option 2 of uniform distribution. With uniform distribution model in the simulation, the derived phase offset can be referred to define the corresponding UE requirements, i.e., to define the maximum allowable phase offset based on the upper limit of the uniform distribution. In contrast, if we use Gaussian distribution, it is not straightforward to derive the UE requirements based on the standard deviation.

Proposal 1: Use option 2 of uniform distribution for the phase offset model, since it can be referred to define UE requirements in terms of maximum allowed phase offset.

For the above definition of the offset, in our understanding, it means that an independent offset is generated and applied for each individual slot with respect to the slot 0. So we’d like to clarify the definition of the offset as:

- Definition of the offset

  - For each individual slot (0…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

Proposal 2: Clarify the definition of the offset as:

- Definition of the offset

  - For each individual slot (0…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

For the phase continuity tolerance, the following options were captured in the WF:

Phase continuity tolerance

- On phase continuity tolerance initial thoughts based on simulation results

  - 1: within 40 degrees based on simulation for PUCCH

  - 2: within 30 degrees based on simulation for PUSCH QPSK

  - 3:

    - For PUSCH QPSK, in the order of 20 degrees for 8 repetitions

    - For PUCCH format 3, 40 degree’s STD for 2 repetition and 20 degree’s STD for 8 repetition

  - Align on simulation assumption, decide the phase tolerance in next RAN4 meeting, key factors for alignment are as follow

    - Physical channel: PUSCH and PUCCH

    - Repetition number: 2/8

    - Model of phase variation: in issue 1-4-2

    - DMRS configuration per slot

    - Modulation order: focus on Pi/2 BPSK (PUCCH/PUSCH), QPSK (PUCCH/PUSCH), BPSK (PUCCH)

In the following, we will provide link-level simulation results with different phase offsets, using the simulation assumptions listed in Table 1.

Table 1. Simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency and duplex mode | 4GHz, FDD |
| Physical channel | PUSCH |
| UE velocity | 3km/h |
| TBS | 320 (VoIP) |
| MCS | 4 |
| RB number | 4 |
| Repetition number | 2 and 8 |
| UE Tx number | 1 |
| BS Rx number | 2 and 8 |
| Model of phase variation for JCE | Uniform distribution within ±{0, 10, 20, 30, 40, 50, 60} degrees • For each individual slot (0…n) within the bundle, an independent offset is generated and applied with respect to the slot 0. |

Figure 1. Results for 2Rx and 2 repetitions        Figure 2. Results for 2Rx and 8 repetitions

Figure 3. Results for 8Rx and 2 repetitions

Table 2. Required SINR (dB) at 2% BLER

|  | Without JCE | With JCE, with different phase offsets |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 degree | 10 degrees | 20 degrees | 30 degrees | 40 degrees | 50 degrees | 60 degrees |
| 2Rx, 2 repetitions | 3.95 | 2.21 | 2.29 | 2.40 | 2.63 | 2.76 | 2.88 | 3.10 |
| 2Rx, 8 repetitions | -0.27 | -4.75 | -4.37 | -4.23 | -4.08 | -3.79 | -3.40 | -3.03 |
| 8Rx, 2 repetitions | -6.03 | -7.58 | -7.54 | -7.48 | -7.39 | -7.25 | -7.16 | -7.05 |

Table 3. JCE performance degradation (dB) due to phase offset

|  | Without JCE | With JCE, with different phase offsets |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 degree | 10 degrees | 20 degrees | 30 degrees | 40 degrees | 50 degrees | 60 degrees |
| 2Rx, 2 repetitions | N.A. | Baseline | -0.08 | -0.19 | -0.42 | -0.55 | -0.67 | -0.89 |
| 2Rx, 8 repetitions | N.A. | Baseline | -0.38 | -0.52 | -0.67 | -0.96 | -1.35 | -1.72 |
| 8Rx, 2 repetitions | N.A. | Baseline | -0.04 | -0.10 | -0.19 | -0.33 | -0.42 | -0.53 |

We have the following observations based on our simulation results:

Observation 1: When 20 degrees of phase offset with uniform distribution is modeled, the JCE performance degradation compared to no phase offset is -0.19 dB, -0.52 dB, -0.10 dB for 2Rx + 2 repetitions, 2Rx + 8 repetitions and 8Rx + 2 repetitions respectively.

Observation 2: When 30 degrees of phase offset with uniform distribution is modeled, the JCE performance degradation compared to no phase offset is -0.42 dB, -0.67 dB, -0.19 dB for 2Rx + 2 repetitions, 2Rx + 8 repetitions and 8Rx + 2 repetitions respectively.

Therefore, we propose:

Proposal 3: To guarantee the JCE performance with up to 8 repetitions, define the phase continuity tolerance as 20 degrees.

2.2  Power consistency tolerance

For the power consistency tolerance, the following options were captured in the WF:

Power consistency tolerance

- Proposed power variation based on simulation results

  - Option 1: 2 dB power variation with uniform distribution

  - Option 2: 1 dB fixed power offset (0.5 dB amplitude offset)

  - Option 3: Further study

  - Option 4: Reuse existing power consistency requirements defined in RAN4 spec.

- Focus on Pi/2 BPSK (PUCCH/PUSCH), QPSK (PUCCH/PUSCH), BPSK (PUCCH).

- Further discuss the model of power variation if existing power consistency requirements cannot be reused:

  - Option 1: uniform distribution

  - Option 2: fixed offset

<< To be added >>

- Conclusion

This contribution presented our views on phase continuity and power consistency tolerance, with the following observations and proposals:

Proposal 1: Use option 2 of uniform distribution for the phase offset model, since it can be referred to define UE requirements in terms of maximum allowed phase offset.

Proposal 2: Clarify the definition of the offset as:

- Definition of the offset

  - For each individual slot (0…n) within the bundle, an independent offset is generated and applied with respect to the slot 0.

Observation 1: When 20 degrees of phase offset with uniform distribution is modeled, the JCE performance degradation compared to no phase offset is -0.19 dB, -0.52 dB, -0.10 dB for 2Rx + 2 repetitions, 2Rx + 8 repetitions and 8Rx + 2 repetitions respectively.

Observation 2: When 30 degrees of phase offset with uniform distribution is modeled, the JCE performance degradation compared to no phase offset is -0.42 dB, -0.67 dB, -0.19 dB for 2Rx + 2 repetitions, 2Rx + 8 repetitions and 8Rx + 2 repetitions respectively.

Proposal 3: To guarantee the JCE performance with up to 8 repetitions, define the phase continuity tolerance as 20 degrees.

- References

R4-2114991, Reply LS on PUCCH and PUSCH transmissions, Qualcomm, RAN4 #100e, August 2021.

R4-2114992, WF on phase continuity and power consistency for PUCCH and PUSCH transmissions, Huawei, HiSilicon, RAN4 #100e, August 2021.