3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117242

# Electronic Meeting, 1 – 12 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.141 | CR | CRNum | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 25.141: Correction on tables for Band 23 co-location requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14 |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Entries for Band 23 were deleted from table for coexistence spurious emission limits but kept in tables for Home BS and co-location requirements. This would create ambiguity on Band 23 Home BS and co-location requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the entries for Band 23 from tables for Home BS and co-location requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity remains and would lead to different interpretations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.7.5, 6.5.3.7.10, 7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 25.104 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

##### 6.5.3.7.5	Co-existence with co-located and co-sited base stations

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, E-UTRA FDD and/or UTRA FDD BS, CDMA, E-UTRA TDD, NR and or UTRA TDD are co-located with a UTRA FDD BS.

The power of any spurious emission shall not exceed the limits of Table 6.39 for a Wide Area (WA) BS where requirements for co-location with a BS type listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.39 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.39 apply for the operating band supported at that antenna connector.

Table 6.39: BS Spurious emissions limits for Wide Area BS co-located with another BS

| Type of co-located BS | Band for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| Macro GSM900 | 876-915 MHz | -98 dBm | 100 kHz |  |
| Macro DCS1800 | 1710 - 1785 MHz | -98 dBm | 100 kHz |  |
| Macro PCS1900 | 1850 - 1910 MHz | -98 dBm | 100 kHz |  |
| Macro GSM850 or CDMA850 | 824 - 849 MHz | -98 dBm | 100 kHz |  |
| WA UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 1920 - 1980 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1850 - 1910 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1710 - 1785 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 824 - 849 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band VI or XIX or E-UTRA Band 6, 18 or 19 | 815 - 845 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2500 - 2570 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 880 - 915 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 699 - 716 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 17 | 704 - 716 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 832 - 862 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 - 1915 MHz | -96 dBm | 100 kHz |  |
| WA UTRA FDD Band XXVI or E-UTRA Band 26 | 814-849 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 27 | 807 - 824 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 28 or NR band n28 | 703 – 748 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 30 | 2305 – 2315 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 31 | 452.5 – 457.5 MHz | -96 dBm | 100 kHz |  |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -86 dBm | 1 MHz |  |
| WA UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -86 dBm | 1 MHz |  |
| WA UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -86 dBm | 1 MHz |  |
| WA UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -86 dBm | 1 MHz |  |
| WA UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -86 dBm | 1 MHz |  |
| WA UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 - 2620 MHz | -86 dBm | 1 MHz |  |
| WA UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 - 1920MHz | -86 dBm | 1 MHz | Applicable in China |
| WA UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 - 2400MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 41 or NR band n41 | 2496 – 2690 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 42 | 3400 – 3600 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 43 | 3600 – 3800 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 44 | 703 – 803 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 45 | 1447 – 1467 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 48 | 3550 – 3700 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 52 | 3300 – 3400 MHz | -86 dBm | 1 MHz |  |
| WA E-UTRA Band 65 | 1920 – 2010 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 66 or NR band n66 | 1710 – 1780 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 68 | 698 – 728 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 70 or NR band n70 | 1695 – 1710 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 71 or NR band n71 | 663 - 698 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 72 | 451 - 456 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 73 | 450 - 455 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n77 | 3300 – 4200 MHz | -86 dBm | 1 MHz |  |
| WA NR Band n78 | 3300 – 3800 MHz | -86 dBm | 1 MHz |  |
| WA NR Band n79 | 4400 – 5000 MHz | -86 dBm | 1 MHz |  |
| WA NR Band n80 | 1710 – 1785 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n81 | 880 – 915 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n82 | 832 – 862 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n83 | 703 – 748 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n84 | 1920 – 1980 MHz | -96 dBm | 100 kHz |  |
| WA E-UTRA Band 85 | 698 – 716 MHz | -96 dBm | 100 kHz |  |
| WA NR Band n86 | 1710 – 1780 MHz | -96 dBm | 100 kHz |  |
| NOTE 1: The co-location requirements do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Table 3.0). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [2]. NOTE 2: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. |  |  |  |  |

The power of any spurious emission shall not exceed the limits of Table 6.40 for a Medium Range (MR) BS where requirements for co-location with a BS type listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.40 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.40 apply for the operating band supported at that antenna connector.

Table 6.40: BS Spurious emissions limits for Medium Range BS co-located with another BS

| Type of co-located BS | Band for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| Micro GSM900 | 876-915 MHz | -91 dBm | 100 kHz |  |
| Micro DCS1800 | 1710 - 1785 MHz | -96 dBm | 100 kHz |  |
| Micro PCS1900 | 1850 - 1910 MHz | -96 dBm | 100 kHz |  |
| Micro GSM850 | 824 - 849 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 1920 - 1980 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1850 - 1910 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1710 - 1785 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 824 - 849 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band VI or XIX or E-UTRA Band 6, 18 or 19 | 815 - 845 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2500 - 2570 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 880 - 915 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 699 - 716 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 17 | 704 - 716 MHz | -91 dBm | 100 KHz |  |
| MR UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 832 - 862 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -91 dBm | 100 KHz |  |
| MR UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 - 1915 MHz | -91 dBm | 100 kHz |  |
| MR UTRA FDD Band XXVI or E-UTRA Band 26 | 814.849 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 27 | 807 - 824 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 28 or NR band n28 | 703 – 748 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 30 | 2305 - 2315 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 31 | 452.5 – 457.5 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 33 | 1900 - 1920 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -91 dBm | 100 KHz |  |
| MR UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -91 dBm | 100 KHz |  |
| MR UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -91 dBm | 100 KHz |  |
| MR UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 38 or NR band n38 | 2570 - 2620 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 39 or NR band n39 | 1880 - 1920MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 40 or NR band n40 | 2300 - 2400MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 41 or NR band n41 | 2496 – 2690 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 42 | 3400 – 3600 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 43 | 3600 – 3800 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 44 | 703 – 803 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 45 | 1447 – 1467 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 46 | 5150 – 5925 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 48 | 3550 – 3700 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 50 or NR band n50 | 1432 – 1517 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 52 | 3300 – 3400 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 53 | 2483.5 – 2495 MHz | -91 dBm | 100 KHz |  |
| MR E-UTRA Band 65 | 1920 – 2010 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 66 or NR band n66 | 1710 – 1780 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 68 | 698 – 728 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 70 or NR band n70 | 1695 – 1710 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 71 or NR band n71 | 663 - 698 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 72 | 451 - 456 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 73 | 450 - 455 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n77 | 3300 – 4200 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n78 | 3300 – 3800 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n79 | 4400 – 5000 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n80 | 1710 – 1785 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n81 | 880 – 915 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n82 | 832 – 862 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n83 | 703 – 748 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n84 | 1920 – 1980 MHz | -91 dBm | 100 kHz |  |
| MR E-UTRA Band 85 | 698 – 716 MHz | -91 dBm | 100 kHz |  |
| MR NR Band n86 | 1710 – 1780 MHz | -91 dBm | 100 kHz |  |
| NOTE 1: The co-location requirements do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Table 3.0). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [2]. NOTE 2: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 3: The requirements in this table do not apply to a Rel-10 or an earlier release UTRA MR BS, which is manufactured before 31 December 2015 and upgraded to support features in the present release, where the upgrade does not affect existing RF parts of the radio unit related to this requirement. For such a UTRA MR BS, the corresponding requirements in Rel-10 or the earlier release, which the BS was manufactured for, shall apply. |  |  |  |  |

The power of any spurious emission shall not exceed the limits of Table 6.41 for a Local Area (LA) BS where requirements for co-location with a BS type listed in the first column apply. For BS capable of multi-band operation, the exclusions and conditions in the Note column of Table 6.41 apply for each supported operating band. For BS capable of multi-band operation where multiple bands are mapped on separate antenna connectors, the exclusions and conditions in the Note column of Table 6.41 apply for the operating band supported at that antenna connector.

Table 6.41: BS Spurious emissions limits for Local Area BS co-located with another BS

| Type of co-located BS | Band for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| Pico GSM900 | 876-915 MHz | -70 dBm | 100 kHz |  |
| Pico DCS1800 | 1710 - 1785 MHz | -80 dBm | 100 kHz |  |
| Pico PCS1900 | 1850 - 1910 MHz | -80 dBm | 100 kHz |  |
| Pico GSM850 | 824 - 849 MHz | -70 dBm | 100 kHz |  |
| LA UTRA FDD Band I or E-UTRA Band 1 or NR band n1 | 1920 - 1980 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band II or E-UTRA Band 2 or NR band n2 | 1850 - 1910 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band III or E-UTRA Band 3 or NR band n3 | 1710 - 1785 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band V or E-UTRA Band 5 or NR band n5 | 824 - 849 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band VI or XIX or E-UTRA Band 6, 18 or 19 | 815 - 845 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band VII or E-UTRA Band 7 or NR band n7 | 2500 - 2570 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band VIII or E-UTRA Band 8 or NR band n8 | 880 - 915 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XII or E-UTRA Band 12 or NR band n12 | 699 - 716 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 17 | 704 - 716 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XX or E-UTRA Band 20 or NR band n20 | 832 - 862 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXII or E-UTRA Band 22 | 3410 – 3490 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 24 | 1626.5 – 1660.5 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1850 - 1915 MHz | -88 dBm | 100 kHz |  |
| LA UTRA FDD Band XXVI or E-UTRA Band 26 | 814-849 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 27 | 807 - 824 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 28 or NR band n28 | 703 – 748 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 30 | 2305 – 2315 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 31 | 452.5 – 457.5 MHz | -88 dBm | 100 kHz |  |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -78 dBm | 1 MHz |  |
| LA UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -78 dBm | 1 MHz |  |
| LA UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -78 dBm | 1 MHz |  |
| LA UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -78 dBm | 1 MHz |  |
| LA UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -78 dBm | 1 MHz |  |
| LA UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 - 2620 MHz | -78 dBm | 1 MHz |  |
| LA UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 - 1920MHz | -78 dBm | 1 MHz | Applicable in China |
| LA UTRA TDD Band e) or E-UTRA Band 40 or NR band n40 | 2300 - 2400MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 41 or NR band n41 | 2496 - 2690 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 42 | 3400 - 3600MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 43 | 3600 - 3800MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 44 | 703 - 803 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 45 | 1447 - 1467 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 46 | 5150 - 5925 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 48 | 3550 - 3700 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 49 | 3550 - 3700 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 50 or NR band n50 | 1432 - 1517 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 51 or NR band n51 | 1427 - 1432 MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 52 | 3300 - 3400MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 53 | 2483.5 - 2495MHz | -78 dBm | 1 MHz |  |
| LA E-UTRA Band 65 | 1920 – 2010 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 66 or NR band n66 | 1710 – 1780 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 68 | 698 – 728 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 70 or NR band n70 | 1695 – 1710 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 71 or NR band n71 | 663 - 698 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 72 | 451 - 456 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 73 | 450 - 455 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 74 or NR band n74 | 1427 – 1470 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n77 | 3300 – 4200 MHz | -78 dBm | 1 MHz |  |
| LA NR Band n78 | 3300 – 3800 MHz | -78 dBm | 1 MHz |  |
| LA NR Band n79 | 4400 – 5000 MHz | -78 dBm | 1 MHz |  |
| LA NR Band n80 | 1710 – 1785 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n81 | 880 – 915 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n82 | 832 – 862 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n83 | 703 – 748 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n84 | 1920 – 1980 MHz | -88 dBm | 100 kHz |  |
| LA E-UTRA Band 85 | 698 – 716 MHz | -88 dBm | 100 kHz |  |
| LA NR Band n86 | 1710 – 1780 MHz | -88 dBm | 100 kHz |  |
| NOTE 1: The co-location requirements do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see Table 3.0). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [2]. NOTE 2: The table above assumes that two operating bands, where the frequency ranges would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications. NOTE 3: The requirements in this table do not apply to a Rel-10 or an earlier release UTRA LA BS, which is manufactured before 31 December 2013 and upgraded to support features in the present release, where the upgrade does not affect existing RF parts of the radio unit related to this requirement. For such a UTRA LA BS, the corresponding requirements in Rel-10 or the earlier release, which the BS was manufactured for, shall apply. |  |  |  |  |

<Next change>

##### 6.5.3.7.10	Co-existence with Home BS operating in other bands

These requirements may be applied for the protection of Home BS receivers operating in other bands. These requirements are only applicable to Home BS.

The power of any spurious emission shall not exceed the limits of Table 6.47 for a Home BS where requirements for co-existence with a Home BS type listed in the first column apply.

Table 6.47: Home BS Spurious emissions limits for co-existence with Home BS operating in other bands

| Type of Home BS | Band for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| UTRA FDD Band I or E-UTRA Band 1 | 1920 - 1980 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band II or E-UTRA Band 2 | 1850 - 1910 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band III or E-UTRA Band 3 | 1710 - 1785 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band IV or E-UTRA Band 4 | 1710 - 1755 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band V or E-UTRA Band 5 | 824 - 849 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band VI or XIX or E-UTRA Band 6, 19 | 815 - 845 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band VII or E-UTRA Band 7 | 2500 - 2570 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band VIII or E-UTRA Band 8 | 880 - 915 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band IX or E-UTRA Band 9 | 1749.9 - 1784.9 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band X or E-UTRA Band 10 | 1710 - 1770 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XI or E-UTRA Band 11 | 1427.9 - 1447.9 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XII or E-UTRA Band 12 | 699 - 716 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XIII or E-UTRA Band 13 | 777 - 787 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XIV or E-UTRA Band 14 | 788 - 798 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 17 | 704 - 716 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XX or E-UTRA Band 20 | 832 - 862 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1447.9 - 1462.9 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3410 - 3490 MHz | -71 dBm | 100 kHz |  |
| E-UTRA FDD Band 24 | 1626.5 – 1660.5 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1850-1915 MHz | -71 dBm | 100 kHz |  |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 814-849 MHz | -71dBm | 100 kHz |  |
| E-UTRA FDD Band 27 | 807 – 824 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 28 | 703 – 748 MHz | -71dBm | 100 kHz |  |
| E-UTRA Band 30 | 2305 – 2315 MHz | -71dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -71 dBm | 100 kHz |  |
| UTRA TDD Band a) or E-UTRA Band 34 | 2010 - 2025 MHz | -71 dBm | 100 kHz |  |
| UTRA TDD Band d) or E-UTRA Band 38 | 2570 - 2620 MHz | -71 dBm | 100 kHz |  |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880 - 1920 MHz | -71 dBm | 100 kHz |  |
| UTRA TDD Band e) E-UTRA Band 40 | 2300 - 2400 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 41 | 2496 - 2690 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 42 | 3400 -3600 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 43 | 3600 -3800 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 44 | 703 -803 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 48 | 3550 -3700 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 50 | 1432 - 1517 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 51 | 1427 - 1432 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 52 | 3300 -3400 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 65 | 1920 – 2110 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 66 | 1710 – 1780 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 68 | 698 – 728 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 70 | 1695 – 1710 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 71 | 663 - 698 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 74 | 1427 – 1470 MHz | -71 dBm | 100 kHz |  |
| E-UTRA Band 85 | 698 – 716 MHz | -71 dBm | 100 kHz |  |

<Next change>

### 7.5.5	Test Requirements

For each measured carrier, the BER shall not exceed 0.001 for the parameters specified in table 7.4K to 7.4V if applicable for the BS under test.

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the lower/upper Base Station RF Bandwith edges or Maximum Radio Bandwidth edges.

For a BS operating in non-contiguous spectrum within any operating band, the blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15MHz. The interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -7.5MHz/+7.5MHz, respectively.

For a BS operating in non-contiguous spectrum within any operating band, the narrowband blocking requirements in Tables 7.4R-7.4T apply in addition inside any sub-block gap, in case the sub-block gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to the lower/upper sub-block edge inside the sub-block gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

For a BS capable of multi-band operation, the requirement in the in-band blocking frequency range applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 15MHz. The interfering signal offset is defined relative to lower/upper Base Station RF bandwidth edges inside the Inter RF Bandwidth gap and is equal to -7.5MHz/+7.5MHz, respectively.

For a BS capable of multi-band operation, the requirement in the out-of-band blocking frequency ranges apply for each operating band, with the exception that the in-band blocking frequency ranges of all supported operating bands according to Tables 7.4K - 7.4M shall be excluded from the out-of-band blocking requirement.

For a BS capable of multi-band operation, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

Table 7.4K: Blocking characteristics for Wide Area BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| V | 824-849 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 804-824 MHz 849-869 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 860 - 880 MHz 915 - 925 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 679 - 699 MHz 716 - 729 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 679 MHz 729 MHz – 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 757 - 777 MHz 787 - 807 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 768 - 788 MHz 798 - 818 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 810 - 830 MHz 845 - 865 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 821 - 832 MHz 862 - 882 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -40 dBm | -115 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 794-814 MHz 849-859 MHz | -40 dBm | -115 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -115 dBm |  | CW carrier |
| Note *: The characteristics of the W-CDMA interference signal are specified in Annex I. NOTE**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -119.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.4K assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.4L: Blocking characteristics for Medium Range BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal Level | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| V | 824-849 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 804-824 MHz 849-869 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 860 - 880 MHz 915 - 925 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 679 - 699 MHz 716 - 729 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 679 MHz 729 MHz – 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 757 - 777 MHz 787 - 807 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 768 - 788 MHz 798 - 818 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 810 - 830 MHz 845 - 865 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 821 - 832 MHz 862 - 882 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -35 dBm | -105 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 794-814 MHz 849-859 MHz | -35 dBm | -105 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -105 dBm |  | CW carrier |
| Note *: The characteristics of the W-CDMA interference signal are specified in Annex I. NOTE**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -109.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.4L assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.4M: Blocking characteristics for Local Area / Home BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal Level | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| I | 1920 - 1980 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1900 - 1920 MHz 1980 - 2000 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -1900 MHz 2000 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| II | 1850 - 1910 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1910 - 1930 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| III | 1710 - 1785 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1785 - 1805 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1805 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| IV | 1710 - 1755 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1755 - 1775 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1775 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| V | 824-849 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 804-824 MHz 849-869 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 804 MHz 869 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VI | 810 - 830 MHz 840 - 860 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 860 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VII | 2500 - 2570 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 2480 - 2500 MHz 2570 - 2590 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -2480 MHz 2590 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| VIII | 880 - 915 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 860 - 880 MHz 915 - 925 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz -860 MHz 925 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| IX | 1749.9 - 1784.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1729.9 - 1749.9 MHz 1784.9 - 1804.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1729.9 MHz 1804.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| X | 1710 - 1770 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1690 - 1710 MHz 1770 - 1790 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1690 MHz 1790 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XI | 1427.9 - 1447.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1407.9 - 1427.9 MHz 1447.9 - 1467.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1407.9 MHz 1467.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XII | 699 - 716 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 679 - 699 MHz 716 - 729 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 679 MHz 729 MHz – 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIII | 777 - 787 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 757 - 777 MHz 787 - 807 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 757 MHz 807 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIV | 788 - 798 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 768 - 788 MHz 798 - 818 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 - 768 MHz 818 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XIX | 830 - 845 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 810 - 830 MHz 845 - 865 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 810 MHz 865 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XX | 832 - 862 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 821 - 832 MHz 862 - 882 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 821 MHz 882 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXI | 1447.9 - 1462.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1427.9 - 1447.9 MHz 1462.9 - 1482.9 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 1427.9 MHz 1482.9 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXII | 3410 - 3490 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 3390 - 3410 MHz 3490 - 3510 MHz | -30 dBm | -101 dBm | 10 MHz | WCDMA signal * |
|  | 1 MHz - 3390 MHz 3510 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXV | 1850 - 1915 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 1830 - 1850 MHz 1915 - 1930 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 1830 MHz 1930 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| XXVI | 814-849 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 794-814 MHz 849-859 MHz | -30 dBm | -101 dBm | ±10 MHz | WCDMA signal * |
|  | 1 MHz - 794 MHz 859 MHz - 12750 MHz | -15 dBm | -101 dBm |  | CW carrier |
| Note *: The characteristics of the W-CDMA interference signal are specified in Annex I. NOTE**: For a BS capable of multiband operation, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in an adjacent or overlapping band, the wanted signal mean power is equal to -105.6 dBm. |  |  |  |  |  |

NOTE:	Table 7.4M assumes that two operating bands, where the downlink frequencies (see Table 3.0) of one band would be within the in-band blocking region of the other band, are not deployed in the same geographical area.

Table 7.4N: Blocking performance requirement for Wide Area BS when co-located with BS in other bands.

| Co-located BS type | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Macro GSM900 | 921 - 960 MHz | +16 dBm | -115 dBm | CW carrier |
| Macro DCS1800 | 1805 - 1880 MHz | +16 dBm | -115 dBm | CW carrier |
| Macro PCS1900 | 1930 - 1990 MHz | +16 dBm | -115 dBm | CW carrier |
| Macro GSM850 or CDMA850 | 869 - 894 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 - 2170 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 - 1990 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 - 1880 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band V or E-UTRA Band 5 or NR band n5 | 869 - 894 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band VI or E-UTRA Band 6 | 875 - 885 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 - 2690 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 - 960 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 - 746 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 17 | 734 - 746 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 18 | 860 – 875 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XIX or E-UTRA Band 19 | 875 - 890 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 24 | 1525 – 1559 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 - 1995 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 27 | 852 – 869 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 28 or NR band n28 | 758 – 803 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 29 | 717 - 728 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 30 | 2350 – 2360 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 31 | 462.5 – 467.5 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA-FDD Band XXXII or E-UTRA Band 32 | 1452 – 1496 MHz (NOTE 3) | + 16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | +16 dBm | -115 dBm | CW carrie |
| WA UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | +16 dBm | -115 dBm | CW carrie |
| WA UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 - 2620 MHz | +16 dBm | -115 dBm | CW carrier |
| WA UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 - 1920 MHz | +16 dBm | -115 dBm | CW carrie |
| WA UTRA TDD Band e) E-UTRA Band 40 or NR band n40 | 2300 - 2400 MHz | +16 dBm | -115 dBm | CW carrie |
| WA E-UTRA Band 41 or NR band n41 | 2496 - 2690 MHz | +16 dBm | -115 dBm | CW carrie |
| WA E-UTRA Band 42 | 3400 - 3600 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 43 | 3600 - 3800 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 44 | 703 - 803 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 45 | 1447 - 1467 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 48 | 3550 - 3700 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 50 or NR band n50 | 1432 - 1517 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 52 | 3300 - 3400 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 65 | 2110 - 2200 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 66 or NR band n66 | 2110 - 2200 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 67 | 738 - 758 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 68 | 753 - 783 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 69 | 2570 - 2620 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 70 or NR band n70 | 1995 – 2020 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 71 or NR band n71 | 617 - 652 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 72 | 461 - 466 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 73 | 460 - 465 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 74 or NR band n74 | 1475 – 1518 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 75 or NR band n75 | 1432 - 1517 MHz | +16 dBm | -115 dBm | CW carrier |
| WA NR band n77 | 3300 – 4200 MHz | +16 dBm | -115 dBm | CW carrier |
| WA NR band n78 | 3300 – 3800 Mz | +16 dBm | -115 dBm | CW carrier |
| WA NR band n79 | 4400 – 5000 MHz | +16 dBm | -115 dBm | CW carrier |
| WA E-UTRA Band 85 | 728 - 746 MHz | +16 dBm | -115 dBm | CW carrier |
| NOTE 1: Except for a BS operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of th supported e uplink operating band. For a BS operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA FDD with UTRA TDD or E-UTRA TDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4]. NOTE 3: For a BS operating in band XI or XXI, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. |  |  |  |  |

Table 7.4P: Blocking performance requirement for Medium Range BS when co-located with BS in other bands.

| Co-located BS type |  | Center Frequency of Interfering Signal |  | Interfering Signal mean power |  | Wanted Signal mean power |  | Type of Interfering Signal |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Micro GSM900 |  | 921 - 960 MHz |  | -3 dBm |  | -105 dBm |  | CW carrier |  |
| Micro DCS1800 |  | 1805 - 1880 MHz |  | +5 dBm |  | -105 dBm |  | CW carrier |  |
| Micro PCS1900 |  | 1930 - 1990 MHz |  | +5 dBm |  | -105 dBm |  | CW carrier |  |
| Micro GSM850 |  | 869 - 894 MHz |  | -3 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band I or E-UTRA Band 1 or NR band n1 |  | 2110 - 2170 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band II or E-UTRA Band 2 or NR band n2 |  | 1930 - 1990 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band III or E-UTRA Band 3 or NR band n3 |  | 1805 - 1880 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band IV or E-UTRA Band 4 |  | 2110 - 2155 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band V or E-UTRA Band 5 or NR band n5 |  | 869 - 894 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band VI or E-UTRA Band 6 |  | 875 - 885 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band VII or E-UTRA Band 7 or NR band n7 |  | 2620 - 2690 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band VIII or E-UTRA Band 8 or NR band n8 |  | 925 - 960 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band IX or E-UTRA Band 9 |  | 1844.9 - 1879.9 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band X or E-UTRA Band 10 |  | 2110 - 2170 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XI or E-UTRA Band 11 |  | 1475.9 - 1495.9 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XII or E-UTRA Band 12 or NR band n12 |  | 729 - 746 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XIII or E-UTRA Band 13 |  | 746 - 756 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XIV or E-UTRA Band 14 |  | 758 - 768 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 17 |  | 734 - 746 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 18 |  | 860 – 875 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XIX or E-UTRA Band 19 |  | 875 - 890 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XX or E-UTRA Band 20 or NR band n20 |  | 791 - 821 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XXI or E-UTRA Band 21 |  | 1495.9 - 1510.9 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XXII or E-UTRA Band 22 |  | 3510 - 3590 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 24 |  | 1525 – 1559 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XXV or E-UTRA Band 25 or NR band n25 |  | 1930 - 1995 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XXVI or E-UTRA Band 26 |  | 859 - 894 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 27 |  | 852 – 869 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 28 or NR band n28 |  | 758 – 803 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 29 |  | 717 - 728 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 30 |  | 2350 - 2360 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 31 |  | 462.5 – 467.5 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA-FDD Band XXXII or E-UTRA Band 32 |  | 1452 – 1496 MHz (NOTE 3) |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 33 |  | 1900 - 1920 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 34 or NR band n34 |  | 2010 - 2025 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA TDD Band b) or E-UTRA Band 35 |  | 1850 – 1910 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA TDD Band b) or E-UTRA Band 36 |  | 1930 – 1990 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR UTRA TDD Band c) or E-UTRA Band 37 |  | 1910 – 1930 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 38 or NR band n38 |  | 2570 - 2620 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 39 or NR band n39 |  | 1880 - 1920 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 40 or NR band n40 |  | 2300 - 2400 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 41 or NR band n41 |  | 2496 - 2690 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 42 |  | 3400 - 3600 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 43 |  | 3600 - 3800 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 44 |  | 703 - 803 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 45 |  | 1447 - 1467 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 46 |  | 5150 - 5925 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 48 |  | 3550 - 3700 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 50 or NR band n50 |  | 1432 - 1517 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 52 |  | 3300 - 3400 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 53 |  | 2483.5 - 2495 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 65 |  | 2110 - 2200 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 66 or NR band n66 |  | 2110 - 2200 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 67 |  | 738 - 758 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 68 |  | 753 - 783 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 69 |  | 2570 - 2620 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 70 or NR band n70 |  | 1995 - 2020 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 71 or NR band n71 |  | 617 - 652 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 72 |  | 461 - 466 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 73 |  | 460 - 465 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 74 or NR band n74 |  | 1475 – 1518 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 75 or NR band n75 |  | 1432 - 1517 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR NR band n77 |  | 3300 – 4200 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR NR band n78 |  | 3300 – 3800 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR NR band n79 |  | 4400 – 5000 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| MR E-UTRA Band 85 |  | 728 - 746 MHz |  | +8 dBm |  | -105 dBm |  | CW carrier |  |
| NOTE 1: Except for a BS operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA FDD with UTRA TDD or E-UTRA TDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4]. NOTE 3: For a BS operating in band XI or XXI, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. |  |  |  |  |  |  |  |  |  |

Table 7.4Q: Blocking performance requirement for Local Area BS when co-located with BS in other bands.

| Co-located BS type | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Pico GSM900 | 921 - 960 MHz | -7 dBm | -101 dBm | CW carrier |
| Pico DCS1800 | 1805 - 1880 MHz | -4 dBm | -101 dBm | CW carrier |
| Pico PCS1900 | 1930 - 1990 MHz | -4 dBm | -101 dBm | CW carrier |
| Pico GSM850 | 869 - 894 MHz | -7 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band I or E-UTRA Band 1 or NR band n1 | 2110 - 2170 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band II or E-UTRA Band 2 or NR band n2 | 1930 - 1990 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band III or E-UTRA Band 3 or NR band n3 | 1805 - 1880 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band IV or E-UTRA Band 4 | 2110 - 2155 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band V or E-UTRA Band 5 or NR band n5 | 869 - 894 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band VI or E-UTRA Band 6 | 875 - 885 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band VII or E-UTRA Band 7 or NR band n7 | 2620 - 2690 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band VIII or E-UTRA Band 8 or NR band n8 | 925 - 960 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band IX or E-UTRA Band 9 | 1844.9 - 1879.9 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band X or E-UTRA Band 10 | 2110 - 2170 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XII or E-UTRA Band 12 or NR band n12 | 729 - 746 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XIII or E-UTRA Band 13 | 746 - 756 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XIV or E-UTRA Band 14 | 758 - 768 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 17 | 734 - 746 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 18 | 860 - 875 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XIX or E-UTRA Band 19 | 875 - 890 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XX or E-UTRA Band 20 or NR band n20 | 791 - 821 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XXI or E-UTRA Band 21 | 1495.9 - 1510.9 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XXII or E-UTRA Band 22 | 3510 - 3590 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 24 | 1525 – 1559 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XXV or E-UTRA Band 25 or NR band n25 | 1930 - 1995 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA-FDD Band XXVI or E-UTRA Band 26 | 859 - 894 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 27 | 852 - 869 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 28 or NR band n28 | 758 – 803 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 29 | 717 - 728 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 30 | 2350 – 2360 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 31 | 462.5 – 467.5 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA_FDD Band XXXII or E-UTRA Band 32 | 1452 – 1496 MHz (NOTE 3) | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band a) or E-UTRA Band 33 | 1900 - 1920 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band a) or E-UTRA Band 34 or NR band n34 | 2010 - 2025 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band b) or E-UTRA Band 35 | 1850 – 1910 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band b) or E-UTRA Band 36 | 1930 – 1990 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band c) or E-UTRA Band 37 | 1910 – 1930 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band d) or E-UTRA Band 38 or NR band n38 | 2570 - 2620 MHz | -4 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band f) or E-UTRA Band 39 or NR band n39 | 1880 - 1920 MHz | -6 dBm | -101 dBm | CW carrier |
| LA UTRA TDD Band e) E-UTRA Band 40 or NR band n40 | 2300 - 2400 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 41 or NR band n41 | 2496 - 2690 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 42 | 3400 - 3600 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 43 | 3600 – 3800 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 44 | 703 – 803 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 45 | 1447 - 1467 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 46 | 5150 - 5925 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 48 | 3550 – 3700 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 49 | 3550 – 3700 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 50 or NR band n50 | 1432 - 1517 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 51 or NR band n51 | 1427 - 1432 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 52 | 3300 - 3400 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 53 | 2483.5 - 2495 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 65 | 2110 - 2200 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 66 or NR band n66 | 2110 - 2200 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 67 | 738 - 758 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 68 | 753 - 783 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 69 | 2570 - 2620 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 70 or NR band n70 | 1995 - 2020 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 71 or NR band n71 | 617 - 652 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 72 | 461 - 466 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 73 | 460 - 465 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 74 or NR band n74 | 1475 – 1518 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 75 or NR band n75 | 1432 - 1517 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 76 or NR band n76 | 1427 - 1432 MHz | -6 dBm | -101 dBm | CW carrier |
| LA NR band n77 | 3300 – 4200 MHz | -6 dBm | -101 dBm | CW carrier |
| LA NR band n78 | 3300 – 3800 MHz | -6 dBm | -101 dBm | CW carrier |
| LA NR band n79 | 4400 – 5000 MHz | -6 dBm | -101 dBm | CW carrier |
| LA E-UTRA Band 85 | 728 - 746 MHz | -6 dBm | -101 dBm | CW carrier |
| NOTE 1: Except for a BS operating in Band XIII, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported the uplink operating band. For a BS operating in band XIII the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. NOTE 2: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA FDD with UTRA TDD or E-UTRA TDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4]. NOTE 3: For a BS operating in band XI or XXI, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. |  |  |  |  |

Table 7.4R: Blocking performance requirement (narrowband) for Wide Area BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| II | 1850 - 1910 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| III | 1710 - 1785 MHz | - 47 dBm | -115 dBm | 2.8 MHz | GMSK modulated* |
| IV | 1710 - 1755 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| V | 824 - 849 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| VIII | 880 - 915 MHz | - 47 dBm | -115 dBm | 2.8 MHz | GMSK modulated* |
| X | 1710 - 1770 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| XII | 699 - 716 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| XIII | 777 - 787 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| XIV | 788 - 798 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| XXV | 1850 - 1915 MHz | - 47 dBm | -115 dBm | 2.7 MHz | GMSK modulated* |
| XXVI | 814-849 MHz | -47 dBm | -115 dBm | ±2.7 MHz | GMSK modulated* |
| Note *: GMSK modulation as defined in TS 45.004 [12]. |  |  |  |  |  |

Table 7.4S: Blocking performance requirement (narrowband) for Medium range BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| II | 1850 - 1910 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| III | 1710 - 1785 MHz | - 42 dBm | -105 dBm | 2.8 MHz | GMSK modulated* |
| IV | 1710 - 1755 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| V | 824 - 849 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| VIII | 880 - 915 MHz | - 42 dBm | -105 dBm | 2.8 MHz | GMSK modulated* |
| X | 1710 - 1770 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| XII | 699 - 716 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| XIII | 777 - 787 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| XIV | 788 - 798 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| XXV | 1850 - 1915 MHz | - 42 dBm | -105 dBm | 2.7 MHz | GMSK modulated* |
| XXVI | 814-849 MHz | - 42 dBm | -105 dBm | ±2.7 MHz | GMSK modulated* |
| Note *: GMSK modulation as defined in TS 45.004 [12]. |  |  |  |  |  |

Table 7.4T: Blocking performance requirement (narrowband) for Local Area / Home BS

| Operating Band | Center Frequency of Interfering Signal | Interfering Signal mean power | Wanted Signal mean power | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- |
| II | 1850 - 1910 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| III | 1710 - 1785 MHz | - 37 dBm | -101 dBm | 2.8 MHz | GMSK modulated* |
| IV | 1710 - 1755 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| V | 824 - 849 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| VIII | 880 - 915 MHz | - 37 dBm | -101 dBm | 2.8 MHz | GMSK modulated* |
| X | 1710 - 1770 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| XII | 699 - 716 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| XIII | 777 - 787 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| XIV | 788 - 798 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| XXV | 1850 – 1915 MHz | - 37 dBm | -101 dBm | 2.7 MHz | GMSK modulated* |
| XXVI | 814-849 MHz | - 37 dBm | -101 dBm | ±2.7 MHz | GMSK modulated* |
| Note *: GMSK modulation as defined in TS 45.004 [12]. |  |  |  |  |  |

Table 7.4U: Void

Table 7.4V: Void

NOTE 1:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex F.

NOTE 2:	Annex C describes the procedure for BER tests taking into account the statistical consequence of frequent repetition of BER measurements within the blocking test . The consequence is: a DUT exactly on the limit may fail due to the statistical nature 2.55 times(mean value) in 12750 BER measurements using the predefined wrong decision probability of 0.02%. If the fail cases are ≤12, it is allowed to repeat the fail cases 1 time before the final verdict.

<End of change>