3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117322

Electronic Meeting, November 01-12, 2021

Title: 	Discussion on NR repeater FR2 ACRR requirement

Source: 	CATT

Agenda item:	8.5.3.3

Document for:	Approval

- Introduction

This contribution provides our view for the FR2 out of band gain and ACRR requirements. This contribution only discusses the FR2 specific issues. For some general issues such as relationship of out of band gain, ACRR and ACLR, our FR1 contribution [1] can be refered.

- Discussion

As discussed in our FR1 contribution [1], out of band gain requirement can be derived from ACRR requirement and ACRR requirement can guarantee the system performance, so we think out of band gain requirement may not be needed. Only ACRR requirement is discussed in this contribution.

  - ACRR requirement

Similar with the FR1 analysis in [1], the following simple deployment scenario is used to analyse ACRR requirement.

Figure 1: The scenario being used to analyze ACRR

    - DL ACRR

As discussed in [1], the following interferences should be considered to derive ACRR requirement,

- Out of band emission from repeater including BSA emission-CLBSA-RepA+ACG, BSB power – BSB ACLR-CLBSB-RepA+ACG (The repeater’s emission at 2nd channel), should make sure emission requirement can be met.

- ACLR contribution to repeater from BSB power: BSB power -CLBSB-RepA+ACG, should be less than repeater’s ACLR without BSB contribution.

- ACS interference to UEA DL, i.e. BSB power – CLBSB- RepA + ACG-CLRepA-UEA, should be less than UEA ACS level.

- In channel interference to UEB, i.e. BSB power –CLBSB-RepA + ACG-CLRepA-UEB, should be less than UE B received signal directly from BSB, i.e. BSB power –CLBSB-UEB

From the detail analysis in [1], the 2nd aspect is the worst scenario to be considered. The FR2 ACLR and emission requirement are based on TRP, so in this analysis, TRP is also used as the metric.

Analysing the 2nd aspect ACLR contribution, if BSA Power = 43 dBm/50MHz, considering 95 dB repeater gain and 33 dBm output power, then BSB power – CLBSB-RepA + ACG should be less than 5dBm/50MHz (-12 dBm/MHz) which is the noise floor after repeater 95 dB gain. Assuming ACRR is [15] dB, ACG = G- 15(ACRR) = 75 dB, the coupling loss between BSB and repeater A CLBSB-RepA should be larger than 113 dB.

Very similar with FR1 analysis, the interference is highly related to CLBSB-RepA and ACRR. If CLBSB-RepA can be well controlled, ACRR can be relatively relaxed. However, ACRR also is related to the ACLR performance for repeater, considering [15-20] dB ACRR corresponding to FR2 BS 28 dB ACLR requirement may be a reasonable requirement for repeater.

Proposal 1: [15-20] dB DL ACRR is the requirement for the FR2 repeater with corresponding to the pass band bandwidth. TRP is used as the metric. The detail requirement can be defined similar with Table 1.

Table 1: FR2 ACRR limit

| Repeater channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | Repeater adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACRR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWChannel | NR of same BW | Square (BWConfig) | [15-20] |

2.2.1 UL ACRR

As the filter performance for DL and UL is similar, the UL ACRR requirement can be defined the same as DL ACRR.

Proposal 2: UL ACRR requirement is defined the same as DL ACRR.

- Conclusion

Some further consideration of the out of band gain and ACRR requirements are provided. We have the following observations and proposals.

Proposal 1: [15-20] dB DL ACRR is the requirement for the FR2 repeater with corresponding to the pass band bandwidth. TRP is used as the metric. The detail requirement can be defined similar with Table 1.

Table 1: FR2 ACRR limit

| Repeater channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz) | Repeater adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | ACRR limit (dB) |
| --- | --- | --- | --- | --- |
| 50, 100, 200, 400 | BWChannel | NR of same BW | Square (BWConfig) | [15-20] |

Proposal 2: UL ACRR requirement is defined the same as DL ACRR.

Reference

[1] R4-2117321, “Discussion on NR repeater FR1 ACRR requirement”, CATT

[2] R4-2115723, “WF on FR2 OOB gain further studies”, Ericsson