3GPP TSG-RAN WG4 Meeting #101-e	R4-2117327

Electronic Meeting, Nov. 1-12, 2021

Title: 	Further discussion on HO with PSCell

Source: 	CATT

Agenda item:	8.10.2.2

Document for:	Discussion

- Introduction

In RAN4#100e meeting, we further discussed the handover with PSCell, and approved a way forward [1]. Some agreements have been reached for following issues:

Sub-topic 2-1 Scenarios for RRM requirement of HO with PSCell

- RAN4 specifies RRM requirement for HO with PSCell for the following scenarios as in the WID RP-202874:

  - from NR SA to EN-DC

  - from EN-DC to EN-DC

  - from NE-DC to NE-DC

  - from NR-DC to NR-DC

- In Rel-17, RAN4 define RRM requirements for NR-DC and NE-DC mode

  - FR1+FR2 NR-DC

  - FR1+LTE NE-DC

- The baseline RRM requirements for FR1+FR1 NR-DC are not in the scope of Rel-17 FeRRM WI. It is up to RAN plenary decision whether and in which release the baseline RRM requirements for FR1+FR1 NR-DC are specified.

Sub-topic 2-2 Delay requirement design of HO with PSCell

- In HO with PSCell for NR-DC to NR-DC

  - Parallel processing shall be the baseline for delay requirements

  - Sequential processing shall be assumed for the following cases

    - Case 1: If SMTC of target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync.

    - Sequential processing is used for cell search and [timing sync]. FFS if additional margin shall be added.

- RACH processing for PCell and PSCell are performed in parallel independently.

- For UE which is already configured with DC, the UE’s behavior is the same whether the configured PSCell is same as the original one or not.

- In HO with PSCell for EN-DC to EN-DC

  - Parallel processing shall be the baseline for delay requirements

- In HO with PSCell for NE-DC to NE-DC

  - Parallel processing shall be the baseline for delay requirements

- Tprocessing for PCell HO

  - Reuse the Tprocessing defined for legacy PCell HO

    - 20ms, when source and target cells are in the same FR

    - 40ms, when source and target cells are in different FRs

Sub-topic 2-4 Generic RACH assumption for HO with PSCell

- RACH occasion on NR-U CC for HO with PSCell

  - Continue discussion on RACH occasion on NR-U CC for HO with PSCell in RAN4 #101e

    - Prioritize EN-DC to EN-DC scenario

    - Companies are encouraged to provide inputs on the candidate requirements

    - FFS whether to introduce requirements

Most issues are FFS and have multiple options. This document will further discuss the issues for handover with PSCell and present our understanding and proposals.

- Discussion

Here we will discuss those issues that have not reached agreements and present our understandings one by one.

Issue 2-2-1a-1: Condition of parallel processing without considering RACH for NR SA to EN-DC

It is FFS with 4 options in [1]. From the discussion and study for the item, it may be summarized that sequential processing may be used only when PSCell is an unknown cell and the network configured SMTC of PSCell with target PCell timing reference. In this case, the UE may search PCell, fine time tracking and acquire full timing information of the target PCell firstly, and then search target PSCell based on PCell timing. Otherwise, UE should use parallel processing for handover and PSCell addition.

So for handover with PSCell from NR SA to EN-DC, Parallel processing shall be the baseline for delay requirements. Sequential processing will be considered only when the NR PSCell to be added is an unknown cell, and the handover command carries NR PSCell SMTC information with E-UTRAN target PCell timing reference for UE.

Proposal 1: For handover with PSCell from NR SA to EN-DC, Parallel processing shall be the baseline for delay requirements. Sequential processing will be considered only when the NR PSCell to be added is an unknown cell, and the handover command carries NR PSCell SMTC information with E-UTRAN target PCell timing reference for UE.

Issue 2-2-1b: Whether requirements for sequential processing are needed if parallel processing is only possible under certain condition

As discussed in Issue 2-2-1a-1, the delay requirements for handover will be defined based on sequential processing only when the NR PSCell to be added is unknown cell, and the handover command carries NR PSCell SMTC information with target PCell timing reference for UE, otherwise, the requirements will be defined based on parallel processing.

On the other hand, the parallel processing can also be used when the NR PSCell to be added is unknown cell, and the handover command carries NR PSCell SMTC information with target PCell timing reference for UE, i.e. UE can directly search PSCell not using reference SMTC information. The parallel processing should be faster for UE to get PSCell.

Proposal 2: The delay requirements for handover will be defined based on sequential processing only when the NR PSCell to be added is an unknown cell, and the handover command carries NR PSCell SMTC information with target PCell timing reference for UE, otherwise, the requirements will be defined based on parallel processing.

Issue 2-2-2a: How the requirements for parallel processing and sequential processing are defined without considering Tprocessing and RA procedures

Generally, we think parallel processing will be faster and will be better performance for handover with PSCell. Sequential processing time will be longer than parallel processing for handover with PSCell, due to UE may turn on the RF chain and start PSCell search after getting the PCell timing information, while the performance of power saving and workload reducing may be better for UE. But since the handover with PSCell is not frequent work, the power saving of sequential processing for handover with PSCell may not be observable while the delay requirements will be definitely extended. So we slightly prefer defining different requirements for parallel processing cases and sequential processing cases.

Proposal 3: Slightly prefer defining different requirements for parallel processing cases and sequential processing cases.

Issue 2-2-2b: Timeline for delay requirements without considering Tprocessing and RA procedures

As above discussion, we think that for parallel processing cases, PCell HO and PSCell addition are performed in parallel independently, for the scenarios of sequential processing, the timeline is Tsearch_PCell + T∆_PCell (i.e. the fine time tracking and acquiring full timing information of the target PCell) + Tmargin_PCell (time for PCell SSB post-processing) + Tsearch_PSCell + T∆_PSCell + Tmargin_PSCell.

Proposal 4: For parallel processing cases, PCell HO and PSCell addition are performed in parallel independently. For the scenarios of sequential processing, the timeline without considering Tprocessing and RA procedures is: Tsearch_PCell + T∆_PCell + Tmargin_PCell + Tsearch_PSCell + T∆_PSCell + Tmargin_PSCell.

Issue 2-2-3a: Timeline of Tprocessing (UE SW processing and RF warm-up (if needed) time) for HO with PSCell

We prefer option 1 in WF [1] for delay requirements, i.e. for both parallel and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are performed in parallel. However it should be UE implementation when UE turn on PSCell RF chain and warm up during period of target PCell search, fine time tracking and acquiring full timing information for sequential processing case. The PSCell RF turning on should have no interruption on target PCell timing search, and no additional Tprocessing for PSCell is need in the requirements.

Proposal 4: For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are assumed performed in parallel.

Issue 2-2-3b: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in parallel

We prefer option 1 in WF [1], i.e. Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change).

Proposal 5: Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

Issue 2-2-3c: If UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in sequential

As discussed in Issue 2-2-3a, Tprocessing for PSCell addition/change may not be considered in the delay requirements even if UE SW processing and RF warm-up for PCell HO and PSCell addition/change are performed in sequential.

Proposal 6: The Tprocessing for HO with PSCell is sum Tprocessing for PCell HO and Tprocessing for PSCell addition/ change, or equal to Tprocessing for PCell HO for sequential processing.

Issue 2-2-3e: Tprocessing for PSCell addition/change

In current spec TS38.133, Tprocessing is

20ms, for E-UTRA PSCell addition in NE-DC,

40ms, for NR FR2 PSCell addition in FR1+FR2 NR-DC,

20ms. for NR PSCell change when source and target cells are in the same FR for EN-DC/NR-DC,

40ms, for NR PSCell change when source and target cells are in different FRs for EN-DC/NR-DC.

In current spec TS36.133, Tprocessing is

20ms, for NR FR1 PSCell addition in EN-DC,

40ms, for NR FR2 PSCell addition in EN-DC,

The Tprocessing for PSCell addition in HO with PSCell is proposed to reuse the values in current spec, i.e. 20ms for from SA to EN-DC, from EN-DC to EN-DC, and from NR-DC to NR-DC when NR Cells before and after handover are in the same FR, and for from NE-DC to NE-DC; 40ms for from SA to EN-DC, from EN-DC to EN-DC, and from NR-DC to NR-DC when NR Cells before and after handover are in different FRs.

Proposal 7: 20ms for the cases that PSCells before and after handover with PSCell are in the same FR; 40ms for the cases that PSCells before and after handover with PSCell are in different FRs.

Issue 2-2-5: Ending point of the delay requirement for HO with PSCell

It is FFS with 3 options in WF [1]. We think these options are not contradictory. The UE will meet all delay requirements for PCell handover by transmitting PRACH preamble on target PCell, and for PSCell addition by transmitting PRACH preamble on target PSCell for handover with PSCell. The requirements can be defined as “the UE shall be capable to transmit PRACH preamble towards target PCell no later than…., and shall be capable to transmit PRACH preamble towards target PSCell no later than….”, not need clearly defining ending point of the delay requirement for HO with PSCell.

Proposal 8: The requirements are defined as “the UE shall be capable to transmit PRACH preamble towards target PCell no later than…., and shall be capable to transmit PRACH preamble towards target PSCell no later than….”, not need clearly defining ending point of the delay requirement for HO with PSCell.

Issue 2-2-8: Delay requirements design

As above discussion, we can define delay requirements for HO and PSCell addition/change separately with the ending points defined as PCell PRACH and PSCell PRACH respectively, and require UE meet all these delay requirements. The suggested requirements can be found in our draft CR [2-3].

Proposal 9: The delay requirements for handover are defined as PCell handover delay requirements and PScell addition delay requirements respectively and the suggested context can be found in our draft CR [2-3].

Issue 2-3-2a: Interruption requirements, similar as Tinterrupt for in legacy handover requirements, for HO with PSCell

There are options 1a/1b/2 in WF [1]. We think option 1a and 1b is aligned and it should be consensus for this issue, i.e. interruption in legacy handover delay requirement can be applied for PCell, and no interruption is defined for PSCell addition.

Proposal 10: Interruption in legacy handover delay requirement can be applied for PCell, and no interruption is defined for PSCell addition.

Issue 2-3-2b: Interruption requirements on PCell/PSCell due to PSCell/PCell RF retuning

As above discussion, the RF chain of PSCell can be turned on together with PCell RF chain for parallel processing, or after UE get PCell timing information and before random access process for sequential processing. The UE is not scheduled on PCell, and no interruption will occur on PCell due to PSCell/PCell RF retuning. So it doesn’t need to define interruption requirement.

Proposal 11: No interruption requirement should be defined during HO with PSCell.

Issue 2-4-1: 2 step and 4 step RACH for HO with PSCell

As above discussion, the ending point is UE transmits PRACH on PCell and PSCell respectively for parallel processing. No different requirements for 2 step and 4 step RACH for handover with PSCell is needed. For sequential process, the PSCell addition process should start before random access process, the requirements will not be impacted by 2 step or 4 step RACH. So we think that the requirements defined for handover with PSCell will be applied for both 2-step RA and 4-step RA. No need to mention it in the specification.

Proposal 12: The requirements defined for handover with PSCell will be applied both 2-step RA and 4-step RA. No need to mention it in the specification.

Issue 2-4-2: RACH occasion collision between PCell and PSCell

We prefer the option 1 in WF [1]. It is FFS how to capture the extra uncertain delay in spec. We think the UE behaviour for PRACH collision is defined in RAN1 specification (TS38.213). It is not specific issue for handover with PSCell and no need to be included in the delay requirement for handover with PSCell. But in the requirement for handover with PSCell, a clarification can be added that additional uncertainty delay will be expected when PSCell PRACH collision with PCell PRACH.

Proposal 13: The requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for this case.

Issue 2-4-4: CSI-RS based CFRA

We prefer not considering CSI-RS based CFRA for handover with PSCell in this WI.

Proposal 14: Do not consider CSI-RS based CFRA for handover with PSCell in this WI.

Based on above proposals, the initial draft CRs on TS38.133 and TS36.133 for requirements for HO with PSCell are presented in another two papers for discussion [2][3].

- Conclusion

This document discussed the topic of HO with PSCell and presented following proposals:

Proposal 1: For handover with PSCell from NR SA to EN-DC, Parallel processing shall be the baseline for delay requirements. Sequential processing will be considered only when the NR PSCell to be added is an unknown cell, and the handover command carries NR PSCell SMTC information with E-UTRAN target PCell timing reference for UE.

Proposal 2: The delay requirements for handover will be defined based on sequential processing only when the NR PSCell to be added is an unknown cell, and the handover command carries NR PSCell SMTC information with target PCell timing reference for UE, otherwise, the requirements will be defined based on parallel processing.

Proposal 3: Slightly prefer defining different requirements for parallel processing cases and sequential processing cases.

Proposal 4: For both parallel processing cases and sequential processing cases, UE SW processing and RF warm-up for PCell handover and PSCell addition/change are assumed performed in parallel.

Proposal 5: Tprocessing for HO with PSCell = max(Tprocessing for PCell HO, Tprocessing for PSCell addition/change)

Proposal 6: The Tprocessing for HO with PSCell is sum Tprocessing for PCell HO and Tprocessing for PSCell addition/ change, or equal to Tprocessing for PCell HO for sequential processing.

Proposal 7: 20ms for the cases that PSCells before and after handover with PSCell are in the same FR; 40ms for the cases that PSCells before and after handover with PSCell are in different FRs.

Proposal 8: The requirements are defined as “the UE shall be capable to transmit PRACH preamble towards target PCell no later than…., and shall be capable to transmit PRACH preamble towards target PSCell no later than….”, not need clearly defining ending point of the delay requirement for HO with PSCell.

Proposal 9: The delay requirements for handover are defined as PCell handover delay requirements and PScell addition delay requirements respectively and the suggested context can be found in our draft CR [2-3].

Proposal 10: Interruption in legacy handover delay requirement can be applied for PCell, and no interruption is defined for PSCell addition.

Proposal 11: No interruption requirement should be defined during HO with PSCell.

Proposal 12: The requirements defined for handover with PSCell will be applied both 2-step RA and 4-step RA. No need to mention it in the specification.

Proposal 13: The requirement for handover with PSCell will be defined for no collision of PSCell PRACH with PCell PRACH, and adding clarification that additional uncertainty delay can be expected for this case.

Proposal 14: Do not consider CSI-RS based CFRA for handover with PSCell in this WI.

An initial draft CRs on TS38.133 and TS36.133 for requirements for HO with PSCell is presented in another papers for discussion [2][3].

- Reference

[1]	R4-2115337, WF on further RRM enhancement for NR and MR-DC – HO with PSCell, vivo

[2]	R4-2117328, Requirements for HO with PSCell, CATT

[3]	R4-2117329, Requirements for HO with PSCell for EN-DC, CATT