3GPP TSG-RAN WG4 Meeting #101-e                                                                                     R4-2117389

Electronic Meeting, November 1 - 12, 2021

Source: 	CATT

Title: 	Discussion on the BS TX RF requirements for 52.6-71GHz

Agenda item:	8.16.4.1

Document for:	Approval

- Introduction

In RAN4#100-e, a WF [1] on BS RF TX requirements for 52.6 – 71 GHz was approved. TAE, OBUE and ACLR, and spurious emissions need to be further discussed

In this contribution, we provide our views on the remaining issues in the WF.

- Discussion

TAE

As per the WF [1], the remaining issues concerning TAE are shown as below:

Continue discussion on TAE requirements in next meeting

The TAE is defined between the different reference symbols on different beams, For example, the two layer transmissions are configured at +45 polarized beam and -45 polarized beam, which are parallel beams instead of one beam. And the EIRP accuracy is defined for one beam. So EIRP accuracy can’t guarantee the TAE.

Observation 1: The EIRP accuracy can’t guarantee the TAE.

Figure 2-1: The impact of TAE on throughput for 960 kHz SCS

The simulation results with different TAEs for two layers transmission without HARQ are shown in Figure 2-1.  And the link level simulation assumptions for TAE are shown in Table A5-1 in annex. From Figure 2-1, it is observed that,

  - For 10ns TAE for 960kHz SCS, the throughput can reach 70% of maximum throughput.

  - For 20ns, 35ns and 65ns TAE for 960kHz SCS, the throughput is difficult to reach 70% of maximum throughput. Especially for 35ns and 65ns TAE, throughput @ 35dB SNR is close to 0.

From simulation result, the 20ns, 35ns and 65ns TAEs result in obvious performance degradation for DL two layers transmission for 960kHz SCS. So the existing FR2-1 TAE 65ns isn’t applicable for 52.6-71GHz.

Observation 2: Existing FR2-1 TAE 65ns isn’t applicable for 52.6-71GHz.

OBUE

As per the WF [1], the remaining issues concerning OBUE are shown as below:

For OBUE, further discuss opportunities to re-use or adapt either ETSI BRAN or FR2-1 OBUE requirements to be used for FR2-2 in next meeting.

In ETSI EN 302 567 V2.2.0[4] and Draft ETSI EN 303 722 V0.0.11 (2021-06)[5], the boundary between the spurious domain and the out-of-band domain for 60GHz is defined as min(2,5 × nominal channel BW, 1,5 × nominal channel BW + 500 MHz), so the ΔfOBUE for 52.6GHz can be defined as 1.5*BWcontiguous + 500 MHz = 3500MHz. Therefore, we propose to specify ΔfOBUE = 3500 MHz.

Proposal 1: To specifyΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

To compare OBUE differences among FR2-1 in TS38.104, ITU-R[2] and ETSI BRAN[3], the category A OBUE for FR2-1 with F/BW are summarized in Table 2-1, and the category B OBUE for FR2-1 with F/BW can be summarized to Table 2-2a, Table 2-2b.

Table 2-1: The category A OBUE for FR2-1 with F/BW

| Frequency offset of measurement filter -3B point, f | F/BWcontiguous | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 F/BWcontiguous < 0.6 | -5dBm | 1 MHz |
| 0.1*BWcontiguous f < fmax | 0.6 F/BWcontiguous < 0.5+ΔfOBUE /BWcontiguous | -13dBm | 1 MHz |

Table 2-2a: The category B OBUE for FR2-1 with F/BW, when BWcontiguous ≤ 500 MHz

| Frequency offset of measurement filter -3 dB point, f | F/BWcontiguous | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 F/BWcontiguous < 0.6 | -5dBm | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.6 F/BWcontiguous < 2 | -13dBm | 1 MHz |
| fB f < fmax | 2 F/BWcontiguous < 0.5+ΔfOBUE /BWcontiguous | -15dBm | 1MHz |

Table 2-2b: The category B OBUE for FR2-1 with F/BW, when BWcontiguous > 500 MHz

| Frequency offset of measurement filter -3 dB point, f | F/BWcontiguous | Limit | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1*BWcontiguous | 0.5 F/BWcontiguous < 0.6 | -5dBm | 1 MHz |
| 0.1*BWcontiguous f < fB | 0.6 F/BWcontiguous < 1+ 500 MHz /BWcontiguous | -13dBm | 1 MHz |
| fB f < fmax | 1+ 500 MHz /BWcontiguous F/BWcontiguous <0.5+ΔfOBUE /BWcontiguous | -15dBm | 1MHz |

Figure 2-2:  OBUE mask comparison between FR2 approach with proposed ΔfOBUE = 3500 MHz,

ITU-R and ETSI BRAN

The category A OBUE with proposedΔfOBUE = 3500 MHz, the category B OBUE for 52.6GHz with proposed ΔfOBUE = 3500 MHz, the ITU-R recommended mask for single carrier case, the ETSI BRAN transmit emission mask for c1 and c3, and the ETSI BRAN transmit emission mask for c2 are shown in Figure 2-2. From figure 2-2, the followings are observed,

- OBUE mask of ITU-R is similar with ETSI BRAN transmit emission mask for c1 and c3. The minor difference is between F/BW=0.44 and F/BW=0.56..

- When F/BW >0.56, For ITU-R, the max F/BW for ITU-R is 2.5, the mask emission at 2.5 F/BW is -30dBr. The acquired ACLR for 70GHz for co-exist in TR 38.803 is 23.5dB, so, in-band zone, the -30dBr is unnecessary. The ITU-R OBUE mask is unnecessary.

- When F/BW<0.56, the mask using FR2 approach is between ITU-R mask and ETSI BRAN mask for c2. So the FR2 approach when F/BW<0.56 is more reasonable.

- When F/BW >0.56, FR2 approach (category B), the max F/BW for is 2.25, the mask emission at 2.25 F/BW is -15dBr. For ETSI BRAN for c2, the max F/BW is 2.5, the mask emission at 2.5F/BW is -22dBr.  So OBUE mask of the ETSI BRAN for c2 is 7dB stringent than OBUE mask of FR2 approach (category B), which means BS products need more linearization. This increases difficulty of BS products implementation.

From the above analysis, the OBUE mask of FR2 approach is relax than that of ITU-R and ETSI BRAN, to reduce difficulty of BS products implementation, we think OBUE mask for 52.6-71GHz doesn’t need to be tightened.  Therefore, we propose that the FR2 approach (category B) can be reused for FR2-2.

Proposal 2: Re-use the FR2-1 OBUE requirement with ΔfOBUE = 3500 MHz for 52.6- 71 GHz.

Spurious emissions

As per the WF [1], the remaining issues concerning spurious emissions are shown as below:

Continue discussion and down-selection of the following options in next meeting. Option 1: Use FR2 approach with necessary adaptations on step size for spurious emissions. Option 2: Use ETSI EN 303 722 and/or 303 753 for spurious emissions.

Figure 2-3: FR2 approach spurious emission mask for frequency >=18GHz[6].

For option 1, the FR2 approach for frequency >=18GHz is referenced to Figure 7 in ERC Recommendation 74-01[6], the spurious emission mask is shown in Figure 2-3.

According to Table 9 and Figure 7 in Annex 2 of ERC Recommendation 74-01, the step frequencies can be calculated by the following equation.

WB = FDL,high-FDL,low

Fstep,1 = max(18, FDL,low-max(10,4*WB))

Fstep,2 = FDL,low-max(2,WB)

Fstep,3 = FDL,low-ΔfOBUE

Fstep,4 =FDL,high+ΔfOBUE

Fstep,5 =FDL,high+max(2,WB)

Fstep,6 =FDL,high+max(10,4*WB)

Using above equations for step frequencies, the step frequencies for 66-71GHz, and 57-71GHz with proposedΔfOBUE = 3500 MHz are show in Table 2-3.

Table 2-3: Step frequencies for 66-71GHz, and 57-71GHz with proposedΔfOBUE = 3500 MHz

| Operating band | Fstep,1 | Fstep,2 | Fstep,3 | FDL,low | FDL,high | fOBUE | WB | Fstep,4 | Fstep,5 | Fstep,6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) | (GHz) |
| n257 | 18 | 23.5 | 25 | 26.5 | 29.5 | 1.5 | 3 | 31 | 32.5 | 41.5 |
| n258 | 18 | 21 | 22.75 | 24.25 | 27.5 | 1.5 | 3.25 | 29 | 30.75 | 40.5 |
| n259 | 23.5 | 35.5 | 38 | 39.5 | 43.5 | 1.5 | 4 | 45 | 47.5 | 59.5 |
| n260 | 25 | 34 | 35.5 | 37 | 40 | 1.5 | 3 | 41.5 | 43 | 52 |
| n261 | 18 | 25.5 | 26 | 27.5 | 28.35 | 1.5 | 0.85 | 29.85 | 30.35 | 38.35 |
| nxxx (lic) | 46 | 61 | 62.5 | 66 | 71 | 3.5 | 5 | 74.5 | 76 | 91 |
| nyyy (unlic) | 18 | 43 | 53.5 | 57 | 71 | 3.5 | 14 | 74.5 | 85 | 127 |

For option 2, the spurious emission defined in section 4.2.3.2 of ETSI EN 303 722 are shown as the below:

#### 4.2.3.2 Limit The level of unwanted emissions in the spurious domain shall be less than or equal to the limits given in table 4,where the lower boundary between the spurious domain and the out-of-band domain shall be at a frequency FL: FL = min (57 GHz; fC - min(2,5 × nominal channel BW, 1,5 × nominal channel BW + 500 MHz)) where fC is the nominal centre frequency of the transmission. The upper boundary between the spurious domain and the out-of-band domain shall be at a frequency FH: FH = max (71 GHz; fC + min(2,5 × nominal channel BW, 1,5 × nominal channel BW + 500 MHz Table 4: Transmitter unwanted emissions in the spurious domain Frequency range; Emission Limit ERP (≤ 1 GHz) EIRP (> 1 GHz); Measurement Bandwidth. 30 MHz to 47 MHz; -36 dBm; 100 kHz. 47 MHz to 74 MHz; -54 dBm; 100 kHz. 74 MHz to 87,5 MHz; -36 dBm; 100 kHz. 87,5 MHz to 118 MHz; -54 dBm; 100 kHz. 118 MHz to 174 MHz; -36 dBm; 100 kHz. 174 MHz to 230 MHz; -54 dBm; 100 kHz. 230 MHz to 470 MHz; -36 dBm; 100 kHz. 470 MHz to 694 MHz; -54 dBm; 100 kHz. 694 MHz to 1 GHz; -36 dBm; 100 kHz. 1 GHz to FL GHz; -30 dBm; 1 MHz. FH GHz to 142 GHz; -30 dBm; 1 MHz. Note: See ERC REC 74-01 [i.7].

Comparing the spurious emission mask of FR2 approach and ETSI EN 303 722, the metric of emission limit for frequency >1GHz in ETSI EN 303 722 is EIRP instead of TRP used in FR2 approach in TS 38.104, the -30dBM/1MHz EIRP is more stringent than -30dBm/1MHz(-20dBm/10MHz) TRP. So the spurious emission limit of FR2-1 requirement is relax than that of ETSI EN 303 722, considering large path loss for 52.6GHz, the spurious emission limit doesn’t need to be tightened. So FR2 approach with necessary adaptations on step size for spurious emissions can be re-used for 52.6-71GHz, we propose option1. And the step frequencies for spurious emissions are shown in table 2-3.

Proposal 3: Use FR2 approach with necessary adaptations on step size for spurious emissions (option 1)

- Conclusion

This contribution provides analysis on BS TX RF requirements for 52.6-71GHz. The following proposals and observations are concluded as follows:

Observation 1: The EIRP accuracy can’t guarantee the TAE.

Observation 2: Existing FR2-1 TAE 65ns isn’t applicable for 52.6-71GHz.

Proposal 1: To specifyΔfOBUE = 3500 MHz for FDL,high – FDL,low > 4000 MHz for 52.6-71GHz.

Proposal 2: Re-use the FR2-1 OBUE requirement with ΔfOBUE = 3500 MHz for 52.6- 71 GHz.

Proposal 3: Use FR2 approach with necessary adaptations on step size for spurious emissions (option 1)

- References

- R4-2115643,WF on BS TX RF requirements, Nokia, Nokia Shanghai Bell, RAN4#100-e

- Recommendation ITU-R M.2003-2, “Multiple Gigabit Wireless Systems in frequencies around 60 GHz”, ITU-R

- ERC Recommendation 70-03, “Relating to the use of Short Range Devices (SRD)”, CEPT.

- Final draft ETSI EN 302 567 V2.2.1 (2021-05),Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,ETSI

- Draft ETSI EN 303 722 V0.0.11 (2021-06),Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 GHz to 71 GHz band; Harmonised Standard for access to radio spectrum, ETSI

- ERC Recommendation 74-01, unwanted emissions in the spurious domain, ECC

- Annex

The link level simulation assumptions for TAE are shown in Table A5-1.

Table A5-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 60GHz |
| CBW | 400MHz |
| SCS | 960kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | MCS 16 in 64QAM table of TS 38.214 |
| Pre-coding | Fixed PMI |
| Symbol type | CP-OFDM |
| HARQ | NO |
| RANK | 2 |
| BS antenna configuration | 2 |
| UE antenna configuration | 2 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 0, Duration 14 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | NO |
| PN model | NO |
| PN compensation | NO |