3GPP TSG RAN WG4 Meeting #101-e	R4-2117515

Online, 1st – 12th Nov. 2021

Agenda item:	7.8.2

Source:	SoftBank Corp.

Title:	TP for TR 38.717-03-01: NR CA_n1-n28-n41

Document for:	Approval

1	Introduction

3B NR CA of CA_n1-n28-n41 was approved in RAN#93[1]. This TP is to capture the basic aspects for NR CA.

2.  Reference

[1]  RP-212239  Revised WID: Rel-16 NR inter-band CA for 3 bands DL with 1 band UL

Annex.

[Unchanged Parts Skipped]

6.X	CA_n1-n28-n41

6.X.1	Operating band for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n28-n413 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |  |
| CA_n1A-n28A-n41A | - | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  |
| NOTE 3: The SCS of each channel bandwidth for NR band refer to Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.X.3	Co-existence studies

Co-existence studies can be omitted because harmonic interference have been already studied for 2DL/1UL fallback combinations such as CA n1-n28, CA_n1-n41 and n28-n41.

6.X.4	∆TIB and ∆RIB values

For CA_n1-n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 6.X.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n28-n41 | n1 | 0.5 |
|  | n28 | 0.6 |
|  | n41 | 0.5 |

Table 6.X.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n1-n28-n41 | n1 | 0 |
|  | n28 | 0.2 |
|  | n41 | 0 |

6.X.5	REFSENS requirements

Based on 6.X.3, there are no additional MSD requirements for this band combination.

[Unchanged Parts Skipped]