3GPP TSG-RAN WG4 Meeting#101      	R4-2117550

E-meeting, 1st – 12th Nov, 2021

Title:	TP to FWA TR General NR UE Tx requirements

Source:	Nokia

Agenda item:	7.28.3.2

Document for:	Approval

# 1	Introduction

In this contribution we discuss common NR Tx UE RF requirement for FWA HPUE. Discussion is built on work done during SI and captured into TR [1]. TP is proposed.

# 2	Discussion

## 2.1	General

From the WID[2] we can see the necessary common objectives the WI needs to address.

Common Objectives:

- UE MOP, MPR, and ACLR, considering the interactions between them (e.g. MRP and ACLR) and other related parameters

This contribution addresses these issues for NR.

## 2.2	Maximum output power NR

So far PC1 for NR has been defined for n14 only. Relevant NR PC1 bands will be added into this table.

However n14 has only 5 and 10 MHz channel bandwidths therefore we know that current MPR is applicable only for PC1 operation with 5 and 10 MHz channel bandwidths therefore before new NR bands can be added to specification MPR analysis is needed for channel bandwidths > 10 MHz.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| NOTE 6: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

## 2.3	MPR NR

NR MPR for PC 1 is defined for n14 only.

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | ≤ 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

MPR values in Table 6.2.2-5 are same as MPR for PC3. Because PC1 MPR is valid only for n14 operation it currently applies only for 5 and 10 MHz channel bandwidths. In E-UTRA PC3 MPR applies to all channel bandwidths but because NR has higher spectrum utilization and larger channel bandwidths a study is needed.

From Table below we can see the NR SU for different channel bandwidths.

| SCS (kHz) | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz | 35 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB |  | NRB | NRB | NRB | NRB |  | NRB | NRB |  |
| 15 | 25 |  | 52 | 79 | 106 | 133 |  | 160 | 188 |  |
| 30 | 11 |  | 24 | 38 | 51 | 65 |  | 78 | 92 |  |
| 60 | N/A |  | 11 | 18 | 24 | 31 |  | 38 | 44 |  |
| SCS (kHz) | 40 MHz | 45 MHz |  | 50 MHz | 60 MHz |  | 70 MHz | 80 MHz |  | 90 MHz |  |
|  | NRB | NRB |  | NRB | NRB |  | NRB | NRB |  | NRB |  |
| 15 | 216 | 242 |  | 270 | N/A |  | N/A | N/A |  | N/A |  |
| 30 | 106 | 119 |  | 133 | 162 |  | 189 | 217 |  | 245 |  |
| 60 | 51 | 58 |  | 65 | 79 |  | 93 | 107 |  | 121 |  |

It is encouraged to provide MPR proposals at least for 15 and 20 MHz channel bandwidths.

## 2.4	ACLR NR

NR ACLR for band n14 is defined to be 37 dB which is same as for E-UTRA PC1. This value of 37 dB can be applied to all bands and channel bandwidths once MPR studies are complete.

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 11 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |
| NOTE 1: Applicable for power class 1 UE operating in Band n14. |  |  |  |  |

# 3	Conclusion

In this contribution we have discussed UE MOP, MPR, and ACLR for FWA HPUE and provide a TP to TR 37.828.

# 4 	References

[1] https://www.3gpp.org/ftp/Specs/archive/37_series/37.880/37880-h10.zip

[2] RP-212533, Revised WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands, Nokia, RAN#93e

******************************** Start of the TP ********************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	3GPP TS 38.101-1: “User Equipment (UE) radio transmission and reception;Part 1: Range 1 Standalone”

******************************** No change ********************************

# 6	Common requirements for a UE

Editor note: This section relates to the common objectives of the WI.

## 6.1	Maximum output power

### 6.1.2	NR

So far PC1 for NR has been defined for n14 only. Relevant NR PC1 bands will be added into Table 6.2.1-1 in [3].

However n14 has only 5 and 10 MHz channel bandwidths therefore we know that current MPR is applicable only for PC1 operation with 5 and 10 MHz channel bandwidths therefore before new NR bands can be added to specification MPR analysis is needed for channel bandwidths > 10 MHz.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ±2 |
| n14 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| NOTE 6: Generally, PC1 UE for Band n14 is not targeted for smartphone form factor. The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. |  |  |  |  |  |  |  |  |

## 6.2	MPR

### 6.2.2	NR

NR MPR for PC 1 is defined for n14 only in Table 6.2.2-5 in [3].

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | ≤ 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

MPR values in Table 6.2.2-5 in [3] are same as MPR for PC3. Because PC1 MPR is valid only for n14 operation it currently applies only for 5 and 10 MHz channel bandwidths. In E-UTRA PC3 MPR applies to all channel bandwidths but because NR has higher spectrum utilization and larger channel bandwidths a study is needed.

From Table below we can see the NR SU for different channel bandwidths.

| SCS (kHz) | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz | 35 MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB |  | NRB | NRB | NRB | NRB |  | NRB | NRB |  |
| 15 | 25 |  | 52 | 79 | 106 | 133 |  | 160 | 188 |  |
| 30 | 11 |  | 24 | 38 | 51 | 65 |  | 78 | 92 |  |
| 60 | N/A |  | 11 | 18 | 24 | 31 |  | 38 | 44 |  |
| SCS (kHz) | 40 MHz | 45 MHz |  | 50 MHz | 60 MHz |  | 70 MHz | 80 MHz |  | 90 MHz |  |
|  | NRB | NRB |  | NRB | NRB |  | NRB | NRB |  | NRB |  |
| 15 | 216 | 242 |  | 270 | N/A |  | N/A | N/A |  | N/A |  |
| 30 | 106 | 119 |  | 133 | 162 |  | 189 | 217 |  | 245 |  |
| 60 | 51 | 58 |  | 65 | 79 |  | 93 | 107 |  | 121 |  |

#### 6.2.2.1	MPR studies

## 6.3	ACLR

### 6.3.2	NR

NR ACLR for band n14 is defined to be 37 dB in Table 6.5.2.4.1-2 in [3] which is same as for E-UTRA PC1.

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 11 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |
| NOTE 1: Applicable for power class 1 UE operating in Band n14. |  |  |  |  |

******************************** End of the TP ********************************