3GPP TSG-RAN WG4 Meeting #101-e	R4-2117668

# Elecrtronic Meeting,  Nov. 1 - 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | XXXX | rev | - | Current version: | 16.9 0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications and corrections on additional UE co-ex requirements for 2 Band UL CA/DC(R16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SoftBank Corp., |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the texts added in the last meeting, some clarifications and corrections are needed: - Applicability of 2 UL bands text and NS_xxU/100 should be made clear.. - Applicability texts for a single Band UL are duplicated. - In R16-, two Band UL applicability is also duplicated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicability of additional requirements are clarified and relevant text is added to NOTE 1 for NS_xxU/100. One of the duplicated text for a single Band UL is removed. One of the duplicated two band UL applicability text is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relevant requirements remain unclear/duplicated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR38.521-3 ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Unaffected Portions Skipped]

##### 6.2A.3.1.3	UE additional maximum output power reduction for Inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in subclause 6.2.3 apply.

Unless otherwise stated, for inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the additional maximum output power reduction requirements for intra-band contiguous carrier aggregation in subclause 6.2A.3.1.1 apply for that band.

Unless specified in Table 6.2A.3.1.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.1.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR.Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

The emission requirements specified in Table 6.2A.3.1.3-1 also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.2A.3.1.3-1: Additional Requirements for uplink inter-band carrier aggregation (two-bands)

| NR CA combination | Band | Applied NS | Requirements (clause) | A-MPR (table/clause) | Note |
| --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
|  | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 |  |
| CA_n1-n8 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
|  | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 |  |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n1-n28 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1,2 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
|  | n28 | 17 | 6.5.3.3.2 | N/A |  |
| CA_n1-n40 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
| CA_n1-n41 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n1-n78 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
| CA_n1-n79 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
| CA_n3-n8 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
|  | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 |  |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n3-n28 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1,2 |
|  | n28 | 17 | 6.5.3.3.2 | N/A |  |
| CA_n3-n40 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n3-n41 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n3-n77 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n3-n78 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n3-n79 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n5-n77 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n5-n78 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n5-n79 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n8-n40 | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 | 1 |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n8-n41 | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 | 1 |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n8-n78 | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 | 1 |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n8-n79 | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 | 1 |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n28-n40 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
| CA_n28-n41 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n28-n77 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
| CA_n28-n78 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
| CA_n40-n41 | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n41-n78 | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n41-n79 | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and the requirements in clause 6.5.2.4.2 are only applicable to the signalling carrier. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |  |  |

[Unaffected Portions Skipped]