3GPP TSG-RAN WG4 Meeting #101-e                                     R4-2117816

Electronic Meeting, 1st-12th November, 2021

Source: 	Xiaomi

Title: 	Discussion on timing requirements for RedCap UE

Agenda Item:	8.20.3.1.3

Document for:	Discussion

# 1 Introduction

In last RAN4#100-e meeting, discussion on RRM requirements for RedCap UE was conducted and a way forward was agreed in [1]. In this contribution, we provide our consideration on the timing requirements for RedCap UE.

- Discussion

Impact on UE transmit timing requirements

- Proposal 1 (Apple, Huawei, CMCC, Oppo, Vivo, Ericsson, Nokia): The existing UE transmit timing requirements (including Te and Tq) can be applied for RedCap UE. - Proposal 2 (Qualcomm): A RedCap UE shall meet the Te requirements for an initial transmission provided that at-least one SSB is available in the active DL BWP of the UE during the last 160ms.

UE transmit timing requirements, e.g. Te and Tq, depend on the reception timing evaluation error and transmission sampling rate. For Rel-17 RedCap UE, the physical design makes no change, we think the Rel-16 UE transmit timing requirements could be reused. For Option2 listed above, there exists the case that the SSB is not available in the active DL BWP for legacy UE, and we don’t think it necessary to set an additional restriction for RedCap UE.

Proposal 1: The existing UE transmit timing requirements (including Te and Tq) can be applied for RedCap UE.

Reference cell timing in RedCap BWP

- Proposal 1 (Qualcomm Incorporated, CMCC): When a RedCap UE is operating in a RRC configured DL BWP without CORESET0 or SSB (FG 6-1a), CSI-RS/TRS may be used to acquire the reference cell timing. - Proposal 2 (Huawei, Apple, Vivo, Ericsson, MTK, Nokia): Need more conclusion from RAN1.

For the acquirement of cell timing, in our knowledge, this issue is still under discussion in RAN1. RAN4 would wait for RAN1’s decision.

Proposal 2: RAN4 would wait for RAN1’s final decision about using CSI-RS/TRS to acquire the reference cell timing.

- Conclusion

Proposal 1: The existing UE transmit timing requirements (including Te and Tq) can be applied for RedCap UE.

Proposal 2: RAN4 would wait for RAN1’s final decision about using CSI-RS/TRS to acquire the reference cell timing.

- Reference

[1] R4-2115358 WF on RedCap RRM requirements, Ericsson