3GPP TSG-RAN WG4 Meeting # 101-e	R4-2117873

# Electronic Meeting, November 1-12, 2021

Source:	Huawei, HiSilicon

Title:	Link simulation for support of 1024QAM

Agenda item:	8.6.3

Document for:	Discussion

1 Introduction

In RAN#89 e-meeting, a new work item, i.e. introduction of DL 1024-QAM for NR FR1 was approved [1]. The objective is to specify downlink 1024-QAM for NR PDSCH operation in FR1, together with related procedures, signaling and necessary RF requirements. The RAN4 objectives are listed below.

- Specify corresponding UE and BS RF core requirements [RAN4]

  - UE and BS RF core requirements are specified for stationary wireless scenarios with up to 2 layer DL MIMO

  - The cell size(s) and type of stationary wireless scenarios for which UE and BS RF core requirements are defined will be studied and decided by RAN4.

In last meeting, link simulations for DL 1024QAM were provided in [4]. In this contribution, we provide more simulation results considering the agreed WF [5].

2 Discussion

The link level simulation assumptions are listed as in Table 2-1.

Table 2-1 link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| CBW | 40 MHz |
| SCS | 15kHz |
| Allocated RBs | Full allocation |
| Propagation | TDL-A 10ns delay spread, Maximum Doppler frequency: 5Hz TDL-D 10ns delay spread, Maximum Doppler frequency: 5Hz |
| MCS | 256QAM: MCS 24 in TS 38.214 Table 5.1.3.1-2: MCS index table 2 for PDSCH, and other MCSs are not precluded 1024QAM: MCS 24 in the following Table accroding to the agreement in RAN1 #104, and other MCSs are not precluded MCS Index; Modulation Order; Target code Rate R x [1024]; Spectral. IMCS; Qm; efficiency. 23; 10; 805.5; 7.8662. 24; 10; 853; 8.3301. 25; 10; 900.5; 8.7939. 26; 10; 948; 9.2578. |
| Precoding | Precoding configuration defined in 38.101-4 Section 7.2 for fading channels; follow PMI |
| Symbol type | CP-OFDM |
| HARQ | 8, None |
| RANK | 1 and 2 |
| BS antenna configuration | 1, 2 and 8 |
| UE antenna configuration | 4 |
| Antenna correlation (Tx and Rx) | Low correlation |
| Channel estimation | Practical |
| Receiver type | MMSE |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| txEVM | 2%, 2.5%, 3% |
| rxEVM | 1%, 1.5%, 2%, 3% |

In last meeting for TX EVM requirements, 3 options were under discussion and intent to make decision on RAN4#101 meeting.

- Option 1: 2.5% for all classes

- Option 2: 2.5% for LA and MR classes; 2.8% for WA class

- Option 3: 2.8% for all BS classes

We update the simulation to include the case for 2.8% TX EVM.

The spectral efficiency simulation results for 1T4R are shown in figure 2-1, figure 2-2, and figure 2-3 for different TX/RX EVM assumptions for TDL-A.

Figure 2-1 SE for TDL-A RX EVM =1%

Figure 2-2 SE for TDL-A RX EVM =1.5%

Figure 2-3 SE for TDL-A RX EVM =2%

The spectral efficiency simulation results for 2T4R are shown in figure 2-4, figure 2-5, and figure 2-6 for different TX/RX EVM assumptions for TDL-A.

Figure 2-4 SE for TDL-A RX EVM =1%

Figure 2-5 SE for TDL-A RX EVM =1.5%

Figure 2-6 SE for TDL-A RX EVM =2%

The spectral efficiency simulation results for 8T4R are shown in figure 2-1, figure 2-2, and figure 2-3 for different TX/RX EVM assumptions for TDL-A.

Figure 2-7 SE for TDL-A RX EVM =1%

Figure 2-8 SE for TDL-A RX EVM =1.5%

Figure 2-9 SE for TDL-A RX EVM =2%

The summary of throughput gain of 1024 QAM to 256 QAM (@32dB) is shown in Table 2-2

Table 2-2

| Channel | rxEVM txEVM | 1% | 1.5% | 2% |
| --- | --- | --- | --- | --- |
| TDL-A, 1T4R | 2% | 25.51% | 25.36% | 25.23% |
| TDL-A, 1T4R | 2.5% | 24.32% | 24.05% | 23.62% |
| TDL-A, 1T4R | 2.8% | 22.14% | 21.7% | 21.19% |
| TDL-A, 1T4R | 3% | 21.83% | 21.21% | 20.38% |
| TDL-A, 2T4R | 2% | 25.56% | 25.31% | 24.94% |
| TDL-A, 2T4R | 2.5% | 23.28% | 23.05% | 22.67% |
| TDL-A, 2T4R | 2.8% | 21.12% | 20.91% | 20.40 |
| TDL-A, 2T4R | 3% | 19.28% | 18.52% | 17.84% |
| TDL-A, 8T4R | 2% | 23.97% | 23.85% | 23.69% |
| TDL-A, 8T4R | 2.5% | 22.69% | 22.47% | 22.08% |
| TDL-A, 8T4R | 2.8% | 21.27% | 20.98% | 20.45% |
| TDL-A, 8T4R | 3% | 19.96% | 19.57% | 19.17% |

From the simulation results above, it is observed,

Observation: from link level simulation, for RANK1 even 3% TX EVM can provide observed gain for 1024 QAM compared to 256 QAM.

3 Conclusion

In this contribution, we provide link level simulation results for 1024-QAM.

Observation: from link level simulation, for RANK1 even 3% TX EVM can provide observed gain for 1024 QAM compared to 256 QAM.

Reference

- “RAN1 Chairman’s Notes”, RAN1#103-e, October 26th – November 13th, 2020.

- RP-202866, “Revised WID on Introduction of DL 1024-QAM for NR FR1”, Nokia, RAN#90e, December 7-11, 2020.

- R4-1706112, Reply LS regarding 1024QAM, RAN4

- R4-2113047, Link simulation for support of 1024QAM, Huawei, HiSilicon

- R4-2115638, WF on 1024QAM requirements, Ericsson