3GPP RAN WG4 Meeting #101-e		R4-2117971

Electronic meeting, November 1 - 12, 2021

Agenda item:	9.1.1.1

Source:	Apple

Title:	Preliminary uncertainty assessment for the high DL power and low UL power objective

WI/SI:	FS_FR2_enhTestMethods

Release:	Rel-17

Document for:	Discussion and Decision

# 1	Introduction

This contribution provides our views on the remaining open issues with the preliminary uncertainty assessment for Objective 1 (high DL power and low UL power).

# 2	Discussion

Following the approval of the latest text proposal on low PSD test cases, the evaluation of error statistics (mean and standard deviation) has concluded for a large number of scenarios.  We attempt to begin to classify these in order to derive the corresponding MU element descriptions and values.

EIRP error of CFFDNF (with black&white box approach)

-	Table 5.1.4.4-1: Statistical results of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets and the probe pattern/gain were compensated

-	Table 5.1.4.4-2: Statistical results of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets were compensated while the probe pattern/gain was not compensated.

-	Table 5.1.4.4-3: Statistical results of 100k vs 1k vs 250 EIRP CFFDNF offset simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3)/10cm (PC1) in a single hemisphere. The antenna array offsets were compensated.

-	Table 5.1.4.4-5: Statistical results of 28GHz vs 49GHz EIRP CFFDNF offset simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3)/10cm (PC1) in a single

-	Table 5.1.4.4-6: Statistical results of EIRP CFFDNF simulations performed with Matlab and CST.

-	Table 5.1.4.4-7: Statistical results of 100,000 EIRP CFFDNF simulations based on black&white-box approach with random antenna array offsets uniformly spaced within 12.5cm (PC3)/10cm (PC1) in a single hemisphere. The antenna array offsets were compensated.

-	Table 5.1.4.4-8: CFFDNF simulation results utilizing black&white-box with antenna array offset and feed antenna pattern compensated.

-	Table 5.1.4.5-1: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black&white-box approach with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets and probe pattern were compensated.

-	Table 5.1.4.5-2: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black&white-box approach with random 12x12 antenna array offsets uniformly spaced within 10cm in a single hemisphere. The antenna array offsets and probe pattern were compensated.

EIRP error of CFFNF (with black box approach)

-	Table 5.1.4.6-1: Statistical results of 500 EIRP CFFNF & CFFDNF CST simulations based on black- box, black&white-box approach with random 8x2 and 12x12 antenna array offsets uniformly spaced within 12.5cm for 8x2 and within 10cm for 12x12 in a single hemisphere. The antenna array offsets and probe pattern were compensated.

-	Table 5.1.4.6-2: Statistical results of 500 EIRP CFFNF (black box) simulations with random 8x2 antenna array offsets uniformly spaced within 12.5cm in a single hemisphere. The antenna array offsets and probe pattern were compensated.

Sensitivity of CFFNF to relative MU

-	Table 5.1.4.8-1: Impact of power measurement uncertainty ui applied to p(d) measurements on FF EIRP for fixed offsets.

-	Table 5.1.4.8-2: Impact of measurement uncertainty ui applied to p(d) measurements on FF EIRP with random offsets

Influence of noise on CFFDNF and CFFNF

-	Table 5.1.4.9-1: SNR Improvement due to reduced measurement distance w.r.t. 1m DFF/IFF FSPL with fixed noise at TE Input

-	Table 5.1.4.9-2: Effective SNRs at the TE Input based on assumed SNR at the TE input for DFF/IFF

-	Table 5.1.4.9-3: Influence of Noise Simulations and Calculations for DFF/IFF, CFFDNF, CFFNF based on shortest measurement distance dmin

-	Table 5.1.4.9-4: Influence of Noise Simulations and Calculations for DFF/IFF, CFFDNF, CFFNF based on largest measurement distance dmax

Offset error MU

-	Table 5.1.4.10-1: Statistical results of 1M EIRP CFFDNF simulations to determine the effect of offset declaration error on EIRP/EIS

In an attempt to summarize the MU element descriptions, references to the related analysis, and an extract of applicable values, Tables 1 and 2 below summarize our understanding for two range lengths (L=22cm and L=35cm).

Table 1: Summary of MU element analysis for L=22 cm

| UID | Description of uncertainty contribution | Ref in TR38.884 | CFFNF |  | CFFDNF |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean error | STD of error | Mean error | STD of error |  |
| 1 | DUT antenna location estimation | Table 5.1.4.9-1 | reuse CFFDNF values? |  | [0.07 - 0.13] | 0.35 | Assuming error in declared offset = 1 cm (worst-case of available results) |
| 2 | Probe antenna pattern | Table 5.1.4.4-3 | reuse CFFDNF values? |  | 0.48 | 0.22 |  |
| 3 | EIRP measurement error | Table 5.1.4.5-1 | 0.04 | 0.04 | 0.42 | 0.19 |  |
| 3b | TRP measurement error (uniform grid) | Table 5.1.4.4-9 | N/A | N/A | 0.01 | 0.04 |  |
| 3c | TRP measurement error (non-uniform grid) | Table 5.1.4.4-12 | N/A | N/A | 0.05 | 0.05 |  |
| 4 | Near-field interaction between probe antenna and DUT antenna | TBD | TBD | TBD | TBD | TBD | Analysis of near-field interaction is not available in the TR |
| 5 | Influence of power measurement uncertainty | Tables 5.1.4.7-1, 5.1.4.7-2 | 0.16 | 0.44 | 0.16 | 0.44 | Worst-case assuming N=30 and ui=0.4 |
| 6 | Influence of noise | Table 5.1.4.8-5 | 3.6 |  | 5.6 |  | Assuming SNR = -15 dB |

Table 3: Summary of MU element analysis for L=35 cm

| UID | Description of uncertainty contribution | Ref in TR38.884 | CFFNF |  | CFFDNF |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean error | STD of error | Mean error | STD of error |  |
| 1 | DUT antenna location estimation | Table 5.1.4.9-1 | reuse CFFDNF values? |  | [0.07 - 0.13] | 0.16 | Assuming error in declared offset = 1 cm (worst-case of available results) |
| 2 | Probe antenna pattern | Table 5.1.4.4-3 | reuse CFFDNF values? |  | 0.09 | 0.02 |  |
| 3 | EIRP measurement error | Table 5.1.4.5-1 | 0.03 | 0.04 | 0.14 | 0.04 |  |
| 3c | TRP measurement error (uniform grid) | Table 5.1.4.4-9 | N/A | N/A | 0.02 | 0.02 |  |
| 3c | TRP measurement error (non-uniform grid) | Table 5.1.4.4-9 | N/A | N/A | 0.06 | 0.04 |  |
| 4 | Near-field interaction between probe antenna and DUT antenna | TBD | TBD | TBD | TBD | TBD | Analysis of near-field interaction is not available in the TR |
| 5 | Influence of power measurement uncertainty | Tables 5.1.4.7-1, 5.1.4.7-2 | 0.16 | 0.44 | 0.16 | 0.44 | Worst-case assuming N=30 and ui=0.4 |
| 6 | Influence of noise | Table 5.1.4.8-5 | 4.0 |  | 6.9 |  | Assuming SNR = -15 dB |

Observation 1:	Although the TR has captured the MU element "Near-field interaction between probe antenna and DUT antenna," the related analysis is not explicitly identified in the TR section dedicated to Objective 1.

Observation 2:	Along with the EIRP measurement error MU element, another element is needed to quantify TRP measurement error; further discussion is needed to to capture the uniform/non-uniform grid applicability.

Observation 3:	Given the extensive analysis of the influence of noise on both the CFFNF and CFFDNF systems, this MU element should be included in the study's outcome.

Proposal 1:	In an effort to conclude the preliminary uncertainty assessment of Objective 1, it is proposed to take the above observations into account and to agree on a clear table of MU element applicability to the two methods (CFFNF/CFFDNF) and related methodology parameters (e.g. grid spacing)

# 3	References

- RP-211600, “Study on enhanced test methods for FR2,” Apple, vivo, Intel, 3GPP RAN #92, June 2021

- RP-211504, "Status report for the Study on enhanced test methods for FR2; rapporteur: Apple," 3GPP RAN #92, June 2021

- R4-2115820, " TR38.884 Study on enhanced test methods for FR2 NR UEs v1.1.0," Apple, vivo, Intel, 3GPP RAN4 #100, August 2021

- Email discussion notes