3GPP RAN WG4 Meeting #99-e	R4-2107985

Electronic meeting, May 19 – 28, 2021

# Title:	[DRAFT] LS on 60 GHz time-related issues with further updates

# Response to:	R4-2104451 (R1-2102202) LS on beam switching gap for 60 GHz band

# Release:	Rel-17

# Work Item:	NR_ext_to_71GHz

Source:	Apple (to be RAN4)

To:	RAN1

Cc:

Contact Person:

Name:	Anatoliy Ioffe

Tel. Number:

E-mail Address:	aioffe@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN4 would like follow up on the previous LS sent to RAN1 in response to their LS on beam switching gap for 60 GHz band (please see R4-2107985).  RAN4 has discussed the related issues further and would like to share the following update, as summarized in Table 1 below.

Table 1: Summary of 60 GHz time-related issues and agreements

| Issue | Agreement |
| --- | --- |
| RX-TX and TX-RX beam switching | - For NR operation in the 52.6 – 71 GHz range, the Rx-Tx and Tx-Rx transition time shall reuse the FR2 value of 13792 Tc. (7.015 us) for 120/480/960 kHz SCS |
| Minimum duration between beam switches | - RAN4 understood that this operation might include switching across panels, which is similar to triggering the OFF/ON transition - RAN4 does not see the need to define this parameter as a requirement, and the OFF/ON transition time can be assumed |
| UE Beam switch time (beam direction switch only) | - RAN4 does not see the need to define this parameter as a requirement, and the FR2-1 value of 200 ns can be assumed |
| UE Inter-panel Beam switch time (beam direction switch only) | - Depends on conclusion of the intra-panel beam switch time and analysis of delays in addition to intra-panel, if any, associated with inter-panel beam switch time |
| gNB Beam switch time (beam direction switch only) | - RAN4 tentatively agrees [59 ns] with the understanding that the value can be confirmed once open issues related to BS output power are resolved |
| TX ON-OFF transient period | - Re-use UE transient time from FR2-1 (5 us) for 120/480/960 kHz SCS |
| TX ON-ON transient period | - Re-use UE transient time from FR2-1 (5 us) for 120 kHz SCS - Consider and discuss further a UE capability (can be dependent on 480/960 SCS support and can be linked to UE type) which is < 5 us - RAN4 assumes that the ON-ON transient period is applicable to all power change and allocated BW change procedures |

RAN4 respectfully asks RAN1 to take this information into account in their work.

2. Actions:

None

3. Date of Next RAN WG4 Meetings:

RAN WG4 Meeting #101-bis-e	17 - 25 January, 2022		Electronic meeting

RAN WG4 Meeting #102-e	21 February - 3 March, 2022	Electronic meeting