3GPP TSG RAN WG4 Meeting #101-e	R4-2118062

Electronic meeting, 01st -12th November, 2021

Agenda item:	7.14.2

Source:	LG Electronics, KDDI, Softbank

Title:	Self interference analysis and MSD results for PC3 NR inter-band CA band combinations

Document for:	Approval

1	Introduction

In the latest RAN Plenary, some new CA band are included to support PC3 NR inter-band combinations in Rel-17 as follow [1].

Table 1: PC3 NR inter-band CA/DC band combinations in Rel-17

| NR CA configuration | Uplink CA configuration | Harmonic issue in 3rd band | IMD problem in 3rd band |
| --- | --- | --- | --- |
| CA_n1A-n18A-n28A | CA_n1A-n28A | - | 5th IMD into n18 |
|  | CA_n18A-n28A | - | 5th IMD into n1 |
| CA_n1A-n18A-n41A | CA_n1A-n41A | - | 5th IMD into n18 |
| CA_n18A-n28A-n41A | CA_n18A-n28A | - | 5th IMD into n41 |
|  | CA_n18A-n41A | - | 5th IMD into n28 |
| CA_n18A-n41A-n77A | CA_n41A-n77A | - | 2nd & 3rd IMDs into n18 |
| CA_n18A-n41A-n78A | CA_n41A-n78A | - | 2nd IMD into n18 |
| CA_n1A-n18A-n77A | CA_n1A-n77A | - | 5th IMD into n18 |
| CA_n3A-n77A-n79A | CA_n77A-n79A | - | 4th & 5th IMD into n3 |

Based on the revised WID, we analyse the self interference problems for the new DC band combinations.

2	MSD analysis

2.1 IMD problems by dual uplink CA/DC

In this session, we analyze the required MSD values in the own Rx band by IMD product from dual uplink transmission. In table 2 and Table 3, we show the basic RF simulation assumptions.

Table 2: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. or dual ant. |  |  |  | Cascaded Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CA_n18-n41-n77, CA_n18-n41-n78, CA_n1-n18-n77, CA_n3-n77-n79 |  |  |  | CA_n1-n18-n28, CA_n1-n18-n41, CA_n18-n28-n41 |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |  |  |  |  |
| Quadplexer |  |  |  |  | 112 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 3 show the isolation levels according to the RF component.

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

Based on above assumptions and test configuration, we proposed the MSD levels as below.

Table 4: Proposed MSD test configuration and results by IMD problems for NR inter-band CA UE

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n18A-n28A | n1 | IMD5 | ¦fn1 -4*fn28¦ | 1965 | 5 | 25 | 2155 | 5 | N/A |
|  | n28 |  |  | 708 | 5 | 25 | 763 | 5 |  |
|  | n18 |  |  | 822 | 5 | 25 | 867 | 5 | 4.6 |
|  | n18 | IMD5 | ¦fn18 -4*fn28¦ | 825 | 5 | 25 | 870 | 5 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 5 |  |
|  | n1 |  |  | 1937 | 5 | 25 | 2127 | 5 | 4.0 |
| CA_n1A_n18A-n41A | n1 | IMD5 | ¦3*fn1 -2*fn41¦ | 1960 | 5 | 25 | 2150 | 5 | N/A |
|  | n41 |  |  | 2505 | 10 | 50 | 2505 | 10 |  |
|  | n18 |  |  | 825 | 5 | 25 | 870 | 5 | 3.3 |
| CA_n18A-n28A-n41A | n18 | IMD5 | ¦4*fn18 -fn28¦ | 825 | 5 | 25 | 870 | 5 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 5 |  |
|  | n41 |  |  | 2562 | 10 | 50 | 2562 | 10 | 4.4 |
|  | n18 | IMD5 | ¦4*fn18 -fn41¦ | 825 | 5 | 25 | 870 | 5 | N/A |
|  | n41 |  |  | 2505 | 10 | 50 | 2505 | 10 |  |
|  | n28 |  |  | 740 | 5 | 25 | 795 | 5 | 3.9 |
| CA_n18A_n41A-n77A | n41 | IMD2 | ¦fn41 -fn77¦ | 2590 | 10 | 50 | 2590 | 10 | N/A |
|  | n77 |  |  | 3460 | 10 | 50 | 3460 | 10 |  |
|  | n18 |  |  | 825 | 5 | 25 | 870 | 5 | 29.3 |
| CA_n18A_n41A-n78A | n41 | IMD2 | ¦fn41 -fn78¦ | 2590 | 10 | 50 | 2590 | 10 | N/A |
|  | n78 |  |  | 3460 | 10 | 50 | 3460 | 10 |  |
|  | n18 |  |  | 825 | 5 | 25 | 870 | 5 | 29.3 |
| CA_n1A_n18A-n77A | n1 | IMD5 | ¦3*fn1 -2*fn77¦ | 1970 | 5 | 25 | 2160 | 5 | N/A |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10 |  |
|  | n18 |  |  | 825 | 5 | 25 | 870 | 5 | 3.5 |
| CA_n3A_n77A-n79A | n77 | IMD4 | ¦2*fn77 -fn79¦ | 3350 | 10 | 50 | 3350 | 10 | N/A |
|  | n79 |  |  | 4840 | 40 | 216 | 4840 | 40 |  |
|  | n3 |  |  | 1765 | 5 | 25 | 1860 | 5 | 15.7 |

4	Conclusion

Above simulation results, we recommend to merged the other interested companies’ simulation results as average manner to derive MSD levels and propose as follow

Proposal 1: For IMD problem by dual uplink transmission of PC3 NR inter band CA UE, the proposed MSD values in Table 4 shall be considered in TR38.717-03-02 and TS38.101-1.

Reference

[1] RP-212512, “Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL” ZTE

[2] TR38.717-03-02 v0.6.0