TSG-RAN Working Group 4 (Radio) meeting #101-e	  R4-2118133

Electronic meeting, 1 – 12 November 2021

Source:	Ericsson

Title:	SRS antenna switching with antenna virtualization

Agenda item:	8.7.3.1

Document for:	Approval

# Background

Power reductions due to the insertion loss the ∆TRxSRS up to 6/7.5 dB can significantly degrade quality of CSI at the gNB, differences in the UE antenna element peformance not accounted for. This implies unbalanced antenna measurements for DL CSI estimation and can lead to degraded DL MIMO performance.

In this contribution we revisit the specification of the Pcmax for SRS usage set to antenna switching, and the allowed ∆TRxSRS in particular.

In [1] the following agreement on SRS switching and TxD was captured

- In GTW, the following were agreed

  - SRS antenna switching which was targeted for DL CSI would not use UL antenna virtualization, i.e. UL TxD

  - SRS antenna switching functionality cannot be excluded for UE supporting TxD.

with the following targets

- Introduce PC1.5 to spec

- Explicit introduce TxD for SRS antenna switching IL, but how to harmonize with the current SRS conditions are FFS, and the exact IL values are FFS

- At least following PC2 UE architectures with TxD but without antenna virtualization for all antenna ports are to be analyzed in #100e

  - 23PA+23PA

  - 26PA+23PA

  - 26PA+26PA

- At least 1T2R, 1T4R, 2T4R and 1T4R/2T4R srs-TxSwitch are to be analyzed in #100e

- A big CR will be used to capture the agreement in #100e together with other TxD issues.

In this contribution we propose specification of the configured power for SRS transmission occasions with antenna switching for UEs supporting

- TxD capability [TxDiversity-r16]

- PC1.5

- ULFPTx modes 0, 1 and 2

according to the following principles

- the UE should not virtualize the SRS, but instead transmit on each 'receive’ chain

- the routing loss captured by the ∆TRxSRS is indepent of the power class with one exception for the 23PA+26PA implementation and transmissions on one SRS port.

# Configured maximum power for SRS

The SRS antenna switching for the purpose of DL CSI acquisistion should give an indication of the antenna strength. A UE indicating TxD and implemented with two half-power PAs should therefore not virtualize the antennas, which would add further power ambiguity if only one of the said PAs is switched to the receiver (R) antenna connectors. The switching should yield an accurate relative difference between the antenna connectors (the antenna gains not considered).

The configured maximum output power for SRS transmissions is also given by the standard expression

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

where

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2'

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4'

c)	UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', or

d)	UE transmits SRS to a DL-only carrier

The value of ∆TRxSRS is 4.5dB for n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 3 in the band, or when the device is capable of power class 2 in the band and ΔPPowerClass = 3 dB.  The value of ∆TRxSRS is 7.5dB for n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 2 and 1.5 in the band and ΔPPowerClass = 0 dB.

For other SRS transmissions ∆TRxSRS is zero;

First a few observations on the existing specification. The 3/4.5 dB value of ∆TRxSRS accounts for additional antenna routing loss for RX antennas, presumably assumed to be the same per RX connector, but also depends on the power class.

∆TRxSRS should be maximum allowance due to routing loss for the RX antennas and not depend on the power class. Now, the 6/7.5 dB maximum allowance for PC2 apparently already covers implementations based on e.g. TxD but would not be relevant for switching in a band supported by a single full-power PA supporting PC2. The power-class dependence of ∆TRxSRS only further increases the ambiguity in the power levels of the SRS transmissions from each antenna connector.

Ideally, the SRS switching should be carried out with one full-power PA, Alternatively, for vitualization, a half-power PA is switched across all antenna ports including the TX port, which implies a 3 dB reduction of the maximum power of all SRS transmissions (case of two TX chains), e.g. for transmission of all SRS resources for 1T4R.

Proposal 1: the ∆TRxSRS is a maximum allowance due to additional routing loss for RX antennas, the same value for all power classes (but can be band dependent)

Assuming that a TxD capable UE is equipped with two half-power PA, then

A. For one-port SRS

For TxD: add 3 dB reduction for the “first port”, since the UE cannot virtualize Tx chains when transmitting SRS during SRS switching without destroying its value for determining CSI for downlink MIMO.  Therefore, a TxD UE should be assumed to transmit the SRS port on the “T” port with one half-power PA as well as on the ‘R’ port

A UE configured for UL full power Mode 1 should also use the updated value of ∆TRxSRS independent of the power class with 3 dB reduction added for the “first” port, since Mode 1 is designed to support (although not require) half-power PAs for 2 Tx.

A UE configured for UL full power Mode 2 should again use the updated value of ∆TRxSRS with 3 dB reduction added for the “first” port unless the UE supports one or more full power TPMIs (or TPMI groups).  Mode 2 without the full power TPMIs is designed to support (although not require) half power PAs for 2Tx.

B. For two-port SRS

No changes are needed (always one half-power per T/R-connector and SRS port), the same for full power PAs, the power should be split across SRS ports.

For PC1.5, the allowance would be the same as for UEs indicating TxD or a UE indicating full power Mode 1.

The 3 dB additional reduction of the SRS maximum power for switching with a half-power PA should be accounted for by setting ΔPPowerClass = 3 dB for TxD and ULFPTx capable UEs except Mode 2 supporting full-power TPMI. This means the the upper limit PCMAX_H,f,c is reduced by 3 dB and the lower PCMAX_L,f,c by 3 + ∆TRxSRS dB, thus specifying an output-power requirement not based on that the SRS port is virtualized with antenna switching.

Proposal 2: for UEs indicating txDiversity-r16 (TxD) and ULFPTx except for Mode 0 and Mode 2 supporting full-power TPMI, ΔPPowerClass = 3 dB for single-port SRS transmissions with usage set to ‘antennaSwitching’

Mode 0 also supports full-power PAs.

An allowance ∆TRxSRS up to 6/7.5 dB is needed for implementations with 23PA + 26PA for accommodating switching using two PAs of different power rating. A relaxation is only motivated for single-port SRS transmissions, for two-port transmissions the SRS power shall be split between the ports no matter the PA capability. Instead of the blanket 6 dB allowance for PC2 we propose the following

Proposal 3: for UEs indicating ULFPTx Mode 2 supporting full-power TPMI, a exceptional ∆TRxSRS = 6/7.5 dB allowed for single-port transmissions; for two-port SRS transmissions ∆TRxSRS = 3/4.5 dB.

In order to faciliate gNB measurements of SRS antenna strength, a capability inidcating ∆TRxSRS = 0 dB or indication of different SRS transmission power across T- and R-connectors for ULFPTx Mode 2 could be considered. We therefore propose that

Proposal 4: a capability indicating ∆TRxSRS = 0 dB or an indication of different SRS transmission power across T- and R-connectors for ULFPTx Mode 2 could be considered with a view to avoid exceptions and faciliate measurements of SRS antenna strength.

The downgrading capabilities for 1T2R, 1T4R and 2T4R should also be included in the specification of the configured power for SRS antenna switching, from 38.331:

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch       ENUMERATED {t1r2, t1r4, t2r4, t1r4-t2r4, t1r1, t2r2, t4r4, notSupported},

txSwitchImpactToRx              INTEGER (1..32)                            OPTIONAL,

txSwitchWithAnotherBand         INTEGER (1..32)                            OPTIONAL

}                                                                              OPTIONAL

}

BandParameters-v1610 ::=         SEQUENCE {

srs-TxSwitch-v1610               SEQUENCE {

supportedSRS-TxPortSwitch-v1610  ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL

}

In view of the above, we propose the following changes [2]

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’ when

a)	UE transmits SRS on the second SRS resource in every configured SRS resource set when the SRS-TxSwitch capability is indicated as 't1r2', ‘t1r1-t1r2’, ‘t1r1-t1r2-t1r4’ or ‘t1r1-t1r2-t2r2-t1r4-t2r4’  with all the configured SRS resources in the SRS resource set(s) consisting of one SRS port, or

b)	UE transmits SRS on the second, third and fourth SRS resources of the total 4 SRS resources from all configured SRS resource set(s) consisting of one SRS port when the SRS-TxSwitch capability is indicated as 't1r4' or, 't1r4-t2r4', ‘t1r1-t1r2-t1r4’ or ‘t1r1-t1r2-t2r2-t1r4-t2r4’ with all the configured SRS resources in the SRS resource set(s) consisting of one SRS port, or

c)	UE transmits SRS from the second SRS port pair on the second SRS resource in every configured SRS resource set consisting of two SRS ports when the SRS-TxSwitch capability is indicated as ' t2r4' or ' t1r4-t2r4', ‘t1r1-t1r2-t2r2-t2r4’ or ‘t1r1-t1r2-t2r2-t1r4-t2r4’ with all the configured SRS resources in the SRS resource set(s) consisting of two SRS ports, or

d)	UE transmits SRS to a DL-only carrier

For a UE indicating [txDiversity-r16] or a UE supporting ul-FullPwrMode1-r16 or ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 but not ul-FullPwrMode2-TPMIGroup-r16 or indicating support of power class 1.5, ΔPPowerClass = 3 dB during SRS transmission occasions with configured SRS resources in the SRS resource set(s) consisting of one SRS port.

The value of ∆TRxSRS is 4.5dB for n79 and 3 dB for bands whose FUL_high is lower than the FUL_low of n79. For a UE supporting ul-FullPwrMode2-TPMIGroup-r16, the value of ∆TRxSRS is 7.5dB for n79 and 6 dB for bands whose FUL_high is lower than the FUL_low of n79 when the device is capable of power class 2 during SRS transmission occasions with configured SRS resources in the SRS resource set(s) consisting of one SRS port,.

For other SRS transmissions ∆TRxSRS is zero;

# UE behaviour

Specifying in 38.214 that the UE should not virtualize the SRS with usage set to antenna switching is not stratightforward. The condition ΔPPowerClass = 3 dB for TxD in the specification of the configured maximum power in 38.101-1 at least specify an output power requirement not based on antenna virtualization.

# Proposal

For the configured maximum output power with antenna switching, we propose that

Proposal 1: the ∆TRxSRS is a maximum allowance due to additional routing loss for RX antennas, the same value for all power classes (but can be band dependent)

Proposal 2: for UEs indicating txDiversity-r16 (TxD) and ULFPTx except for Mode 0 and Mode 2 supporting full-power TPMI, ΔPPowerClass = 3 dB for single-port SRS transmissions with usage set to ‘antennaSwitching’

with one exception

Proposal 3: for UEs indicating ULFPTx Mode 2 supporting full-power TPMI, a exceptional ∆TRxSRS = 6/7.5 dB allowed for single-port transmissions; for two-port SRS transmissions ∆TRxSRS = 3/4.5 dB.

Exceptions could be avoided and SRS antenna measurements be faciliated by introducing

Proposal 4: a capability indicating ∆TRxSRS = 0 dB or an indication of different SRS transmission power across T- and R-connectors for ULFPTx Mode 2 could be considered with a view to avoid exceptions and faciliate measurements of SRS antenna strength.

If not ensuring, the above provisions are at least impeding UE implementations virtualizing the SRS ports with switching. An accompanying CR is provided in [2].

# References

1.     R4-2107981, “Way Forward on SRS antenna switching requirements for TxD”, OPPO

2.     R4-2118134, “Correction to Pcmax for SRS usage set as antenna switching”, Ericsson, Apple