3GPP TSG-RAN WG4 Meeting #101-e                                                         R4-2118186

# Electronic Meeting, Nov. 1-12, 2021

Source:	ZTE Corporation

Title:	Discussion on reply LS on Clarification on exception requirements for Intermodulation due to Dual uplink (IMD)

Agenda Item:	12.1

Document for:	Approval

- 1	Introduction

A LS from RAN5 on clarification on exception requirements for Intermodulation due to Dual uplink (IMD) was agreed [1], which requires the following actions from RAN4:

2. Actions: To RAN4 group. ACTION: RAN5 kindly asks RAN4 group to clarify if the EN-DC IMD exceptions are applicable only when the IMD product falls into the victim carrier, and if SA requirements apply otherwise in the case of 2UL. Also, to clarify the criteria that need to be fulfilled in order for MSD=0 to apply.

- In the past meetings, two questions for clarification were included in the RAN4 discussion.

- Clarification on Q1: If the EN-DC IMD exceptions are applicable only when the IMD product falls into the victim carrier, and if SA requirements apply otherwise in the case of 2UL.

- Clarification on Q2: Clarify the criteria that need to be fulfilled in order for MSD=0 to apply.

- For Clarification on Q1, it was agreed in [2] as follow:

- Answer: Yes, SA requirements shall be applied for dual UL carrier frequency combinations when no IMD product (up to 5th orders) falls into the victim’s Rx CBW and no other desensitization components are present, i.e. due to 1) harmonics (UL harmonic or Receiver harmonic mixing), 2) cross-band isolation, 3) counter-intermodulation (C-IM).

- However, no consensus for Clarification on Q2 due to different understanding among companies.

- In this contribution, we provide some discussions on this issue, and draft LS is attached at the end of the contribution.

- 2	Discussion

The candidate options for the answer to Clarification on Q2  are listed in [2] as follow:

- Answer 1: In RAN4 specs, no general criteria is defined in which REFSENS can be fulfilled with MSD=0 for the EN-DC combinations which have MSD exceptions due to IMD interference (2 UL active). However, whether it is meaningful to do this analysis is up to RAN5.

- Answer 2: MSD=0 could be only applied when carrier frequencies and bandwidths are selected for each active UL band such that there is no any interference falling into Rx CBW under all the conditions in Question 1. However, whether it is meaningful to do this analysis is up to RAN5.

In our understanding, there are no general criteria is defined on MSD=0 value. It is hard to say MSD=0 when there is no any interference falling into Rx CBW. Actually the IMD MSD for FDD-TDD band combiantion would be defined when there are intermodulation products falls into its own Rx frequency range. However, sometime MSD may not be defined in RAN4 spec if the MSD is negligible such as <0.5dB or there are no proper test point, or equals to N/A pending on the operator spectrum holding. Moreover, RAN4 only define up to IMD5 MSD (2 bands UL active), which means it will not check the MSD requirements if there exist >5th order IMD interference caused by 2 bands ULs [3]. Therefore, for all of these cases it could be seen that MSD=0 from RAN4 spefication perspective, however it doesn’t mean there is no any interference.

Even, although there are MSD defined in RAN4 spec for a certain ENDC band combination, negligible MSD could still happen during the test considering the actually different RF architectures (such as separate antenna implementation with higher PCB isolation) and more advanced technology RF components are adopted. Therefore, how to do this analysis is up to RAN5.

In summary, we prefer Answer 1 for clarification on Q2.

- 3	Conclusion

In this paper, we provide some discussions on the issue on  exception requirements for Intermodulation due to Dual uplink (IMD). Comparing with the two answers, we prefer Answer 1 for clarificarion on Q2.

In addition, draft LS is attached in the Annex.

- 4	Reference

- R5-211609, Clarification on exception requirements for Intermodulation due to Dual uplink (IMD), TSG RAN5

- R4-2115049, Email discussion summary for [100-e][149] NR_reply_LS_UE_RF, Moderator (Apple)

- R4-2112915, Discussion on reply LS on Clarification on exception requirements for Intermodulation due to Dual uplink (IMD), ZTE

- Annex

3GPP TSG-RAN WG4 Meeting # 101-e 	R4-21xxxxx

# Electronic Meeting, Nov. 1-12, 2021

Title:	DRAFT Reply LS on Clarification on exception requirements for Intermodulation due to Dual uplink (IMD)

Response to:	(R5-211609) Clarification on exception requirements for Intermodulation due to Dual uplink (IMD)

Release:	Release 15

Work Item:	5GS_NR_LTE-UEConTest

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact Person:

#### Name:	Wubin, Zhou

E-mail Address:	zhou.wubin@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN4 would like to thank RAN5 for the LS in R5-211609 for clarification on exception requirements for Intermodulation due to Dual uplink (IMD). RAN4 has discussed the issues raised by RAN5 and concluded the following answers:

The EN-DC IMD exceptions for dual UL carrier frequency combinations are only applicable when the IMD product  (up to 5th orders) falls into the victim’s Rx carrier, and

SA requirements shall be applied for dual UL carrier frequency combinations when no IMD product (up to 5th orders) falls into the victim’s Rx CBW and no other desensitization components are present, i.e. due to 1) harmonics (UL harmonic or Receiver harmonic mixing), 2) cross-band isolation, 3) counter-intermodulation (C-IM).

Moreover, there are no general criteria defined In RAN4 specs in which REFSENS can be fulfilled with MSD=0 for the 	EN-DC combinations which have MSD exceptions due to IMD interference (2 UL active). However, whether it is meaningful to do this analysis is up to RAN5.

2. Actions:

To RAN5 group.

ACTION: 	RAN4 respectfully asks RAN5 to take the above information into account.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting#101-bis-e 	   1st – 17th Jan 2022	Electronic Meeting

TSG-RAN4 Meeting#102-bis-e 	   21st  Feb– 3rd Mar 2022	Electronic Meeting