3GPP TSG-RAN WG4 Meeting #101-e                                                             R4-2118189

Electronic Meeting, Nov. 1-12, 2021

Source:	ZTE Corporation

Title:	TP for TR 36.717-03-02_CA_1A-3A-7A-20A-38A

Agenda Item:	10.5.3

Document for:	Approval

- Introduction

This contribution provides a text proposal on LTE CA_1A-3A-7A-20A-38A with the UL CA configuration of CA_1A-3A, CA_1A-20A and CA_3A-20A for TR36.717-03-02.

- Reference

[1] TR36.717-03-02, LTE-A inter-band CA for x bands (x=3,4,5) DL with 2 bands UL in Rel-17, v0.6.0

- Text Proposal

----- Start of TP -----

  - 8.x			LTE-A inter-band CA: Band 1 and Band 3 and Band 7 and Band 20 and Band 38 with 2 bands UL

    - 8.x.1	List of specific combination issues

      - 8.x.1.1	Channel bandwidths per operating band for CA

Table 8.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-7A-20A-38A | CA_1A-3A CA_1A-20A CA_3A-20A | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |

      - 8.x.1.2	Co-existence studies

For 2UL/5DL LTE CA band combination, the additional MSD caused by the intermodulation products of two ULs from CA_1A-3A, CA_1A-20A and CA_3A-20A falls into its own Rx bands such as band 3, band 7, band 20 or band 38 would be defined.

According to the co-existence of the 2ULs for CA_1A-3A, CA_1A-20A and CA_3A-20A, it can be found that:

- There are no IMDs products fall into own Rx of Band 7, band 20 and band 38 for 2UL of CA_1A-3A

-There are no IMDs products fall into own Rx of Band 3, Band 7 and Band 38 for 2UL of CA_1A-20A.

-There are no IMDs products fall into own Rx of Band 1, but 2rd IMDs products fall into own Rx of  Band7 and Band 38 for 2UL of CA_3A-20A.

      - 8.x.1.3	MSD

According to the co-existence studies analysis, when uplink CA configurations CA_1A-3A, CA_1A-20A and CA_3A-20A are paired with downlink CA configuration CA_1A-3A-7A-20A-38A, there are no additional analysis of MSD because they are solved in fallback band combination.

      - 8.x.1.4	∆TIB and ∆RIB values

The same requirements of fallback combination can be applied.

----- End of TP -----