3GPP TSG-RAN WG4 Meeting #101-e                                                             R4-2118191

Electronic Meeting, Nov. 1-12, 2021

Source:	ZTE Corporation

Title:	TP for TR 37.717-11-21:  DC_3A_n38A-n78A

Agenda Item:	7.18.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on  DC_3A_n38A-n78A for TR37.717-11-21[1] .

- Reference

- TR 37.717-11-21  Dual Connectivity (DC) of x bands (x=1, 2, 3, 4) LTE inter-band CA (x DL/1 UL) and 2 bands NR Inter-band CA  (2 DL/1 UL) band combinations, v0.6.0

- Text Proposal

----- Start of TP -----

  - 6.x		DC_3_n38-n78

    - 6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n38-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

    - 6.x.2	Configuration for DC

Table 6.x.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_n38A-n78A | DC_3A_n78A | 3A | CA_n38A-n78A |

    - 6.x.3	Co-existence studies

Co-existence analysis for DC_3_n78 UL have already been captured in TR 37.863-01-01, where there are no IMD produces may fall into NR Band n38 DL.

    - 6.x.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n38-n78 | 3 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n38-n78 | 3 | 0.5 |
|  | n38 | 0 |
|  | n78 | 0.5 |

    - 6.x.5	MSD

According to the co-existence studies, there are no additional MSD will be defined.

----- End of TP -----