[]3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2118256

# Electronic Meeting, Nov. 1-12, 2021

Title:	[DRAFT] LS on further agreements for RLM and BFD relaxation

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

Name:	Yanliang SUN

E-mail Address:	 yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #100e, LS R4-2115439 on criteria for RLM/BFD relaxation is sent to RAN2. In RAN4 #101e, further agreements are achieved in RAN4 as follows.

For the low mobility criterion,

- Support beam-level low mobility criterion for connected state UE as baseline, at least for the case when UE is configured with BFD relaxation. (to be updated in RAN4 #101e)

  - When the number of serving cell SSB/CSI-RS resources for L3 measurements is more than 1, then UE is expected to enter low mobility state only when all of the max 2 detected SSBs/CSI-RSs meet the low mobility criterion. (to be updated in RAN4 #101e)

[… To be updated based on conclusions in RAN4 #101e]

For the cell quality criterion,

- Use the offset value XdB over the Qout as the entering condition of cell quality criterion for RLM relaxation.

- Use the offset value YdB over the Qout_LR as the entering condition of cell quality criterion for BFD relaxation.

  - X and Y are configurable by network. (to be updated in RAN4 #101e)

[… To be updated based on conclusions in RAN4 #101e]

For the configuration of the relaxation criteria,.

- Low mobility state is configured on per-UE basis

  - UE needs only to identify low mobility state according to RRM measurements in the NR PCell for the case of NR single carrier, NR CA, NE-DC and NR-DC, and according to that in the NR PSCell for the case of EN-DC.

- Cell quality criterion is effective on per-cell basis, activated either

  - by explicit thresholds configuration on per-cell basis, or

  - by other broadcast/dedicated signalling on per-cell basis  (to be updated in RAN4 #101e)

[… To be updated based on conclusions in RAN4 #101e]

RAN4 respectfully ask RAN2 take above agreements into considerations and further discuss on corresponding RRC signalling for the above aspects agreed in RAN4

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 agreements in consideration and specify the corresponding signaling.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #101e-bis	17-25, Jan, 2022	Online