3GPP TSG-RAN WG4 Meeting # 101-e                                						R4-2118334

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-11-11: DC_n8A_3A

Agenda Item:	7.13

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of NR band n8 and LTE band3 for TR 37.717-11-11[1].

# Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.4.0

# Text Proposal

----- Start of TP -----

### 6.1.x	DC_n8A_3A

#### 6.1.x.1	Operating bands for DC

Table 6.1.37.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_n8A_3A | 3 | n8 | No |

#### 6.1.x.2	Configuration for DC

Table 6.1.37.2-1:  Inter-band NE-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_n8A_3A | DC_n8A_3A | 3A | n8A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.37.3-1: Maximum output power for inter-band NE-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance(dB) |
| --- | --- | --- |
| DC_n8A_3A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n8A_3A are the same as DC_3A_n8A, which have been already defined in the TS38.101-3 spec Table 6.5B.3.3.2-1.

#### 6.1.x.5	MSD analysis for DC

The MSD analysis for DC_n8A_3A are the same as DC_3A_n8A which is available in 3GPP TR 37.716-11-11 V2.0.0:

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 795 | 905 | 2590 | 2700 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 25 | 120 | 2505 | 2690 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3470 | 3615 | 4300 | 4485 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 855 | 1035 | 4215 | 4475 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4350 | 4530 | 6010 | 6270 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1810 | 1590 | 5180 | 5400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 5925 | 1950 | 1735 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3595 | 3300 | 675 | 930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7720 | 8055 | 5230 | 5445 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6890 | 7185 | 6060 | 6315 |

- 2nd order harmonic of n8 could fall on the band 3 downlink frequency range.

- IMD4 could fall on the n8 downlink frequency range.

- IMD4 could fall on the downlink frequency range of band 3 and n8.

6.1.x.6	MSD

MSD values for DC_n8A_3A are the same as DC_3A_n8A, which have been already defined in the TS38.101-3 spec.

#### 6.1.x.7	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for DC_n8A_3A are the same as DC_3A_n8A, which have already been defined in the TS38.101-3 spec.

#### 6.1.x.8		Self-interference analysis

#### The analysis of DC_n8A_3A are the same as DC_3A_n8A which is available in 3GPP TR 37.716-11-11 V2.0.0.

#### 6.1.x.9	OOB blocking exception requirements

No OOB blocking exception for DC_n8A_3A.

----- End of TP -----