3GPP TSG-RAN WG4 Meeting # 101-e	R4-2118364

# Electronic Meeting, Nov.1-12, 2021

Agenda Item:	8.11.2.1

Source:	OPPO

Title:	On pre-configured MG pattern(s) for NR_MG_enh

Document for:	Approval

- Introduction

A WF on pre-configured MG pattern was approved in last RAN4#100e-meeting [1]. In this contribution, we provide our views on the remaining issues for pre-configured MG pattern for RRM measurement gap enhancement in R17.

- Discussion

Whether can the pre-MG be used for PRS measurement?

- It is feasible to config Pre-MG for PRS measurements.

  - The exact configuration of Pre-MG used for PRS measurement can be FFS

    - FFS on whether UE assumes the Pre-MG is always activated

    - FFS if UE should always indicate serving cell about the PRS measurement when it is configured with pre-MG.

- It is up to NW to configure either Pre-MG which shall be always activated or legacy MG for PRS measurement

Whether can the pre-MG be used for CSI-RS L3 measurement?

- It is feasible to configure a Pre-MG for CSI-RS L3 inter-frequency measurement.

  - The exact configuration of Pre-MG used for CSI-RS L3 inter-frequency measurement can be FFS

    - FFS on whether UE assumes the Pre-MG is always activated

- It is up to NW to configure either Pre-MG which shall be always activated or legacy MG for CSI-RS L3 inter-frequency

In our view, if PRS measurement was always gap-needed, then UE assumes the Pre-MG is always activated. At least for Rel16 Positioning, the case is true. Whether measurement without gap is going to be supported for Rel17 Positioning is still under discussion in parallel. As baseline, we suggested in R17 Pre-MG used for PRS measurement can be assumed as always activated.

For CSI-RS L3 measurement, MG is always needed for inter-frequency measurement. Similarly, Pre-MG used for CSI-RS L3 inter-frequency measurement should be assumed as always activated. It is up to NW to configure Pre-MG or legacy MG for CSI-RS L3 inter-frequency measurement.

Proposal 1: In R17 Pre-MG used for PRS measurement can be assumed as always activated.

Proposal 2: Pre-MG used for CSI-RS L3 inter-frequency measurement should be assumed as always activated. It is up to NW to configure Pre-MG or legacy MG for CSI-RS L3 inter-frequency measurement.

Parameter to indicate the status (activation/deactivation) of Pre-MG at or during configuration - NW can control activation/deactivation of pre-configured MG - RRC based activation/deactivation method is supported - Network can indicate activation/deactivation status per BWP - Details of signalling are FFS - FFS if MAC CE based activation/deactivation method is supported - Companies are encouraged bring inputs on the conditions when NW based activation/deactivation and explicit rules for activation/deactivation are used.

Both NW signaling or per-defined rule are proposed by companies. As agreed, RRC based activation/deactivation method is supported for network-based activation/deactivation. In our view, MAC CE based activation/deactivation method may not bring any additional benefit but make the mechanism more complex, e.g., extra delay requirements for handling MAC CE processing.

Proposal 3: Not support MAC CE based activation/deactivation method for NW based activation/deactivation.

Relation of pre-configured MG and with the current legacy MG - Without considering concurrent MG and on top of the RRC parameter(s) used to differentiate Pre-MG with the legacy MG, - FFS whether additional mechanism is needed to switch the configured pre-MG to serve as the same functionaries as the legacy MG (MG is always on).

RAN4 agreed to start Pre-MG for single CC BWP switching scenario, and network can indicate activation/deactivation status per BWP. An LS has been already sent to RAN2 for RRC signaling design. Regarding the common configuration parameters of pre-configured MG (e.g. MGRP, MGL, etc) are same as these of Rel16 legacy MG, either the same or separated RRC signaling used for Pre-MG and legacy gap should be considered and further decided up to RAN2 conclusion. In our view, as Pre-MG is associated with BWP(s), it should be different with legacy MG configuration/RRC IE, especially considering the concurrent gap. A switch mechanism could be helpful to avoid the ambiguity of UE behavior.

Observation 1: A switch mechanism for the configured pre-MG to legacy MG could be helpful to avoid the ambiguity of UE behavior.

Whether additional mechanism to switch the configured pre-MG to legacy MG can be impacted by the RRC design of RAN2. So, we suggest to wait for RAN2’s LS reply.

Proposal 4: Wait for RAN2’s reply on RRC design of Pre-MG before deciding additional mechanism to switch the configured pre-MG to legacy MG.

Conditions for pre-configured MG activation/deactivation - FFS if additional conditions beside BWP switching for pre-configured MG activation/deactivation shall be considered - Option 1: pre-configured MG activation/deactivation can be triggered by finishing the following network commands and procedures: BWP switching, adding/removing any measurement object(s), adding/releasing/changing a PSCell, activating/de-activating any SCell(s). - Option 2: pre-configured MG activation/deactivation is triggered ONLY by the DCI/Timer based BWP switch in accordance with the WID objective # 1. - Other options not precluded

Though we agree the conditions mentioned in option 1, option 2 could be simple for current discussion regarding the progress.

Proposal 5: pre-configured MG activation/deactivation is triggered ONLY by the DCI/Timer based BWP switch in accordance with the WID objective # 1.

As agreed, pre-configured MG(s) and legacy MG(s) can be configured simultaneously needs to be considered for concurrent gaps operation at 2nd stage. Whether multiple Pre-MGs can be configured, needs to be discussed. If it is the case, configured Pre-MGs with different gap patterns can be activated/deactivated after BWP switching.

For single Pre-MG, RAN4 agreed its gap configuration can NOT be changed after BWP switching. But for multiple Pre-MGs, the gap configuration could be different, which can be changed with activation/deactivation of different Pre-MG for the same BWP. In short, for one active BWP, network can indicate different Pre-MG (e.g., different MG patterns) with its activation/deactivation status after BWP switching.

Observation 2: For Pre-MG in concurrent gaps operation, network can indicate different Pre-MG with its activation/deactivation status for one BWP after BWP switching.

Proposal 6: RAN4 to consider whether multiple different Pre-MGs can be configured in concurrent gaps operation.

- Summary

In this contribution, we provided our view on Pre-MG.

Proposal 1: In R17 Pre-MG used for PRS measurement can be assumed as always activated.

Proposal 2: Pre-MG used for CSI-RS L3 inter-frequency measurement should be assumed as always activated. It is up to NW to configure Pre-MG or legacy MG for CSI-RS L3 inter-frequency measurement.

Proposal 3: Not support MAC CE based activation/deactivation method for NW based activation/deactivation.

Observation 1: A switch mechanism for the configured pre-MG to legacy MG could be helpful to avoid the ambiguity of UE behavior.

Proposal 4: Wait for RAN2’s reply on RRC design of Pre-MG before deciding additional mechanism to switch the configured pre-MG to legacy MG.

Proposal 5: pre-configured MG activation/deactivation is triggered ONLY by the DCI/Timer based BWP switch in accordance with the WID objective # 1.

Observation 2: For Pre-MG in concurrent gaps operation, network can indicate different Pre-MG with its activation/deactivation status for one BWP after BWP switching.

Proposal 6: RAN4 to consider whether multiple different Pre-MGs can be configured in concurrent gaps operation.

References

[1] R4-2115340, WF on R17 NR MG enhancements – Pre-configured MG patterns and NCSG, Intel Corporation

[2]	R4-2113278, On pre-configured MG pattern(s) for NR_MG_enh, OPPO