3GPP TSG-RAN WG4 Meeting #101-e	R4-2118392

Electronic Meeting, November 01-12, 2021

Agenda item:	8.21.2.1

Source: 	OPPO

Title: 	Discussion on UE Rx/Tx and/or gNB Rx/Tx timing delay mitigation

Document for:	Discussion

Introduction

To improve the positioning accuracy, timing error mitigation mechanism was introduced by RAN1 and RAN4 confirmed its feasibility after initial discussion in the last meeting. The details on TEG grouping method and associated error margins needs further study. This paper will provide our considerations.

Issue 1-2-1 Feasibility of TEGs for timing error mitigation mechanism Agreements: - Confirm that the timing error mitigation mechanism defined by RAN1 is feasible for both UE Rx/Tx and gNB Rx/Tx. - UE/TRP may group the timing errors for UE/TRP Rx/Tx (e.g., based on RF chains and antenna panel) such that timing error difference in the same group is within a certain margin - FFS on RRM requirements for timing error mitigation mechanism, timing error grouping method, criteria and margin. FFS if any specific UE behavior will be defined.

Issue 1-3-1 Impact of the time variation of timing error on the TEGs FFS: - Option 1: (Qualcomm, Huawei) - Time variability of group delays may limit the time scope or useful life of TEGs or, conversely, it may limit the timing error margins that can be achieved if TEGs were to be applied over a prolonged time period. - Option 2: (Nokia, ZTE, Ericsson, Huawei, vivo, Intel, CATT, OPPO) - Study behaviour of residual timing error differences after calibration on static, semi-static of dynamic behaviour and its implications to TEG association. Issue 1-3-2 Whether to define time variant (semi-static or dynamic) TEGs? FFS: - Option 1: No (vivo, CATT, Nokia, OPPO, ZTE) - The timing error can be time variant but TEG is up to UE implementation, i.e., there is no need to consider time variant of TEG. - Option 2: Yes (Qualcomm) - Semi-static or dynamic TEGs configured within the context of a given assistance data, location request, measurement report, or other suitable time period, would be preferable to static TEG configurations. - Option 3: (Huawei) - Timing error is time varying and determination of TEG validity over time can be left to LMF implementation. - Option 4: (Intel, Ericsson) Depending on implementation and RAN1 outcome.

Issue 1-5-1 RRM requirements for verifying the timing error mitigation FFS: - Option 1: (CATT, ZTE, Qualcomm, OPPO) - The testability of this approach on mitigating TRP/UE Tx/Rx timing errors should be considered. - Option 2: (vivo, Ericsson, Qualcomm, Nokia, OPPO) - RAN4 is to further study whether RRM requirements for timing error mitigation are needed. - Option 3: (Huawei) - RAN4 concludes no impacts on core requirements from the TEG framework. RAN4 to discuss whether and how to define new accuracy requirements for the TEG framework in the Performance part.

Discussion

Last meeting confirmed that the timing error mitigation mechanism is feasible for both UE and gNB side. But the grouping method and margin values need further study. One starting point is to group TEG based on the transmission/reception hardware circuit, e.g. RF chains and antenna panels. Then, at least the propagation delay inside UE/gNB can be mitigated. Other timing error components related to PRS/SRS configuration and beam implementation are also identified. Whether TEG can be grouped based on these factors needs discussion, and then the association between TEG and timing error margins can be determined accordingly.

- If TEG is grouped based on hardware circuit only, the associated error margin should be derived in the worst case, e.g. with smallest PRS bandwidth and sampling rate. The performance gain of TEG may be quite limited.

- If TEG is grouped based on hardware circuit and PRS/SRS configuration for example, the associated error margin could be more accurate. However, this will increase the implementation complexity and more TEG IDs are required.

Proposal 1: Hardware circuit, such as RF chains and antenna panels, should be considered when grouping TEG.

Proposal 2: Discuss whether PRS/SRS configuration could be considered when grouping TEG.

We understand that the absolute timing error will vary over time, due to thermal noise, or frequency drift of oscillator. The impacts of noise are completely random and should be considered within TEG margins. The impacts of oscillator drift are linear to time duration and it is easy to determine whether the oscillator timing drift for measurement results at time t1 and t2 is within the TEG margin. It can be left for UE implementation, e.g. by using a new TEG ID.

Proposal 3: Not define time variant TEG and TEG is up to UE implementation.

Another part of feasibility study is whether and how to test the TEG performance. After the TEG reporting and association with error margins are completed, the TEG performance at least for UE side is testable and the UE transmitting accuracy test could be used as the starting point. In our mind, the test frameworks for TEG could be:

- For Tx TEG at UE side:

  - The test system shall monitor the transmitting time of SRS resource #i at antenna or antenna connector and calculate the timing errors, Te, i by comparing to the ideal transmitting time of SRS resource #i at baseband.

  - The test system shall verify that the timing error difference for SRS resources in the same TEG group is within the associated error margin, i.e. ∆Te=Te, i- Te, j≤TmarginTx

- For Rx TEG at UE side,

  - The test system shall calculate the timing error Te, i by comparing arrival time for PRS resource #i reported by UE and the timing of first detected path (in time) at antenna or antenna connector of the same PRS resource #i.

  - The test system shall verify that the timing error difference for PRS resources in the same TEG group is within the associated error margin, i.e. ∆Te=Te, i- Te, j≤TmarginRx

Proposal 4: UE transmitting accuracy test could be used as the starting point for TEG test cases at UE side.

Conclusion

In this contribution, our views on the multiple gaps for MUSIM and the following proposals are given.

Proposal 1: Hardware circuit, such as RF chains and antenna panels, should be considered when grouping TEG.

Proposal 2: Discuss whether PRS/SRS configuration could be considered when grouping TEG.

Proposal 3: Not define time variant TEG and TEG is up to UE implementation.

Proposal 4: UE transmitting accuracy test could be used as the starting point for TEG test cases at UE side.

- References

- R4-2115367, WF on Rel-17 positioning enhancements RRM_2