3GPP TSG-RAN WG4 Meeting #101-e	R4-2118398

Electronic Meeting, 1 Nov. – 12 Nov. 2021

Agenda Item:	8.13.2.3.3

Source:	Ericsson

Title:	Discussion on test set-up for CRS interference handling

Document for:	Discussion

# 1	Introduction

During the last RAN #93-e meeting, several conclusions have been made to guide the next phase study for CRS interference handling.

- The WID in RP-212636 was updated to include the following for CRS interference handling during Phase II work: - Phase II: Define NR PDSCH demodulation requirements for neighbouring cell LTE CRS-IM in scenarios with overlapping spectrum for LTE and NR - Use LLR weighting as baseline reference receiver. - Focus on synchronous network scenario. - 15 kHz SCS for NR is prioritized. - Other aspects will be further discussed in RAN4 and RAN #94e. - Note: The 30 kHz SCS scenario will be discussed after RAN #94e meeting.

Therefore, in this contribution, we are going to share our views on how to design the test set-up for phase II defining requirements.

# 2	Discussion

In this section, we would like to share proposed test set-up for phase II defining requirements.

Sync or async scenario for FDD

Based on the conclusion from the RAN #93-e meeting, we should focus on synchronous network scenario. Thus, we propose to prioritize synchronous test cases in phase II.

Proposal 1: Synchronous scenario for phase II

CBW and SCS for target and interference cells

We propose to prioritize FDD 15kHz for phase II.

Proposal 2: FDD 15kHz or phase II

PDSCH loading level for interfering cell

During the phase I study, companies have investigate several different configurations for PDSCH loading level for interfering cell (one dominant cell):

- PDSCH loading level on interference cell - In time domain, probability of occurrence of data transmission in interference cells: simulate 20%, and it is also encouraged to simulate 0%, 50% and 100%.

We propose to select 20% PDSCH loading level for phase II.

Proposal 3: 20% PDSCH loading level on interfering cell in phase II

MCS for target NR PDSCH

In phase I study, companies have submitted simulation results for MCS4 and MCS13. We propose to continue to consider MCS4 and MCS13 for phase II defining requirements.

Proposal 4: Both MCS4 and MCS13 in phase II

Other parameters can be assumed to be configured the same as agreed in the previous RAN4 meeting.

Detailed simulation assumption can be found in the Annex.

Proposal 5: Reuse other agreed simulation assumption in phase I

# 5	Summary

In this contribution, we share our views on the test set-up for phase II defining requirements. Detailed simulation assumption is provided in the Annex.

Here we summarize our proposals as follows:

Proposal 1: Synchronous scenario for phase II

Proposal 2: FDD 15kHz or phase II

Proposal 3: 20% PDSCH loading level on interfering cell in phase II

Proposal 4: Both MCS4 and MCS13 in phase II

Proposal 5: Reuse other agreed simulation assumption in phase I

# References

- R4-2108662, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom

- R4-2115740, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom

# Annex

Simulation assumption for information

Table 1 NR Cell Parameters

| Parameters | Serving cell FDD |
| --- | --- |
| Network scenario | Sync |
| Number of carriers | single carrier |
| Carrier frequency | 2 GHz |
| PDSCH allocation in frequency domain | Full PRB |
| Physical cell ID | 0 |
| Active DL BWP index | 1 |
| Channel bandwidth | 10MHz |
| TDD configuration | N/A |
| SCS | 15kHz |
| PDCCH allocation | Scenario 1: symbols #2 Scenario 2: symbols #0 and #1 |
| SSB position in burst | SSB index #0 |
| SSB periodicity | 20ms |
| TRS configuration | k0=0 for CSI-RS resource 1,2,3,4 l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 Density 3, 1 port Periodicity 20 slots Offset: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| CSI-RS configuration | Not necessary of PDSCH Demod |
| PRB bundling size for target PDSCH | 2 |
| Antenna configuration | 4x2 4x4 |
| Rank | 1 |
| Overhead for TBS determination | 0 |
| PDSCH DMRS ports | port 0 ~3 |
| Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of additional DMRS | 1 |
| MCS | QPSK, MCS 4 and 16QAM, MCS 13 |
| Precoding granularity | 2 |
| PRB bundling size | 2 |
| PDSCH precoder | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination |
| Antenna Correlation | ULA Low |
| Propagation condition | TDLA30-10 |
| Time Offset relative to serving cell | 0 us |
| Traffic model | Full buffer |
| Number of HARQ Processes | 4 |
| Maximum HARQ transmission | 4 |

Table 2. NR PDSCH Parameters

| PDSCH configuration | Mapping type | Type A |
| --- | --- | --- |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Position of the first DM-RS for downlink | 3 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |

Table 3. LTE Interf. cell parameters

|  | Cell 0(DSS) | LTE Interf. cell 1 | LTE Interf. cell 2 |
| --- | --- | --- | --- |
| carrier centre subcarrier | Same as NR carrier centre subcarrier |  |  |
| BW(MHz) | 10 |  |  |
| Resource allocation | Random full band (50PRB) on/off model, proportional to the average resource utilization in the interfering cells; ON/OFF pattern depends on the Possion distribution |  |  |
| Non-full buffer interference Model | Interfering PDSCH transmissions in interfering cells are randomly & independently active over the full band with an activity in time domain equal on average to the targeted resource utilization |  |  |
| Average resource utilization | 20% |  |  |
| CP Type | Normal |  |  |
| Time offset(us) | 0 | 3 | ​-1 |
| Frequency offset(HZ) | 0 | 300 | -100 |
| PDSCH MCS | Random 16QAM |  |  |
| Precoding | TM4 Random PMI |  |  |
| Rank | Randomly changing rank per allocated subband from subframe to subframe: 80% rank-1, 20% rank-2 |  |  |
| Number of antenna ports | 4 | 4 | 4 |
| v-shift | 0 | 1 | 2 |
| INR(dB) | / | 10.45 | 4.6 |
| Antenna Correlation | ULA Low |  |  |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

TR 36.863 Table 7.2-1

TR 36.863 Table 7.2-1