3GPP TSG-RAN WG4 Meeting #101e	R4-2118596

Electronic Meeting, 1st - 12th Nov, 2021

Source:	ZTE Corporation

Title:	Potential RAN2 specs impacts from overlapping CA

Agenda Item:	9.2.3.3.1

Document for:	Discussion

# Introduction

As one of four potential schemes to make efficient use of an irregular bandwidth, overlapping CA has been discussed mostly on RAN4 aspects. Regarding RAN2 specs impacts, from the configuration perspective, there is no need to introduce any new signaling to support overlapping CA. However, there is still some update possibly required on RAN2 specs. In this contribution, we identify one potential impact on calculating the supported max data rate [1].

# Discussion

In TS 38.306, the UE’s supported max data rate is calculated as shown below. In the calculation, it is assumed that there is no overlapping among aggregated carriers, and each CC has a set of parameters (vLayersj, Qmj, fj,NPRBBWj,μ,OHj). Of all these CC specific parameters, NPRBBW(j),μ should be the maximum RB values for j-th CC defined in RAN4 specs.

### 4.1.2 Supported max data rate for DL/UL For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. is the maximum RB allocation in bandwidth with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where is the UE supported maximum bandwidth in the given band or band combination. is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL

However, this is not the case in overlapping CA. The overlapping PRBs should be accounted only for once.

For example, if an irregular BW are supported via two overlapping CCs, and these two CCs have nominal X RBs and Y RBs for the case when they are not overlapped, and there are O RBs overlapped, as illustrated in Fig. 1.

Fig. 1, Overlapping CA for irregular BW

There are two possible ways to update the calculation of the maximum supported data rate in this example:

- Option 1: By introducing a limitation that both CCs should have the same set of parameters (vLayersj, Qmj, fj,NPRBBWj,μ,OHj), except NPRBBWj,μ. In this case, the maximum PRB number of CC #1 can be X, CC #2 can be (Y-O), or CC#1 for (X-O), and CC#2 for Y. Both will result in the same value, which is equivalent to a single CC in total with NPRBBW,μ=X+Y-O.

- Option 2: If both CCs can have different sets of parameters, then it will make difference if CC#1/CC#2 takes (X, Y-O), or (X-O, Y) respectively. In order to make sure both UE and network have the same understanding on the supported maximum data rate, one bit signaling should be introduced to differentiate these two cases.

Option 1 is obviously simpler. And considering the irregular bandwidth is a contiguous frequency trunk not larger than the maximum channel bandwidth, the limitation required by Option 1 does not hurt deployment flexibility, therefore, we propose to have Option 1.

Proposal 1: RAN4 to consider the same set of parameters (vLayersj, Qmj, fj,OHj) for both overlapping CCs, and update the supported maximum data rate with an equivalent single CC with NPRBBW,μ=X+Y-O.

Proposal 2: Send an LS to RAN2 on the update.

# Conclusion

In this paper, we have the following proposals for potential RAN2 specs impacts from overlapping CA:

Proposal 1: RAN4 to consider the same set of parameters (vLayersj, Qmj, fj,OHj) for both overlapping CCs, and update the supported maximum data rate with an equivalent single CC with NPRBBW,μ=X+Y-O.

Proposal 2: Send an LS to RAN2 on the update.

# Reference

TS 38.306 v16.6.0 UE Radio Access Capabilities