3GPP TSG-RAN WG4 Meeting #101-e	R4-2118685

Electronic Meeting, 1 November – 12 November 2021

Agenda Item:	6.1.4

Source:	Ericsson

Title:	Applicability of FR2 UE demodulation requirements for NR 47GHz band

Document for:	Discussion

# 1	Introduction

RAN4#100-e agreed the existing FR2 UE demodulation performance requirements for 64QAM Rank 2 and 16QAM Rank 1 with Enhanced Receiver Type 1 can be applicable for 47GHz band with extra margin of 0.5dB, but the extra margin for 256QAM rank 1 test case is FFS [1].

256QAM Rank 1 test is applicable to up 48200MHz (including band n262) with extra margin. FFS for the extra margin. - Testability depends on the test system. - Companies are encouraged to study further to decide the required extra margin with the following assumptions: - Phase noise models: - PN model Example 2 in TR 38.803 - Interesting companies can also provide evaluations for other PN models, such as PN model in R4-2010176 - Carrier frequency: 48.2GHz vs. 39GHz - Phase noise compensation at the UE receiver - CPE compensation

This contribution continues the discussion of the applicable extra margin for existing 256QAM UE demodulation requirements.

# 2	Discussion

We have investigated the impact of exiting 256QAM rank 1 test case in Table 1 with two phase noise models according to the way forward. Figure 1 shows again the simulation results in [2], which compares the PDSCH throughput between fc=39GHz and fc=45GHz where we used the phase noise model in TR 38.803 Example 2 and Proposal 1 in [3]. In the simulation results we have also performed two phase noise compensation methods: CPE compensation (CPE comp) and ICI compensation (ICI comp).

Table 1	Evaluation scenarios for UE performance requirements in 47GHz band.

| Test case | Carrier frequency (fc) | CBW/SCS | MCS | Rank | Propagation condition | Antenna configuration |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1 | 30GHz 45GHz | 50MHz / 120kHz | 256QAM CR=0.67 | 1 | TDLD30-75 | 2x2 ULA Low |

| (a) Example 2 in TR38.803 | (b) Proposal 1 in [2] |
| --- | --- |

Figure 1	Performance comparison between fc=39GHz and fc=45GHz with different phase noise model.

It is observed from the simulation results that the performance degradation with 45GHz compared with 39GHz is significant (2.0dB) especially for PN model Example 2. Even with the PN model in [2], the performance degradation is 1.3dB with regard to SNR with 70% of the maximum throughput. Although the applied PN model and compensation methods are up to companies, we think the 256QAM test needs additional margin of 2.0dB, considering the minimum requirements.

Proposal: Set the additional margin of 2.0 dB for the existing FR2 UE performance requirements in TS38.101-4 Table 7.2.2.2.1-3 Test 1-4.

# 3	Summary

Proposal: Set the additional margin of 2.0 dB for the existing FR2 UE performance requirements in TS38.101-4 Table 7.2.2.2.1-3 Test 1-4.

# References

- R4-2115632, “Way forward on UE demodulation on NR 47GHz band”, Ericsson.

- R4-2113459, “Applicability of FR2 UE demodulation requirements for NR 47GHz band”, Ericsson.

- R4-2010176, “On 52.6 to 71 GHz phase noise characteristics and draft LS to RAN1”, Ericsson.