| 3GPP TR 38.854 V0.1.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; NR support for high speed train scenario in frequency range 2 (FR2) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Introduction	7

5	FR2 HST deployment scenario	7

5.1	General	7

5.2	SFN scenario and RRH parameters	9

5.2.1	Unidirectional SFN	9

5.2.2	Bidirectional SFN	11

5.2.3	RRH Parameters	14

5.3	Train roof-mounted high-power CPE parameters	15

5.3.1	Number of panels per CPE	15

5.3.2	Placement of CPE panel(s)	15

5.3.3	Number of CPE devices	15

6	FR2 high speed feasibility evaluation	15

6.1	Evaluation Parameters	15

6.1.1	RRH antenna array parameters for evaluation	15

6.1.2	RRH antenna element parameters for evaluation	16

6.1.3	CPE antenna array parameters for evaluation	16

6.1.4	CPE antenna element parameters for evaluation	17

6.2	Channel model for FR2 HST	17

6.2.1	Pathloss model used for link budget evaluation	17

6.2.2	Channel modelling for performance requirements	19

6.3	FR2 Feasibility Evaluation	22

6.3.1	Idle/inactive mode	22

6.3.2	Connected mode	22

6.3.3	Throughput Performance	22

6.3.4	Mobility Performance	22

6.3.5	Receive time difference	23

6.3.6	Maximum supported Doppler frequency	23

7	Identified RAN4 requirements	24

7.1	CPE RF core requirements 	24

7.2	RRM requirements	24

7.2.1	Idle/inactive mode 	25

7.2.2	Connected mode	25

7.3	Demodulation performance requirements	25

8	Conclusion	31

Annex <A> (informative): <Informative annex for a Technical Specification>	Error! Bookmark not defined.

Annex <X> (informative): Change history	31

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for Rel-17 NR support for high speed train scenario in FR2.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Introduction

5G NR operating in millimeter wave bands (i.e., Frequency Range 2) is recognized as the technology capable of providing ultra-high data-rate transmission, thanks to the availability of enormous amount of bandwidth in FR2 and the advanced 5G NR design for FR2 beamforming-based operation. Inspired by the successful commercial FR2 deployment globally, more potential 5G NR deployment scenarios in FR2 draw attentions from the industry. Among those scenarios identified, high speed train (HST) scenario has the special importance, because of the fast expanding HST systems worldwide deployed and the great demands of high-speed connections from passengers and HST special services. This triggers the new and challenging demand for 5G NR FR2 HST scenario.

In existing study and work items led by 3GPP RAN4 (for either LTE or NR), high speed train scenarios under consideration has the operating bands up to 3.5GHz, however no existing works studied the more challenging millimeter wave frequency range 2, in which Doppler shift and Doppler spread will be further severe (e.g., for 240km/h with 28GHz, the Doppler shift is about 6.22kHz) and more challenging to radio resource management. Specifically, the existing FR2 RRM and demodulation requirements has not yet taken into account the impact of high speed in the above-mentioned scenario, where the channel model and mobility scenario need further study and the demodulation, measurement, mobility and beam management related requirements require to be further specified.

It should be noted that user equipment considered in 5G NR FR2 HST scenario is vehicle-roof mounted customer-premises equipment (CPE), which are expected to communicate with track-side deployed gNBs for the backhaul link and to further provide on-board broadband connections to user terminals and/or for other train-specific demands as access link.

This work will specify NR UE RF requirements, UE RRM requirements and BS/UE performance requirements for high speed train scenario with up to 350km/h in Rel-17.

# 5	FR2 HST deployment scenario

Following section include the agreed FR2 HST deployment specific investigated options and agreements.

## 5.1	General

This section includes the agreed scenario and RRH parameters to be used in the investigated FR2 HST deployments. It captures the agreement and conclusions made during the work on FR2 HST deployment scenario and related aspects. Following figure 1 illustrates the definition of the different used D-values.

Figure 5.1 Illustration of the D-values used on RAN4 work.

General deployment parameters:

RAN4 will at least consider the following general deployment scenarios:

-	Ds and Dmin: Take the 5 scenarios in table 5.1-1 as basic assumption; and

-	Scenario 1 and 4 shall be considered with high priority; and

-	Dmin for [5m, 20, 30 and 50 meters] if found to be necessary; and

-	DRRH_height: 15m as basic assumption, [10,20m] if found to be necessary; and

-	DUE_height: 5m.

| Scenario | Ds (meter) | Dmin (meter) |
| --- | --- | --- |
| 1 | 800 | 10 |
| 2 | 700 | 10 |
| 3 | 500 | 10 |
| 4 | 700 | 150 |
| 5 | 200 | 30 |

Tunnel Deployment Scenario (study tunnel scenario after the prioritized scenarios):

The detailed deployment scenario for tunnel deployment for FR2 HST is still open in RAN4

- RAN4 will further study tunnel deployment scenario for FR2 HST.

Sub-Carrier Spacing (SCS):

It is still open which SCS options to consider. The options are:

-	Option-1: SCS = 120kHz; and

-	Option-2: Consider both SCS = 120kHz and 60kHz.

Concerning the transmissions schemes following schemes were discussed in distinguished:

-	JT: Joint Transmission scheme applied for all channel (SSB, TRS, PDCCH, PDSCH) – Full SFN; and

-	DPS: Dynamic Point Selection based on the Rel-15 beam management (BM) principles; and

-	Multi-DCI based Multi-TRP transmission based on the Rel-15 eMIMO principles.

Among these the following down selection has been agreed:

-	For this WI discussion, FR2 HST transmission schemes which are not compatible with Rel-15/16 NR are precluded; and

-	For this WI discussion, Joint transmission (JT) used for FR2 HST, only full SFN is considered; and

-	For this WI discussion, Multi-DCI based multi-TRP transmission is precluded.

RAN4 primarily consider HST FR2 deployment with

-	One train moving over one railway track in one direction. RAN4 focuses on 1 direction 1 train. If this opposite direction is completely symmetric, the 1 direction study can apply directly; and

-	RRHs are located on one side of the track.

Dedicated network for roof-mounted CPE:

-	RAN4 to assume that in HST FR2 Scenario A, only high-speed CPEs installed on the roof of the train can be present in the network.

RAN4 did comparison between unidirectional and bi-directional RRH deployments for Scenario-A and concluded that from signal strength and beam coverage perspective the bi-directional deployment will not provide significant throughput improvement compared to unidirectional deployment. This conclusion is based on the deployment scenario analysis. RAN4 will only consider unidirectional deployments for Scenario-A. Bi-directional deployment can be considered if the feasibility issue of unidirectional deployment is identified.

RAN4 assume that FR2 HST with CPEs is operated as dedicated network. Hence, assumption in RAN4 is that in HST FR2 Scenario A and B, only high-speed CPEs installed on the roof of the train can be present in the network. There is no need to differentiate roof-mounted CPE from other FR2 UEs in HST FR2 scenario.

RAN4 will not consider curvature when defining the requirements.

## 5.2	SFN scenario and RRH parameters

RAN4 will investigate both unidirectional and bidirectional SFN deployment scenarios for FR2 HST.

The exact understanding and definition of SFN still needs further discussion based on the following interpretations:

-	SFN Interpretation-1: All RRHs under one BBU transmit the same signal.

-	Selected RRH(s) for TX, depending on DPS Tx mode is used or not.

-	SFN Interpretation-2: All RRHs under one BBU in the same cell ID, but for different TCI.

-	Other interpretation is not precluded.

For full SFN JT and unidirectional RRH deployment, only consider following scenario:

-	The setting with only one TCI state transmission.

The value of Ds_offset implicitly limit the RRH beam direction, so there is no need to introduce additional restriction on RRH beam’s possible range of angle on azimuthal plane.

### 5.2.1	Unidirectional SFN

For the unidirectional scenario RAN4 will consider SFN scenario. Hence, one panel per RRH pointed to the same direction for all RRHs (figure 2).

Figure 5.2 SFN scenario with one panel per RRH pointing to the same direction for all RRHs

For the unidirectional scenario the following unidirectional deployment scenario in table 5.2-1 will be prioritized:

Table 5.2-1: Assumed deployment parameters for unidirectional scenario.

| Parameter | Value |
| --- | --- |
| Ds and Dmin | Scenario 2: Ds = 700m and Dmin = 10m Scenario 4: Ds = 700m and Dmin = 150m |
| RRH height | 15m |
| Number of RRH sites per BBU | 4 |
| Number of RRH panels per RRH sites | 1 (i.e. unidirectional) Note 1 |
| Number of analog beams per RRH panel | 1 or 2 Note 2 |
| RRH panel orientation | Option 1: RRH panel boresight pointed to the railway at the direction of Ds (projection of the neighboring RRH on the railway). Option 2: other options not precluded |
| Analog beam orientation | Based on companies’ selection for better performance |
| Note 1: For JT for all channels, 1 beam per RRH panel is considered. Note 2: For DPS, 1 or 2 analog beams per RRH panel can be considered. |  |

Number of Beam for unidirectional RRH deployment, Scenario 2:

-	For scenario 2, unidirectional, RRH parameter: 1 beam per RRH panel; and

-	For scenario 2, unidirectional, UE parameter:

a)	1 beam per panel; and

b)	2 panels assumed to be implemented in the UE side; and

c)	Only the one active panel per UE can be used for Tx and Rx; and FFS whether another panel can be used for beam search

RRH switching point for unidirectional RRH deployment, Scenario 2, figure 5.3:

-	RRH switching point is where the UE switches from the source RRH beam to the target RRH beam based on maximizing SNR among detected beams.

Figure 5.3: RRH switching point for unidirectional RRH deployment.

Number of Beam for unidirectional RRH deployment, Scenario 4:

-	For scenario 4, unidirectional, RRH parameter:

a)	1 beam per RRH panel; or

b)	2 beams per RRH panel; or

c)	3 beams per RRH panel; or

d)	4 beams per RRH panel.

Note that uneven separation between beams can be considered.

RAN4 agreed that at least 2 beams per RRH panel is considered. Other options are not precluded, and it is FFS whether there are benefits of implementing more beams per RRH panel.

-	For scenario 4, unidirectional, UE parameter:

a)	1 beam per UE panel; or

b)	2 beams per UE panel; or

c) 	7 beams per UE panel.

RAN4 assumes 2 panels to be implemented in the UE side. Only the one active panel per UE can be used for Tx and Rx; and FFS whether another panel can be used for beam search.

RAN4 decided that at least option a) of having 1 beam per panel is considered. Other options are not precluded, and it is FFS whether there are benefits of implementing more beams per UE panel.

### 5.2.2	Bidirectional SFN

For the bidirectional scenario RAN4 will consider SFN scenario where there is one panel per RRH with signals to opposite directions along the track (figure 3).

Additionally, also SFN scenario where there are two panels per RRH (figure 4).

Figure 1 SFN scenario with two panels per RRH

For the bi-directional scenario the following bi-directional deployment scenario in table 5.2-2 will be studied:

Table 5.2-2: Assumed deployment parameters for bi-directional scenario.

| Parameter | Value |
| --- | --- |
| Ds and Dmin | Scenario 2: Ds = 700m and Dmin = 10m Scenario 4: Ds = 700m and Dmin = 150m |
| RRH height | 15m |
| Number of RRH sites per BBU | 4 |
| Number of RRH panels per RRH sites | 2 (i.e. bi-directional) |
| Number of analog beams per RRH panel | 1, 2 or 4 |
| RRH panel orientation | Option 1: RRH panel boresight pointed to the railway in the middle point between 2 RRHs. Option 2: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighboring RRH on the railway). Other options not precluded. |
| Analog beam orientation | Based on companies’ selection for better performance |

Candidate schemes for bi-directional deployment for further analysis, scenario 2:

Figure 5.4: Connecting to 2nd-Nearest RRH

Figure 5.5: Scheme-2: Connecting to Nearest RRH except Coverage Hole

For Scenario 2 bi-directional RRH deployment:

-	[Scheme 1 under bi-directional scenario is feasible without coverage hole issue, and no propagation delay jump between switching points];

-	Scheme-2 can be used as starting points for further analysis.

Number of beams for bi-directional RRH deployment, Scenario 2:

-	For scenario 2, bi-directional, RRH parameter: 1 beam per RRH panel, two panels in opposite directions; and

-	For scenario 2, bi-directional, UE parameter: 1 beam per UE panel (i.e., 2 beams per UE).

Candidate schemes for bi-directional deployment for further analysis, for scenario 4:

Figure 5.6: Connecting to 2nd-Nearest RRH

Figure 5.7: Scheme-2: Connecting to Nearest RRH except Coverage Hole

Figure 5.8: Scheme-3: Connecting to Nearest RRH except the area under the RRH

For Scenario 4 bi-directional RRH deployment: the schemes above can be used as starting points for further analysis.

Number of beams for bi-directional RRH deployment, Scenario 4:

-	For scenario B, bi-directional, RRH parameter:

a)	1 beam per RRH panel; or

b)	2 beams per RRH panel; or

c)	3 beams per RRH panel; or

d)	4 beams per RRH panel.

Note that uneven separation between beams can be considered.

RAN4 agreed that at least 2 beams per RRH panel is considered. Other options are not precluded, and it is FFS whether there are benefits of implementing more beams per RRH panel.

-	For scenario 4, bi-directional, UE parameter:

a)	1 beam per UE panel; or

b)	2 beams per UE panel; or

c) 	7 beams per UE panel.

RAN4 assumes 2 panels to be implemented in the UE side. Only the one active panel per UE can be used for Tx and Rx; and FFS whether another panel can be used for beam search.

RAN4 decided that at least option a) of having 1 beam per panel is considered. Other options are not precluded, and it is FFS whether there are benefits of implementing more beams per UE panel.

For bi-directional RRH deployment, DPS transmission scheme shall be considered.

### 5.2.3	RRH Parameters

The detailed RRH parameters are still under discussion in RAN4 and following options have been agreed for potential down selection:

Number of RRH sites per BBU:

-	[1 to 4] RRHs sites per BBU; and

-	Other values are not precluded.

Number of Analog Beams per panel in RRH:

-	[1,2,4] analogue beam(s) per panel in RRH; and

-	Other values are not precluded.

SSB index to Beam Mapping. The impact of following options for SSB index to Beam mapping is FFS:

1)	Option 1:

a)	All RRHs (connected to one BBU with fiber) share the same cell ID; and

b)	All RRHs under the same cell use the same set of SSB indexes, e.g., all RRHs use SSB-0 to SSB-3.

2)	Option 2:

a)	All RRHs (connected to one BBU with fiber) share the same cell ID; and

b)	RRHs under the same cell use the different sets of SSB indexes, e.g., RRH-1 uses SSB-0 to SSB-3, RRH-2 uses SSB-4 to SSB-7.

When DPS is used as transmission scheme, SSB index to Beam Mapping used in the discussion for this WI is as follows:

RRHs under the same cell use different sets of SSB indexes, e.g., RRH-1 uses SSB-0 to SSB-3, RRH-2 uses SSB-4 to SSB-7, etc.

RAN4 discussed the aspect of potential handover problem due to sudden RX signal increase of the target cell. RAN4 thinks that this can be alleviated by DPS transmission scheme with carefully allocated SSB-index among neighboring cells to avoid inter-cell interference.

Concerning RRH antenna array orientation the impact of following options for RRH antenna array orientation is FFS:

-	Option 1: RRH panel boresight pointed to the railway in the middle point between 2 RRHs; or

-	Option 2: RRH panel boresight pointed to the railway at the distance of Ds (projection of the neighbouring RRH on the railway); or

-	Other option is not precluded.

Related to RRH/UE boresight direction of Antenna Panel and beam direction, RAN4 may not need to specify RRH/UE boresight direction of antenna panel and beam direction for deployment scenario study, but left for companies’ choice:

-	RRH/UE boresight direction of antenna panel and beam direction information can be provided by individual company to accompany their deployment scenario analysis result, which can be captured in TR.

## 5.3	Train roof-mounted high-power CPE parameters

Note: Single panel, i.e. only one active antenna panel at a time, as baseline antenna assumption

### 5.3.1	Number of panels per CPE

The number of panels per CPE was open and for discussion. Concerning the number of panels per CPE following will be considered:

-	2 panels per CPE each for both for TX and RX. Each panel points in opposite directions.

For the bi-directional Operation for Two Panels in the CPE following is considered:

-	Follow the Rel-15/16 principle of “only one active Rx/Tx panel at a time”.

### 5.3.2	Placement of CPE panel(s)

UE antenna panel(s) for forward and backward directions:

-	RAN4 to consider CPE to be equipped with two panels pointed forward and backward along the track.

UE boresight direction of Antenna Panel and beam direction, RAN4 may not need to specify RRH/UE boresight direction of antenna panel and beam direction for deployment scenario study, but left for companies’ choice:

-	RRH/UE boresight direction of antenna panel and beam direction information can be provided by individual company to accompany their deployment scenario analysis result, which can be captured in TR.

### 5.3.3	Number of CPE devices

Number of CPE devices per train/carriage:

-	RAN4 requirement can be defined based on the baseline of 1 CPE device per train.

# 6	FR2 high speed feasibility evaluation

This section will include the evaluation parameters and channel model used in the FR2 HST and used for evaluating the feasibility FR2 HST.

## 6.1	Evaluation Parameters

### 6.1.1	RRH antenna array parameters for evaluation

RAN4 will perform FR2 HST feasibility study based on following RRH antenna array parameters in table 6.1.1.1 for evaluation:

Table 6.1.1.1

| Parameter | Urban Macro 30 GHz |
| --- | --- |
| Am | 30 |
| SLAy | 30 |
| j3dB | 90 |
| q3dB | 90 |
| Ge,max | 5.5 |
| LE | 1.8 |
| P | 2 |
| dy | 0.5l |
| dh | 0.5l |

Antenna array configuration options were considered:

-	Option-1: [Mg, Ng, M, N, P] = [1, 1, 4, 8, 2]; and

-	Option-2: [Mg, Ng, M, N, P] = [1, 1, 8, 8, 2]; and

-	Option-3: [Mg, Ng, M, N, P] = [1, 1, 8, 16, 2].

RAN4 also agreed that other options are not precluded. RRH Antenna Element Assumption for RRH side following option is assumed:

-	Option-2: [Mg, Ng, M, N, P] = [1, 1, 8, 8, 2].

RF session can trigger relevant discussion on RF requirements taking above agreements into account.

### 6.1.2	RRH antenna element parameters for evaluation

RAN4 agreed to use the following RAN1 assumptions in table 6.1.2.1 for BS evaluation as baseline, while other assumptions are not precluded:

Table 6.1.2.1: BS evaluation parameters.

| Radiation power pattern of a single antenna element for TRP | Vertical cut of the radiation power pattern (dB) |  |
| --- | --- | --- |
|  | Horizontal cut of the radiation power pattern (dB) |  |
|  | 3D radiation power pattern (dB) |  |
|  | Maximum directional gain of an antenna element GE,max | [8] dBi |

### CPE antenna array parameters for evaluation

RAN4 will perform the FR2 HST feasibility study based on following CPE antenna array parameters for evaluation:

-	RAN1 assumption: 2 ports: [Mg, Ng, M, N, P] = [1, 1, 2, 4, 2]; and

-	PC4 assumption: 2 ports: [Mg, Ng, M, N, P] = [1, 1, 4, 4, 2].

RAN4 also agreed that other options are not precluded.

UE Antenna Element Assumption: on UE side:

-	Option 1: N=4, M=4 with 2 polarizations as starting point, and other options not precluded pending on further discussion.

RF session can trigger relevant discussion on RF requirements taking above agreements into account.

### 6.1.4	CPE antenna element parameters for evaluation

RAN4 use the following RAN1 assumption for CPE antenna element parameters evaluation in table 6.1.4.1 as baseline:

Table 6.1.4.1: CPE antenna element parameters.

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ’’ dim (dB) |  |
| Antenna element radiation pattern in ϕ’’ dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

## 6.2	Channel model for FR2 HST

This section collects the channel model information used for FR2 HST feasibility evaluation and provides the analysis on channel modelling for performance requirements.

### Pathloss model used for link budget evaluation

To have the link budget analysis for the proposed FR2 HST deployment scenarios, the accurate large-scale pathloss model is one of the prerequisites. The following large scale pathloss models are proposed to be considered as candidate options:

-	Option-1: TR38.901 RMa LoS (baseline option); or

-	Option-2: free space model; or

-	Option-3: TR38.901 UMa LoS.

TS38.901 RMa LoS pathloss model will be used for link budget evaluation at least for scenario 2 in table 5.1-1.

RAN4 to choose TS38.901 RMa LoS pathloss model also for the evaluation of Scenario 4 in table 5.1-1.

For the purpose of demonstrating and validating which large scale channel modeling is suitable for FR2 HST, the analysis has been provided based on the practical field measurement. Specifically, based upon the conditions provided in the Table 6.2.1-1, the practical field testing on a trait along a typical railway has been conducted to obtain measurement data at the frequency of 28GHz, as illustrated in the Figure 6.2.1-1.

Table 6.2.1-1 Parameters for practical field measurement for typical high speed train scenario

| Parameter name | Configuration value |
| --- | --- |
| Minimum TX-RX distance | 60 m |
| Maximum TX-RX distance | 550 m |
| Distance granularity | 1 m |
| Center frequency | 28 GHz |
| TX antenna height | 5 m |
| RX antenna height | 3 m |
| Parameter hE in 3GPP | 1 m |

Figure 6.2.1-1 Illustration of practical field measurement conducted for typical high speed train scenario

By having the analysis based on the measurement data obtained from the measurement campaign as above described, the comparison among measurement results and pathloss models (i.e., the three options of RMa LOS, UMa LOS and free space model) is demonstrated in Figure 6.2.1-2 and accompanying Table 6.2.1-2 in which the numerical results are contained.

Figure 6.2.1-2 Comparison of measurement data and pathloss models for FR2 HST

By leveraging the numerical results in terms of the root mean square error (RMSE), mean error and standard deviation (Std), it has been demonstrated that for the evaluated range there is no significant difference from three different pathloss LoS models, and the field measurement also validate that LoS model can reflect the practical FR2 HST channel condition compared with NLoS models. By further investigating three LoS models’ similarity from measurement data, it has been demonstrated that RMa LoS model can achieve the lowest value of RMSE and best mean error with reasonable standard deviation.

Table 6.2.1-2 Numerical comparison of measurement data and pathloss models for FR2 HST

|  | RMSE | Mean Error | Std |
| --- | --- | --- | --- |
| Free space model | 4.5212 | -0.74819 | 4.4634 |
| RMa LoS model | 4.4716 | 0.13552 | 4.4741 |
| UMa LoS model | 4.4974 | -0.3428 | 4.4889 |
| RMa NLoS model | 35.1499 | 34.4667 | 6.9036 |
| UMa NLoS model | 26.5 | 25.692 | 6.5006 |

Based upon the above analysis on the measurement data from the typical railway environment for 28GHz, it has been demonstrated that TS38.901 RMa LoS model is an accurate large-scale pathloss model and it is agreed that TS38.901 RMa LoS pathloss model is adopted to be used for link budget evaluation at least for Scenario-A.

Editor Note: FFS pathloss model for tunnel deployment scenario and Scenario-B.

### 6.2.2	Channel modelling for performance requirements

Based on WID as follows, RAN4 is tasked to further study the channel model for FR2 HST, where the key question needs to be answer:

-	Whether or not single-tap per RRH channel model is assumed in UL direction; and

-	Whether single- or multi-tap model is assumed in DL direction.

Compared with the FR1 counterpart, the major difference of having analog beamforming in FR2 should be considered in determining channel model for performance requirement. Depending on whether or not joint transmission is allowed for FR2 HST, it could be possible to have multiple taps from neighboring RRHs, while if only DPS is allowed single tap model should be employed.

Furthermore, whether or not the single tap is accurate enough for a single TX-RX link in FR2 HST scenario has been studied. Specifically, a measurement-data-calibrated FR2 HST ray-tracing model is used to simulate various paths of LoS and reflected path for a practical railway scenario. As required by WID, the UE is mounted on top of the driver’s cabin of the train in the simulation. And the traveling length is 2000 m, with a sampling distance of 20 m, thus making 200 snapshots (UE locations) be simulated.

UE moving direction

The entire length is 2000 m

RRH1

RRH2

RRH3

RRH4

UE

(Initial position)

Figure 6.2.2-1 Illustrative of the setup for FR2 HST ray-tracing simulation

For each given UE location, the received signals from 4 RRHs are simulated by using the ray-tracing model. Numerically, it has been demonstrated that all simulated snap shots have the ratio of received non-first-tap power over total received power smaller than 0.01, or in other words, in all snap shots, the first tap can contain more than 99% of the energy, which validate the single-tap assumption from a single TX-RX link in FR2 HST.

Figure 6.2.2-2 CDF of the ratio of non-first-tap power over total power

Based on measurement-data-calibrated ray-tracing modeling at 28GHz for typical railway environment, it has been validated that the single-tap is accurate enough to represent a single TX-RX link for FR2 HST. Therefore, it is agreed that for channel modelling for performance requirement, the single-tap can be assumed for a single TX-RX link at least for Scenario-A.

Editor Note: FFS multi-tap models are needed for SFN and other scenarios and FFS channel model for Scenario-B.

For channel modelling for performance requirement evaluation:

-	Single-tap can be assumed for a single TX-RX link at least for scenario 2 in table 5.1-1.

RAN4 agreed that for both uplink and downlink the cosine of angle θ(t) used in Doppler shift in channel model is applied. This applies for:

-	A particular uni-directional deployment scenario; and

-	A particular bi-directional deployment scenario.

The single tap propagation model can be assumed for each single Tx-Rx link for both scenario 2 and scenario 4 in table 5.1-1

RAN4 agreed that the uplink and downlink channel model in uni-directional deployments one channel model (either toward to serving beam or away from serving beam) is applied for demodulation requirement even if UE can travel in two directions in practice.

RAN4 will use the following HST-DPS channel model as a starting point for FR2 HST uni-directional RRH deployment:

-	UE is moving towards the serving beam

-	The cosine of angle θ(t) used in Doppler shift fst=fd cosθt is provided as

-	cosθt=Ds_offset +Ds-vtDmin2+Ds_offset+Ds-vt2,  0<t≤Dsv 																(1)

-	cosθt=cosθt modDsv,  t>Ds/v  																	(2)

Where:

- 0 ≤Ds_offset<Ds																								(3)

The Ds_offset value for introducing performance requirements is 10m and 100m for Scenarios A and B, respectively. The Ds_offset value has no restriction concerning deployments and the value is only used for developing the demodulation requirements. The Ds_offset is derived from the worst case based on analysis of deployment scenarios. Note, demodulation simulation assumptions should cover at least one Doppler shift jump region.

RAN4 decided regarding the HST FR2 channel model in bi-directional deployment to use option 2 with DPS based channel model as starting point and based on this RAN4 agreed to use the following cosine values for the different schemes:

Option 2(a): To match Bi-directional deployment Scheme-1: UE connect to 2nd-nearest RRH):

-	cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤(0.5*Ds)/v																(4)

-	cosθt=-vtDmin2+vt2,  (0.5*Ds)/v<t≤Ds/v															(5)

-	cosθt modDsv,    t>Ds/v																					(6)

Option 2(b): based on Scheme-2 for Bidirectional RRH Deployment:

-	cosθt=-（Ds+vt）Dmin2+Ds+vt2, 0≤t<Ds_offsetv,																	(7)

-	cosθt=Ds-vtDmin2+Ds-vt2, 0.5*Dsv≤t<(Ds-Ds_offset)v												(8)

-	cosθt=-vtDmin2+-vt2, (Ds-Ds_offset)v≤t<Dsv														(9)

-	cosθt= cosθt mod(Dsv), t>Dsv																		(10)

- Dsoffset=100m (The Ds_offset is derived from the worst case based on analysis of deployment scenarios)

Option 2(c):  based on Scheme-3 for Bidirectional RRH Deployment:

-	cosθt=Ds-vtDmin2+Ds-vt2,  0<t≤Aoffset/v																	(11)

-	cosθt=-vtDmin2+vt2,   Aoffset/v<t≤(0.5*Ds)/v 														(12)

-	cosθt=Ds-vtDmin2+Ds-vt2,  (0.5*Ds)/v <t≤Boffset/v														(13)

-	cosθt=-vtDmin2+vt2,  Boffset/v<t≤(Ds)/v																(14)

-	cosθt modDsv,   t>Ds/v																						(15)

-	Aoffset<(0.5*Ds)<Boffset <Ds																				(16)

RAN4 agrees to use DPS channel model for both uni-directional and bi-directional for performance requirements.

## 6.3	FR2 Feasibility Evaluation

RAN4 perform feasibility study on FR2 HST scenario, by at least considering following aspects:

1)	The feasibility of a deployment based the beam dwelling time and measurement period framework:

a)	How many beams/SSBs per RRH can be deployed (given other deployment parameters such as Dmin, Ds, speed etc) while maintain mobility performance with FR2 BM mechanism?; and

b)	How much beam refinement is needed to achieve coverage and mobility?

-	How much beam overlapping area is needed (given other deployment parameters such as Dmin, Ds, speed etc.) to ensure beam refinement procedure can be executed successfully?

2)	Study throughput performance and mobility performance:

a)	More number of analogue beams and sharper beam may provide better link budget performance but more challenging on mobility performance.

3)	Receive timing difference.

4)	Maximum supported Doppler shift for both UL and DL and maximum supported UE speed.

Other feasibility study is not precluded.

For evaluating the maximum supported speed following numerology will be considered:

-	For FR2 HST evaluations and possible performance requirements definition, RAN4 only consider 120kHz SCS as baseline assumption.

### 6.3.1	Idle/inactive mode

### 6.3.2	Connected mode

### 6.3.3	Throughput Performance

### 6.3.4	Mobility Performance

RAN4 discussed the RX beam number for RRM requirements definition and agreed to define two set of requirements for Scenario A and Scenario B in terms of number of RX beams per UE:

-	Scenario A: [2] RX beams for all scenarios.

-	Scenario B: [6] RX beams for all scenarios.

Note: if there is insignificant difference between Scenario A and B requirements, then further discussion on unified requirements can take place

For RRC CONNECTED mode requirements for DRX (based on GtW):

-	Define requirements for the short DRX configurations (≤ [80] ms).

Handover:

-	Existing FR2 requirement should be applicable to the HST FR2 deployments when the target cell is known.

Requirements on inter-frequency measurements:

-	Do not define inter-frequency measurements requirements for FR2 HST.

Requirements on inter-RAT measurements:

-	Do not define inter-RAT measurements requirements for FR2 HST.

Measurement procedures:

1)	Cell identification - PSS/SSS detection:

-	Option1: The Cell identification - PSS/SSS detection requirements shall be enhanced.

2)	Cell identification - Intra-frequency measurements:

-	Option 1: The intra-frequency measurement requirement shall be enhanced.

3)	Restriction on SMTC periodicity:

-	Restriction on SMTC periodicity configuration are preferred in FR2 HST.

4)	CSI-RS based L3 measurements:

-	The analysis of the requirements to be de-prioritized.

L1 measurements:

-	The L1 measurements shall be enhanced.

### 6.3.5	Receive time difference

Not applicable to FR2 HST

### 6.3.6	Maximum supported Doppler frequency

Carrier frequency for Doppler frequency calculation

-	30GHz.

### 6.3.7	Maximum supported Speed

Companies’ observation on Maximum Speed feasibility:

-	It is feasible to support maximum speed with 350km/h for downlink with TRS (4 symbol interval) for frequency offset tracking under unidirectional RRH deployment with 120KHz SCS.

-	It is feasible to support maximum speed with 350km/h for downlink with TRS (4 symbol interval) +SSB for frequency offset tracking under unidirectional and bi-directional RRH deployment with 120KHz SCS.

-	It is feasible to support maximum speed with 350km/h for downlink with TRS (4 symbol interval) + PTRS (L=1) for frequency offset tracking under bi-directional RRH deployment with 120KHz SCS.

-	It is feasible to support maximum speed with 350km/h for downlink with PTRS or DMRS(1+1+1) + PTRS (L=1,K=2) configuration used for frequency offset tracking under single tap propagation conditions with 120KHz SCS.

Configure PTRS during the PDSCH demodulation test.

RS as baseline for frequency offset tracking to support 350km/h

-	multiple options under discussion.

DMRS configuration for PDSCH demodulation requirement

-	Option 1: 1 DMRS; and

-	Option 2: 1+1+1 DMRS.

### 6.3.8	Beam dwelling time

#### 6.3.8.1	Simulation results

The system simulation assumptions for beam dwelling time are shown in table 6.3.8.1-1. The simulation results of beam dwelling time are obtained from system-level simulations which were carried out to evaluate legacy RRM requirements under high-speed train scenarios.

Table 6.3.8.1-1: Simulation assumptions for beam dwelling time

| Parameter | Value |
| --- | --- |
| Number of sites (separate gNBs) | 12 |
| Inter-site distance (ISD, Ds) | 700 m |
| RRH distance to track (Dmin) | 10 m (Scenario A), 150 m (Scenario B) |
| RRH height (D_RRH_Height) | 15 m |
| CPE height (D_CPE_Height) | 5 m |
| Carrier frequency | 28 GHz |
| Bandwidth | 50 MHz |
| Subcarrier spacing | 120 kHz |
| Propagation and channel model | TR 38.901 RMa with LOS only |
| RRH TX output power | 31 dBm |
| RRH antenna panel | [Mg, Ng, M, N, P] = [1, 1, 8, 8, 2] Panel is pointing towards the track at the x-axis where the next site is situated (ISD away) |
| RRH antenna panel direction in relation to train in uni-directional deployments | Opposite direction (train moves east, RRHs pointing west) Same direction (train moves east, RRHs pointing east) |
| SSB beams per RRH | Uni-directional: 1 beam: Pointing into the boresight of the RRH antenna panel 2 beams: One beam is pointing into the boresight and the other beam is pointing 20 degrees towards the track from boresight 4 beams: One beam is pointing into the boresight and the other beams are pointing 20, 40, 60 degrees towards the track from boresight Bi-directional: 1 beam: Pointing into the boresight of the RRH antenna panel 2 beams: One beam is pointing into the boresight and the other beam is pointing towards the track at Ds/2 4 beams: One beam is pointing into the boresight and the other beams are pointing towards the track at Ds/2, Ds/4, Ds/8 |
| CPE (Train) speed | 350 km/h |
| CPE antenna panel | [Mg, Ng, M, N, P] = [1, 1 or 2, 4, 4, 2] In uni-directional case where RRHs point east CPE has one antenna panel pointing west In bi-directional case CPE has two antenna panels pointing to 180 degrees opposite directions (west-east) MPUE assumption: only one panel can be used at a time for measurements |
| Number of beams per CPE panel | 1 beam (even though it is 1, scaling factor 8 is assumed for RRC measurements, L1 measurements and cell detection delays in simulations) |
| Traffic | DL Full Buffer |
| Inter-cell interference | Only one train with one CPE is simulated meaning there is no inter-cell interference |
| DRX | DRX disabled (DRX 0), 40, 80, 160, 256, 320 ms cycles |
| SMTC period | 20 ms |
| Handover assumptions | Event A3 with SS-RSRP Offset: 3 dB Time-to-trigger: 80 ms |
| RRC measurement period L1 RSRP measurement period | Note: N=8 assumed in scaling DRX 0: 480 ms DRX 40: 1440 ms DRX 80: 2880 ms DRX 160: 5760 ms DRX 256: 9216 ms DRX 320: 11520 ms |
| Cell detection delay (TPSS/SSS_sync_intra) | Note: N = 8 is assumed in scaling DRX 0: 600 ms DRX 40: 1440 ms DRX 80: 2880 ms DRX 160: 5760 ms DRX 256: 9216 ms DRX 320: 11520 ms |
| RLM assumptions | Note: N=8 assumed in scaling TEvaluate_out_CSI-RS: 600, 3600, 7200, 14400, 23040, 28800 ms (DRX 0, 40, 80, 160, 256, 320) TEvaluate_in_CSI-RS: 300, 1800, 3600, 7200, 11520, 14400 ms (DRX 0, 40, 80, 160, 256, 320) N310: 2 samples N311: 2 samples Qout threshold SINR: -8 dB Qin threshold SINR: -6 dB |
| BFD assumptions | Note: N=8 assumed in scaling TEvaluate_BFD_CSI-RS: 300, 1800, 3600, 7200, 11520, 14400 ms (DRX 0, 40, 80, 160, 256, 320) |
| Simulation length | 100 seconds (20 drops of 100 seconds simulated, and statistics samples are gathered from all drops) |

The simulation results of beam dwelling time are shown in figures 6.3.8.1-1-3 for unidirectional scenarios with a different number beams transmitted by RRH. The simulation results of beam dwelling time are shown in figures 6.3.8.1-4-6 for bidirectional scenarios with different number of beams transmitted by RRH.

It is worth noting that the simulation results are the average beam dwelling time. In the simulation, beam dwelling time is influenced by beam coverage and beam switching rate (including hysteresis). Beam coverage is discussed in detail in clause 6.3.8.2, where ideal beam dwelling time is investigated.

In the case of multi-beam operation cases shown in figures 6.3.8.1-2, 6.3.8.1-3, 6.3.8.1-5 and 6.3.8.1-6, it can be observed that the dwelling time of Beam 0 is the shortest because the coverage of Beam 0 is the smallest.

Figure 6.3.8.1-1: Average beam dwelling time with 1 beam per RRH for unidirectional scenarios

Figure 6.3.8.1-2: Average beam dwelling time with 2 beams per RRH for unidirectional scenarios

Figure 6.3.8.1-3: Average beam dwelling time with 4 beams per RRH for unidirectional scenarios

Figure 6.3.8.1-4: Average beam dwelling time with 1 beam per RRH for bidirectional scenarios

Figure 6.3.8.1-5: Average beam dwelling time with 2 beams per RRH for bidirectional scenarios

Figure 6.3.8.1-6: Average beam dwelling time with 2 beams per RRH for bidirectional scenarios

#### 6.3.8.2	Beam coverage analysis

In HST deployment scenarios, the footprint of an RRH beam can be represented by an ellipse as shown in figure 6.3.8.2-1. It is assumed that UE aboard an HST is moving with constant velocity v and its trajectory is a straight line. An RRH is located at the point G(0,0,hRRH) in which its antenna array boresight for the kth beam is oriented towards a point Pk(Dmin,yk,h) which is along the trajectory of UE.

Figure 6.3.8.2-1: RRH beam footprint

The ellipse centred at Cxc,yc,zc can be expressed as

x-xccosϕk+y-ycsinϕka22+y-yccosϕk-x-xcsinϕk2b2=1,      z=h, 0<h<hRRH

where

a=GQ2tanθk'+θHPBW2-tanθk'-θHPBW2,     θk'-θHPBW2<θk'<θk'+θHPBW2<90°

b=GQsecθc'GCtanϕHPBW2

xc=QC1+yku2,    yc=yku xc,    zc=h

From trigonometry, QC can be derived from the elevation HPBW θHPBW, elevation pointing angle θk', semimajor axis a and the magnitude of GQ as follows

QC=GQtanθk'-θHPBW2+a

In the equation of the semi-minor axis, θc' is the angle between the vectors GQ and GC, which can be expressed in terms of the semi-major axis a, elevation HPBW θHPBW and elevation pointing angle θk'.

θc'=tan-1tanθk'-θHPBW2+aGQ

Referring to figure 6.3.8.2-1, the segment length of the UE’s trajectory covered by the footprint of beam k is the line segment joining the points P1Dmin,y1,h and P2Dmin,y2,h, which are the intersection points of the ellipse with the line x=Dmin, z=h. In determining these two points, rewriting the ellipse equation as a quadratic equation for y gives

αy2+βy+γ=0

where the coefficients are

α=a2cos2ϕk+b2sin2ϕk

β=2yc-a2cos2ϕk-b2sin2ϕk+Dmin-xcb2cosϕksinϕk-a2cosϕksinϕk

γ=yc2a2cos2ϕk+b2sin2ϕk+2ycDmin-xca2cosϕksinϕk-b2cosϕksinϕk+

Dmin-xc2a2sin2ϕk+Dmin-xc2b2cos2ϕk -a2b2

The two solutions to the quadratic equation are

y1=-β-β2-4αγ2α,	y2=-β+β2-4αγ2α

The segment length lk (in metres) of the UE’s trajectory covered by beam k is given by

lk=y2-y1

Thus, the dwelling time tk (in seconds) for beam k is defined by

tk=lkv

In order to determine the width of the footprint provided by beam k, it is the points of intersection of the ellipse with a straight line passing through PkDmin,yk,h perpendicular to the line x=Dmin, z=h. The width wk (in metres) of the footprint defined by beam k is expressed as

wk=x2-x1

where

x1=-β'-β'2-4α'γ'2α',	       x2=-β'+β'2-4α'γ'2α'

α'=a2sin2ϕk+b2cos2ϕk

β'=2[xc-a2sin2ϕk-b2cos2ϕk+(yk-yc)(b2cosϕksinϕk-a2cosϕksinϕk)]

γ'=xc2a2sin2ϕk+b2cos2ϕk+2xcyk-yca2cosϕksinϕk-b2cosϕksinϕk+

yk-yc2a2cos2ϕk+yk-yc2b2sin2ϕk

Table 6.3.8.2-1 shows the coverage length, width and dwelling time for 3 beams where the boresight is pointing to different positions along the UE’s trajectory for Scenario A.

Table 6.3.8.2-1: Beam coverage length, width and dwelling time

| Dmin=10 m, θHPBW=12.6°, ϕHPBW=12.6° |  |  |  |  |
| --- | --- | --- | --- | --- |
| Beam k | PkDmin,yk,h | lk | wk | tk |
| 0 | PkDmin,5Dmin,h | 57 m | 14 m | 0.6 s |
| 1 | PkDmin,7.5Dmin,h | 244 m | 31 m | 2.5 s |
| 2 | PkDmin,8.5Dmin,h | 813 m | 60 m | 8.4 s |

# 7	Identified RAN4 requirements

## 7.1	CPE RF core requirements

RAN4 will further study and discuss the CPE core requirements.

Concerning the CPE RF requirements RAN will further discuss and select among following options for the baseline power class:

-	Baseline power class for FR2 HST:

1)	Option-1: PC4 as baseline, and FFS PC4 requirement is applicable to FR2 HST scenario; or

2)	Option-2: To define new PC for FR2 HST; or

3) 	Option-2a: To define new PC for FR2 HST, with PC5 requirement as baseline.

Additionally, RAN4 will further discuss how the specify the UE RF requirements. Options listed are:

-	Option-1: Provide an applicability rule of FR2 PC4 for the train-roof-mounted UE for FR2 HST scenario, i.e., the applicable FR2 PC4 requirement set for FR2 HST scenario; or

-	Option-2: Revisit the full set of UE RF requirements for FR2 PC4 UE; or

-	Option-3: New RF requirement is defined for FR2 HST UE which is different from PC4, specifically, the min peak EIRP for FR2 HST UE follows the agreement for PC5(new FR2 FWA UE).

The RF requirement applicability rule (based on NW flag signalling) is not introduced. FR2 HST UE shall satisfy the relevant RF requirement, regardless of this NW flag signalling. Additionally, RAN4 define unified RF requirement for both uni- and bi-directional RRH deployment.

### 7.1.1	Minimum Peak EIRP

Minimum peak EIRP requirement for FR2 HST UE it is agreed to adopt 30.x dBm (similar to PC5) as baseline. This baseline can be further discussed if any technical issues are identified with the current agreement.

A multi-band relaxation factor of 0.7 dB which is similar to PC5 is introduced as shown in table 7.1.1-1.

Table 7.1.1-1: Multi-band relaxation.

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.7 | 0.7 |
| n258 | 0.7 | 0.7 |
| n261 | 0.7 | 0.7 |

### 7.1.2	Beam Correspondence

For the roof mounted HST UE the Rel-15 beam correspondence apply, and the FR2 HST UE (roof-mounted UE type) shall mandatorily support beamCorrespondenceWithoutUL-BeamSweeping.

No need to introduce Beam Correspondence tolerance requirement because all FR2 HST UE need mandatory support of Rel-15 BC without uplink beam sweeping. If Rel-15 Beam Correspondence feature beamCorrespondenceWithoutUL-BeamSweeping is mandatorily supported by FR2 HST UE, then by following PC3 BC requirement:

-	For Rel-15 Beam Correspondence capable UE, the UE shall meet the minimum peak EIRP requirement and spherical coverage requirement with its autonomously chosen UL beams and without uplink beam sweeping. Such UE is considered to have met the beam correspondence tolerance requirement. In other words, no need to introduce Beam Correspondence requirement for as Rel-15 PC3.

FR2 HST UE support of Rel-16 feature beamCorrespondenceSSB-based-r16 shall be mandated but the support of Rel-16 beamCorrespondenceCSI-RS-based-r16 shall be optional.

## 7.2	RRM requirements

Concerning the maximum supported speed for FR2 HST RAN4 agreed to use 350kmph as a reference maximum train speed and define RRM requirements to guarantee that.

It was agreed to add a flag to enable the UE to identify different/enhanced RRM requirements in HST FR2 deployments. FR2 HST UE has the capacity to support both unidirectional and bidirectional deployment scenarios.

In Scenario A, whether the RRH position on one side or both sides of railway tracks have no impact on RRM requirements under the assumption that FR2 HST UE boresight direction (or the beam direction if there is only one beam) is parallel to the track.

Under FR2 HST scenarios, the PSS/SSS detection is robust to deal with ISI and time differences.

Additional need for network signalling and CPE capabilities for HST FR2 deployments is still FFS and will continue once the deployment options and presence of non-CPE UEs is clarified.

### 7.2.1	Idle/inactive mode

RAN agreed for idle/inactive mode that the cell reselection requirements in IDLE/INACTIVE mode are enhanced.

Concerning requirements for when UE is applying long DRX RAN4 agreed to apply the existing R16 requirements for when the long DRX cycles are used, i.e. above the upper bound of DRX cycle.

HST FR2 enhanced requirement is applied to SMTC <=40ms. SMTC periodicity is not restricted.

### 7.2.2	Connected mode

For connected mode mobility RAN4 agreed not to support RRC Release with Redirection in the FR2 HST scenario.

RAN4 agreed to revise the TCI state known conditions for the FR2 HST scenario.

Concerning requirements for when UE is applying long DRX RAN4 agreed to apply the existing R16 requirements for when the long DRX cycles are used, i.e. above the upper bound of DRX cycle.

HST FR2 enhanced requirement is applied to SMTC <=40ms. SMTC periodicity is not restricted.

For handover and the criteria of known cell for FR2, the target FR2 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds; otherwise, it is unknown.

For RLM and BFD the current sharing factor P and PCBD can be reused for FR2 HST.

## 7.3	Demodulation performance requirements

It is feasible to support maximum speed with 350km for uplink with PTRS or DMRS+PTRS configuration used for frequency offset tracking with 120KHz SCS

Configure PTRS during the PUSCH demodulation test

DMRS+PTRS configuration for PUSCH demodulation requirement with single-tap channel model

-	Option 1: 1 DMRS +PTRS (L=1,K=2); or

-	Option 2: 1+1 DMRS +PTRS (L=1,K=2); or

-	Option 3: 1+1+1 DMRS+PTRS(L=1, K=2).

### 7.3.1	UE demodulation  requirements

Test Scope of DL requirements:

-	Only define PDSCH demodulation performance requirements in Rel-17 FR HST WI.

-	Doppler frequency for PDSCH requirement in Bi-directional deployment scenario, if Bi-directional deployment scenario is introduced:

-	Option 1: 9722Hz targeting 350km/h at 30GHz

-	Option 2: 7000Hz with the smallest RS range of frequency offset estimation

-	the maximum Doppler frequency offset for PDSCH requirement in unidirectional deployment scenario

-	9722 Hz targeting 350km/h at 30 GHz

-	RAN4 decided not define PDSCH requirement with HST single-tap channel.

Requirement for uni-and bi-directional RRH deployment scenarios:

-	Consider output of FR2 HST Deployment scenarios discussion whether to cover uni- and/or bi-directional RRH deployment.

-	No dedicated PDSCH requirement for bidirectional Scenario A

-	Introduce PDSCH requirement for unidirectional Scenario A if the feasibility of unidirectional deployment is confirmed

-	Introduce PDSCH requirement in unidirectional and bidirectional for Scenario B

Transmission schemes:

-	No PDSCH requirement with SFN joint transmission scheme in Rel-17 FR2 HST WI.

-	DPS transmission schemes in uni-directional RRH deployment scenario. Introduce DPS scheme 1a and scheme 1b for PDSCH requirement in uni-directional scenario if the feasibility of uni-directional deployment is confirmed:

-	Option 1: scheme 1a

-	Option 2: scheme 1b

-	Option 3: both scheme 1a and scheme 1b

-	DPS transmission scheme in bi-directional RRH deployment scenario. Introduce DPS scheme 1a for PDSCH requirement in bi-directional scenario of scenario B:

-	Option 1: scheme 1a

-	Option 2: scheme 1b

-	Option 3: both scheme 1a and scheme 1b

SCS and BW:

-	Option 1: 120KHz with 100MHz; or

-	Option 2: 120KHz with 200MHz.

BW:

-	200 MHz CBW is defined.

Regarding the test setup for PDSCH RAN4 has reached following agreements:

RS configuration:

1)	the assumption of RS for frequency offset tracking is up to UE implementation. The RS configuration for PDSCH requirement is FFS and options are:

a)	Configure SSB every 20ms; and

b)	Configure TRS every 10ms; and

c)	Configure PTRS with KPT-RSS=2 and LPT-RS=1.

DMRS configuration. RAN4 has agreed:

-	1+1+1 DMRS configuration for DPS scheme.

UE frequency error:

-	RAN4 agreed not consider extra UE frequency error for demodulation tests in FR2 HST WI. Any impact of UE frequency error can be included in companies’ impairment results when RAN4 sets the UE demodulation requirement for FR2 HST.

UE demodulation test:

### 7.3.2	BS demodulation  requirements

Test scope of UL requirements. RAN4 will only define the following BS demodulation performance requirements in Rel-17 FR HST WI

-	PUSCH

1)	RAN4 agreed to introduce PUSCH requirement with Doppler frequency as 19444Hz targeting 350km/h at 30GHz.

2)	Additionally, RAN4 has agreed not introduce PUSCH requirement with Doppler frequency as 14444Hz targeting 260km/h at 30GHz, if no issue with supporting 350km/h was identified.

3)	No dedicated PUSCH requirement for bidirectional Scenario A

4)	Introduce PUSCH requirement for unidirectional and bidirectional Scenario B

5)	Introduce PUSCH requirement in Uni-directional for Scenario A if the feasibility of Uni-directional deployment is confirmed.

-	UL timing adjustment

1)	PRACH

Test Setup for PUSCH requirements:

1)	Waveform:

a)	Only CP-OFDM.

2)	SCS&BW:

a)	Option 1: 120KHz SCS with 50MHz, 100MHz or 200MHz; and

b)	Option 2: 120KHz SCS with 100MHz; and

c)	Option 3: 120KHz SCS with 200MHz.

3)	Antenna Configuration:

1)	1Tx2Rx Low.

4)	Resource mapping type: type B.

5)	Length of data symbol:

The length of PUSCH data symbol is 10.

6)	MCS:

a)	Option 1: MCS16 with Option 1a: Additional margin can be considered for performance requirement definition to allow different implementation if needed; and

b)	Option 2: MCS16 and MCS17 and define requirements with MCS17 up to BS declaration support.

c)	Option 3: Configure highest MCS that remains below 20dB SNR, i.e, MCS20.

d)	Further discuss how to guarantee 64QAM operation.

e)	Further discuss how to not preclude any possible BS implementations (with pre and post FFT FOC).

f)	Other options are not precluded.

7)	RS configuration:

a)	Option 1: 1 DMRS +PTRS (L=1,K=2); and

b)	Option 2: 2 DMRS+ PTRS (L=1,K=2); and

c)	Option 3: 3 DMRS +PTRS (L=1,K=2) with Option 3a: If companies have strong concern about DMRS 1+1, create an applicability rule that only one DMRS configuration shall be tested by manufacture declaration.

8)	CBW:

Both 50MHz and 200MHz CBWs are defined with the test applicability rule that only one of them is tested based on BS manufacturer.

9)	Phase noise model:

a)	No explicit phase noise modelling in the alignment results

b)	Realistic phase noise modelling is left up to the contributing entities

c)	The phase noise impact can be included in the impairment results, but it is left up to companies

10)	  With regard to the test metric for PUSCH requirements, only 70% of the throughput is used

Table 7.3.2.1

| Parameter |  | Value |
| --- | --- | --- |
| Channel Model |  | Uni-directional scenario A and B Bi-directional scenario B The details can be referred as R4-2115725 |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | [Pos0], [Pos1] and [Pos2] |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 50MHz and 200MHz |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

Test Setup for UL timing adjustment requirement:

It was agreed to introduce UL timing adjustment requirements for FR2 HST.

1)	Waveform:

a)	Only CP-OFDM.

2)	CBW: align CBW for UL timing adjustment and PUCH requirements. 50MHz and 200MHz CBWs are defined with the test applicability rule that only one of them is tested based on BS manufacturer. The existing PUSCH applicability rule for different  channel bandwidth for UL timing adjustment requirements are:

a)	For each subcarrier spacing declared to be supported, the test requirements for a specific channel bandwidth shall apply only if the BS supports it.

b)	Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

3)	PUSCH resource allocation:

a)	50MHz CBW: 16RBs for each UE, Moving UE RBs: 0~15; Stationary UE RBs: 16~31

b)	200MHz CBW: 66RBs for each UE, Moving UE RBs: 0~65; Stationary UE RBs: 66~131

4)	RS configuration:

a)	Align RS configuration for UL timing adjustment requirement and PUSCH requirement

5)	PUSCH mapping type:

a)	Type B.

6)	Length of PUSCH allocation. Align with PUSCH for UL timing adjustment:

a)	Option 1: 10; and

b)	Option 2: 9.

7)	MSC, Align with PUSCH for UL timing adjustment:

a)	Only MCS16 is used.

8)	SRS bandwidth configuration:

a)	50MHz CBW (32RBs)~C_SRS =9, B_SRS =0

b)	 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0

9)	SRS Transmission comb: KTC=2. SRS transmission in UL timing adjustment requirement is optional

10)	SRS Transmission periodicity : KSRS=10.

11)	Slots in which sounding RS is transmitted:

a)	The last symbol in slot#3 in radio frames for 120KHz SCS.

12) Test Parameters for timing offset:

a)	Option 1: A: 1.25 us and Δw:  1.04 s-1 corresponding to 120KHz SCS for HST FR2 UL timing adjustment requirements; and

b)	Option 2:  FFS on A =2.5 us; and

c)	Other options are not preluded

13) Test different between moving UE and stationary UE:

a)	Option 1: [Δt-(TA-31)x16*8Tc]. Note: The timing different can be updated with taken into account the output of possible enhancements for timing adjustment command discussion in RRM session

14)  The length of PUSCH data is 10

15)  Test Scenario:

a)	Scenario Y.

15) Simulation Assumption for scenario Y:

a)	Option 1.

Table 7.3.2.2

| Parameter | Value |
| --- | --- |
| Channel Model | Stationary UE: AWGN, Moving UE: AWGN |
| UE Speed | 350 km/h |
| Waveform | CP-OFDM |
| CP Length | Normal |
| A | 1.25 μs |
| ∆ꞷ | 1T2R |
| MCS | 16 |
| CBW | 50MHz and 200MHz |
| DM-RS type | Type 1 |
| Mapping type | B |
| Length of PUSCH data | 10 |
| RS configuration | Option 1: 1DMRS+PT-RS(L=1,K=2) Option 2: 2DMRS+PT-RS(L=1,K=2) Option 3: 3DMRS+PT-RS(L=1,K=2) |
| Number of DM-RS CDM group(s) without data | 2 |
| Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
| DM-RS port | {0} |
| DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| PUSCH resource allocation | 50MHz: 0 to 15 RB for moving UE, 16 to 31 for stationary UE (if 100MHz 200MHz: 0 to 65RB for moving UE, 66 to 131 for stationary UE |
| SRS resource allocation | 50MHz CBW (32RBs)~C_SRS =9, B_SRS =0 200MHz CBW (132RBs)~ C_SRS=33, B_SRS=0 |

Test setup for PRACH:

1)	PRACH format:

-	only C2;

-	Channel

-	AWGN

2)	Frequency offset:

a)	19444Hz with 350km/h at 30GHz carrier frequency.

3)	Test Preamble Configuration for Ncs. RAN4 agreed to use option 1 as baseline.:

a)	Ncs=0

4)	Timing offset configuration:

a)	Option 1: Reuse Rel-15 FR2 timing offset configuration for PRACH, i.e., 0.8us; and

b)	Option 2: Update the timing offset configuration based on the largest expected cell radius, i.e., derived from scenario B. Note:

-	Scenario A (Ds=700m, Dmin=10m), cell radius = 700m

-	Scenario B (Ds=700m, Dmin=150ms), cell radius = 716ms

5)	Timing error tolerance:

a)	0.07us for AWGN, as a default value for 120KHz SCS.

Table 7.3.2.3

| Parameters | Value |
| --- | --- |
| Ncs | 0 |
| Logical sequence index | 0 |
| v | 0 |
| Channel | AWGN |
| Antenna configuration | 1T2R |
| SCS | 120KHz |
| Frequency offset | 19444Hz |
| Time error tolerance | 0.07us |
| Timing offset | Option 1: 0.8us with TO range [0:0.1:0.8] Option 2: [4.6]us with TO range [0:0.46:4.6] |

# 8	Conclusion

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | RAN4#98 | R4-2103240 |  |  |  | Way forward on Deployment Scenario and UE RF Requirement for FR2 HST | 0.0.1 |
| 2021-05 | RAN4#98bis | R4-2105794 |  |  |  | WF on FR2 HST RRM requirements | 0.0.1 |
| 2021-05 | RAN4#98bis | R4-2106100 |  |  |  | WF on FR2 HST Deployment Scenario Analysis | 0.0.1 |
| 2021-05 | RAN4#98bis | R4-2106101 |  |  |  | Way Forward on Channel Modeling for FR2 HST | 0.0.1 |
| 2021-05 | RAN4#98bis | R4-2106102 |  |  |  | WF on Demodulation requirement for FR2 HST | 0.0.1 |
| 2021-08 | RAN4#99 | R4-2107861 |  |  |  | WF on UE RF requirement for FR2 HST | 0.1.0 |
| 2021-08 | RAN4#99 | R4-2108660 |  |  |  | WF on FR2 HST Deployment Scenario Analysis | 0.1.0 |
| 2021-08 | RAN4#99 | R4-2108661 |  |  |  | WF on Channel Modeling for FR2 HST | 0.1.0 |
| 2021-08 | RAN4#99 | R4-2108637 |  |  |  | WF for FR2 HST Demodulation | 0.1.0 |
| 2021-08 | RAN4#99 | R4-2108342 |  |  |  | WF on FR2 HST RRM requirements | 0.1.0 |
| 2021-08 | RAN4#99 | R4-2105025 |  |  |  | Channel modeling for FR2 HST and TP to TR 38.854 | 0.1.0 |
| 2021-11 | RAN4#100 | R4-2114976 |  |  |  | WF on UE RF requirement for FR2 HST | 0.1.0 |
| 2021-11 | RAN4#100 | R4-2115725 |  |  |  | WF on Remaining issues on FR2 HST deployment scenario and channel modeling | 0.1.0 |
| 2021-11 | RAN4#100 | R4-2115726 |  |  |  | WF for FR2 HST Demodulation | 0.1.0 |
| 2021-11 | RAN4#100 | R4-2115334 |  |  |  | WF on FR2 HST RRM requirements (part 1) | 0.1.0 |
| 2021-11 | RAN4#100 | R4-2115335 |  |  |  | WF on FR2 HST RRM requirements (part 2) | 0.1.0 |
| 2021-11 | RAN4#100 | R4-2115809 |  |  |  | TP to TR 38.854 – beam dwelling time for FR2 HST |  |