3GPP TSG-RAN WG4 Meeting # 101-e                                						R4-2119098

Electronic Meeting, 1– 12 November, 2021

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.717-21-11: DC_1A_(n)3AA

Agenda Item:	7.14

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE B1, B3 and NR n3 for TR 37.717-21-11[1].

# Reference

- . 37.717-21-11 Rel-17 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)

# Text Proposal

----- Start of TP -----

## 5.x	DC_1_(n)3

### 5.x.1	Operating bands for EN-DC

Table 5.x.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1_(n)3 | CA_1-3 | n3 | DC_1_n3 |

### 5.x.2	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A_(n)3AA | DC_1A_n3A | CA_1A-3A | n3A |

### 5.x.3		∆TIB and ∆RIB values

For DC_1-(n)3, the TIB,c and RIB,c values are reused from DC_1_n3 and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_(n)3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n3 | 0.3 |

Table 5.131.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_(n)3 | 1 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |

### 5.131.4	REFSENS requirements

No additional MSD requirements need to be defined.

----- End of TP -----