3GPP TSG-RAN WG4 Meeting # 100-e	R4-2119132

# Electronic Meeting, August 16-27, 2021

Source: 	Huawei

Title: 	Analysis of the updated IEC 61000-4-3 specification: upper frequency range for radiated immunity

Agenda Item:		4.1.5

Document for:	Discussion

# Introduction

IEC 61000-4-3 specification covers testing and measurement techniques for the electromagnetic field immunity testing. This specification is referred by all RAN4 EMC specifications. Recently, IEC 61000-4-3 has been updated in [1], with the following technical improvements [2]:

- testing using multiple test signals has been described;

- Additional information on EUT and cable layout has been added;

- The upper frequency limitation has been removed to take account of new services;

- The characterization of the field as well as the checking of power amplifier linearity of the immunity chain are specified.

Modification in bullet #3 is of importance for RAN4 as it removes the 6 GHz upper frequency limit for the radiated immunity testing.

In this contribution we analyse the latest updates to the IEC 61000-4-3:2020 and related implications to the RAN4 EMC specifications.

# Discussion

# Impacted specifications

Based on the analysis of RAN4 EMC specifications, the following specifications (BS, UE, IAB, Repeater) refer to the IEC 61000-4-3 specification, and the 6000MHz upper limit for the RI test:

- TS 38.113

- TS 38.114 (under construction, but expected to reuse TS 38.113 approach)

- TS 38.124

- TS 38.175

- TS 37.113

- TS 37.114

- TS 36.113

- TS 25.113

Please note that TS 36.124 specification applies the RI test only in “80 MHz - 1000 MHz and 1400 MHZ to 2700 MHz” frequency range. This is considered to be an overlooked issue, which shall be corrected to reflect the IEC 61000-4-3, i.e. to cover the range from 80 MHz to 6000 MHz for the radiated immunity test.

Proposal 1: update TS 36.124 and apply the RI test in 80 – 6000 MHz frequency range, as per IEC 61000-4-3. Apply the correction from Rel-15 onwards, to align with the NR BS EMC spec TS 38.113.

In order to determine which EMC specs require to be updarted (i.e. updates for the upper f. limit for the RI test) to reflect the referred updates in IEC 61000-4-3:2020 in [1], we are checking the highest operating bands for the RATs, as per Rel-17:

| RAT | Applicable EMC spec | Highest operating band | Related frequency range (MHz) | Update required due to IEC 61000-4-3? |
| --- | --- | --- | --- | --- |
| NR | TS 38.113 TS 38.114 TS 38.124 TS 38.175 | FR1: FR2: n262, TDD | FR1: 5925-7125 FR2: 47200-48200 | Yes |
| MSR (NR, E-UTRA, NB-IoT, UTRA) | TS 37.113 TS 37.114 | NR: n77 E-UTRA, NB-IoT: 43 UTRA: d (MSR band 38) | NR: 3300-4200 E-UTRA: 3600-3800 UTRA: 2570-2620 | No |
| E-UTRA | TS 36.113 | band 46, TDD | 5150-5925 | No |
| UTRA | TS 25.113 | band XXII, FDD | UL: 3410-3490 DL: 3510-3590 | No |

Based on the above analysis, it can be concluded, that only 38-series NR-related EMC specifications would require updates due to IEC 61000-4-3:2020 revision.

Proposal 2: the following 38-series NR-related EMC specifications require updates due to IEC 61000-4-3:2020 revision:

- TS 38.113

- TS 38.114 (under construction)

- TS 38.124

- TS 38.175

Proposal 3: before proceeding to formal CRs to all the affected specs, it is proposed to first agree on the draft CR content to e.g. TS 38.113. Once it is stable, its content is to be replicated to other affected EMC specs (38.114, 38.124, 38.175).

# Timeline

One issue to decide is whether implement related modifications from Rel-16 onwards, or from Rle-17 versions of the RAN4 specifications. The referred IEC 61000-4-3 update has been released September 2020. At that time Rel-16 work was still ongoing, for both Core and Performance part (see e.g. agenda of RAN4#97-e in R4-2014000).

As modification of the frequency range applicable to the Radiated Immunity testing is considered to be a Core requirement modification, it is seen more suitable not to modify legacy releases, and to apply such modifications to Rel-17 version of the RAN4 EMC specifications.

Proposal 4: RAN4 EMC modifications related to the IEC 61000-4-3 updates are to be applied from Rel-17 onwards.

# Conclusion

Based on the discussion above, the following proposals are formulated:

Proposal 1: update TS 36.124 and apply the RI test in 80 – 6000 MHz frequency range, as per IEC 61000-4-3.

Proposal 2: the following 38-series NR-related EMC specifications require updates due to IEC 61000-4-3:2020 revision:

- TS 38.113

- TS 38.114 (under construction)

- TS 38.124

- TS 38.175

Proposal 3: before proceeding to formal CRs to all the affected specs, it is proposed to first agree on the draft CR content to e.g. TS 38.113. Once it is stable, its content is to be replicated to other affected EMC specs (38.114, 38.124, 38.175).

Proposal 4: RAN4 EMC modifications related to the IEC 61000-4-3 updates are to be applied from Rel-17 onwards.

# References

[1]	IEC 61000-4-3:2020	Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test, IEC

[2]	https://webstore.iec.ch/publication/59849