3GPP TSG-RAN WG4 Meeting # 101e 	R4-2119297

# Electronic Meeting, Nov. 1-12, 2021

Source: 	vivo

Title: 	Discussion on inter-frequency measurement requirements for NR FR1 HST

Agenda Item:	8.8.2.2

Document for:	Discussion

Introduction

In RAN4 100e, the WF R4-2115439 on FR1 HST RRM enhancements was agreed in [1]. For inter-frequency measurements, the following was discussed and agreed

Issue 2-1: whether to define the enhancement for inter-frequency measurement in idle mode for HST

- Agreements

  - Define the enhancement for inter-frequency measurement in IDLE mode for HST

  - Support of HST Idle mode inter-frequency measurement enhancements is an optional UE feature without capability signalling

  - UEs not supporting inter-frequency measurement enhancements shall meet the existing inter-frequency measurement requirements

Issue 2-5: measurement delay requirement for inter-frequency measurement with MG in HST in connected state for HST

- Agreements:

| DRX cycle | T SSB_measurement_period_intra for FR1 HST |
| --- | --- |
| No DRX | max(200ms, N1 Max(MGRP, SMTC period)) CSSFinter N1 = 7 |
| DRX cycle ≤ 160ms | max(200ms, ceil(N2) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter N2 = 7 x M2 |
| 160ms < DRX cycle ≤ 320ms | ceil(N3) x DRX cycle x CSSFinter N3 = 7 x M2 |
| DRX cycle>320ms | N4 x DRX cycle x CSSFinter N4 = 4 x M2 |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

However, the following remaining issues are identified for inter-frequency HST measurements:

- Inter-frequency requirements in RRC IDLE/INACTIVE.

- PSS/SSS detection time requirements for inter-frequency measurements within gap.

In this paper our views on these issues are provided.

Discussion

On the remaining issues for requirements for connected state inter-frequency measurement

Regarding the cell identification requirements for the case inter-f measurement is done within gaps, for the case of intra-frequency discussed in R16, no enhancements except the removal of the 1.5 scaling factor under certain conditions. In our view, the same methodology should be re-used for R16. The proposed revision for this case can be found in Table I.

Table 1  PSS/SSS detection time requirement for inter-frequency measurement with MG in connected state for HST

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*M2) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: When high speed is not configured, M2 = 1.5. When high speed is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

Proposal 1  For inter-frequency cell identification in R17 FR1 HST, the enhancement is removal of 1.5 scaling factor under certain condition, the same as R16 HST.

On enhancements to idle/inactive state inter-frequency measurement requirements

In R16, inter-RAT measurement was enhanced in the idle mode for HST. The enhancement includes both inter-RAT E-UTRA measurements in NR, and inter-RAT NR measurements in LTE. The main motivation was to support fast cell re-selection in the high-speed scenario. For inter-frequency measurement conducted in idle mode, the UE behaviour would be the same no matter the serving cell is LTE or NR. Therefore, in our view, then inter-RAT NR carrier measurement requirement specified in TS 36.133 can be used at least as a baseline for inter-frequency measurement requirements performed in idle mode.

Table 2 Enhanced NR inter-frequency measurement requirements in idle mode

| DRX cycle length [s] | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 1 | 0.64 x M3 (2 x M3)Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 7.68 (12) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=M2=M3=1. |  |  |  |

Proposal 2  Inter-RAT NR carrier measurement requirements specified in TS 36.133 for R16 HST WI can be used at least as a baseline for inter-frequency measurement requirements in idle/inactive mode in R17.

In last meeting, the following was discussed.

Issue 2-3: FFS on the principle on the requirements if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured

- Option 1 (CATT, CMCC, OPPO, Ericsson, HW, MTK, QC, vivo, Xiaomi): NHST_inter-f carrier * THST_interf + NnonHST_inter-f carrier * TnonHST_interf can be specified if there are HST inter-frequency carriers and non-HST inter-frequency carriers to be measured in idle mode

- Option 2: FFS

This is also reused from R16 inter-RAT measurements. In R16 there was long discussion on how to define inter-RAT requirements. In our view, for R17 the arguments are still valid and therefore reusing R16 approach can be acceptable.

Proposal 3  The principle on the requirements if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured can be re-used in R17 FR1 HST.

Conclusions

Based on above analysis, we have following proposals.

Proposal 1  For inter-frequency cell identification in R17 FR1 HST, the enhancement is removal of 1.5 scaling factor under certain condition, the same as R16 HST.

Proposal 2  Inter-RAT NR carrier measurement requirements specified in TS 36.133 for R16 HST can be used at least as a baseline for inter-frequency measurement requirements in idle/inactive mode in R17 FR1 HST.

Proposal 3  The principle on the requirements if there are both HST inter-frequency layers and non-HST inter-frequency layers to be measured can be re-used in R17 FR1 HST.

- References

[1]  R4-2115439  WF on RRM for FR1 HST, CMCC. RAN4 #100e