3GPP TSG-RAN WG4 Meeting #101-e 	R4-2119307

Online, 01 - 12 Nov 2021

Source: 	Huawei

Title: 	TP to TS 38.106 - Repeater conducted output power

Agenda Item:	8.5.2.1

Document for:	Approval

# Introduction

The repeater class definitions and output power requirements are almost finished and with the discussion paper on output power R4-2119306 this TP shows how the repeater power requirements could be captured.

# TP to TS 38.106 v.0.0.1

- < Start of changes >

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

rated output power, mean power level associated with a particular pass band the manufacturer has declared to be available at the indicated interface.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

Pmax,AC	Maximum output power measured per antenna connector

Prated,AC	The rated output power per antenna connector

Pin_rated_cond	The conducted rated input power intended to produce the maximum rated output power

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

< next change >

## 6.2	Repeater output power

### 6.2.1	General

The repeater conducted output power requirement is at antenna connector.

The maximum rated output power of the repeater type 1-C shall be as specified in table 6.2.1-1 and table 6.2.1-2.

Table 6.2.1-1: repeater type 1-C DL transmission classes rated output power limits for repeater classes

| repeater class | Prated,AC |
| --- | --- |
| Wide Area repeater | (Note) |
| Medium Range repeater | ≤ 38 dBm |
| Local Area repeater | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,AC rated output power of the Wide Area repeater |  |

Table 6.2.1-2: repeater type 1-C UL transmission classes rated output power limits for repeater classes

| repeater class | Prated,AC |
| --- | --- |
| Wide Area repeater | (Note) |
| Local Area repeater | ≤ 24 dBm |
| NOTE: There is no upper limit for the Prated,AC rated output power of the Wide Area repeater. |  |

## 6.1	Minimum requirement

The requirements shall apply at maximum gain, with NR signals in the pass band of the repeater at:

The level that produce the maximum rated output power per channel (Pin_rated_cond).

The level that produce the maximum rated output power per channel (Pin_rated_cond) plus 10dB

In normal conditions, Pmax,AC shall remain within +2 dB and -2 dB of the rated output power Prated,AC, declared by the manufacturer.

In extreme conditions, Pmax,AC shall remain within +2.5 dB and -2.5 dB of the rated output power Prated,AC, declared by the manufacturer.

< End of change >