3GPP TSG-RAN WG4 Meeting #101-e	R4-2119357

Electronic Meeting, 1 – 12 November, 2021

Title: 	Simulation results for reduced sample number for PRS measurement

Source: 	Huawei, HiSilicon

Agenda item:	8.21.2.2

Document for:	Discussion

Introduction

Based on the RAN1 incoming LS [1], RAN4#100-e discussed the reduced number of samples for positioning measurement for positioning latency reduction. The outcome of the discussions are captured in WF [2].

Based on the following agreement in [1], RAN4 is supposed to conduct performance evaluation for PRS measurement with reduced number of samples, in order to obtain the conclusion on the exact sample number, the impact on the accuracy and the related side conditions.

- Low latency enhancement - It is RAN4 understanding that the reduction of the number of DL PRS processing samples is possible under certain conditions - In some cases, the reduction of the number of DL PRS processing samples is feasible under assumption of relaxation of the Rel-16 NR positioning accuracy requirements for the existing side conditions (e.g. SINR, PRS configurations, channel models, etc.) - In some cases, the reduction of the number of DL PRS processing samples is feasible under assumption of keeping Rel-16 NR positioning accuracy requirements and for the case of using different side conditions (e.g. SINR, PRS configurations, channel models, etc.) - For Rel-17, low latency NR Positioning requirements definition the goal is to meet the existing Rel-16 NR positioning accuracy requirements - FFS whether to consider limited relaxations of requirements for specific scenarios - Further study the impact of reducing number of processing samples - Number of processing PRS samples: 1, 2, 3, 4 (reference/R16 assumption) - PRS BW: FFS - SNR conditions: - Option 1: Rel-16 side condition - Option 2: Higher SNR side conditions than in Rel-16 - Channel models: - Option 1: Rel-16 channel models - Option 2: LOS channel models (e.g. TDL-D, TDL-E) - Note: other parameters and options are not precluded

In this paper we will provide our simulation results for reduced number of samples for positioning measurement.

Discussion

Simulation assumption

Some simulation parameters are listed in Table 1, and other simulation assumptions are same as in [3, 4]. The parameters in Table 1 correspond to the cases for which accuracy requirements are defined in Rel-16.

Table 1: Simulation assumption for reduced PRS sample number

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| PRS parameters | SCS (kHz) | RB num | Repetition (Note) |
|  | 15 | 24 | 4 |
|  |  | 52 | 1 |
|  |  | 104 | 1 |
|  | 30 | 24 | 4 |
|  |  | 48 | 1 |
|  |  | 132 | 1 |
|  | 60, FR1 | 24 | 4 |
|  |  | 64 | 1 |
|  |  | 132 | 1 |
|  | 60, FR2 | 24 | 4 |
|  |  | 64 | 1 |
|  |  | 132 | 1 |
|  | 120 | 32 | 4 |
|  |  | 64 | 1 |
|  |  | 128 | 1 |
| PRS comb size | 4 |  |  |
| PRS symb size | 4 |  |  |
| SNR | -6dB for reference cell {-6, -13}dB for neighbour cell |  |  |
| Channel model | AWGN, TDL-A, TDL-D for FR1 AWGN, TDL-C, TDL-D for FR2 |  |  |
| Sample number | 1, 2, 4 |  |  |
| Note: Repetition 4 is achieved by PRS in 4 consecutive slots. |  |  |  |

Simulation results for RSTD

The simulation results for RSTD are shown in Table 1-3, for AWGN, TDL-A/C and TDL-D channels. Same as for Rel-16, TDL-A channel is used for FR1 and TDL-C is used for FR2. The results are for the maximum between 5% and 95% of RSTD error in absolute value.

Note: the results for 120kHz are currently missing, and we will add them when results are ready.

Table 1: RSTD performance for AWGN channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 1 | 2 | 4 |
|  |  | Es/Iot = (-6, -13, -13) dB |  |  | Es/Iot = (-6, -6, -6) dB |  |  |
| 15 | 24 | 76 | 64 | 50 | 52 | 40 | 36 |
|  | 52 | 36 | 32 | 28 | 28 | 26 | 24 |
|  | 104 | 24 | 24 | 23 | 22 | 22 | 22 |
| 30 | 24 | 32 | 30 | 27 | 28 | 26 | 24 |
|  | 48 | 77 | 28 | 26 | 26 | 24 | 23 |
|  | 132 | 86 | 21 | 21 | 20 | 21 | 20 |
| 60, FR1 | 24 | 73 | 32 | 29 | 28 | 25 | 24 |
|  | 64 | 55 | 22 | 21 | 21 | 21 | 21 |
|  | 132 | 79 | 20 | 20 | 20 | 20 | 20 |
| 60, FR2 | 24 | 62 | 32 | 29 | 28 | 25 | 24 |
|  | 64 | 91 | 22 | 21 | 21 | 21 | 21 |
|  | 132 | 69 | 20 | 20 | 20 | 20 | 20 |
| 120 | 32 |  |  |  |  |  |  |
|  | 64 |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |

Table 2: RSTD performance for TDL-A/C channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 1 | 2 | 4 |
|  |  | Es/Iot = (-6, -13, -13) dB |  |  | Es/Iot = (-6, -6, -6) dB |  |  |
| 15 | 24 | 1225 | 2916 | 120 | 116 | 108 | 100 |
|  | 52 | 2637 | 1501 | 82 | 80 | 76 | 74 |
|  | 104 | 2869 | 1263 | 63 | 64 | 64 | 63 |
| 30 | 24 | 2649 | 1537 | 83 | 88 | 86 | 86 |
|  | 48 | 1591 | 1834 | 60 | 54 | 57 | 58 |
|  | 132 | 1265 | 2753 | 47 | 40 | 43 | 45 |
| 60, FR1 | 24 | 1033 | 1516 | 76 | 62 | 67 | 66 |
|  | 64 | 2665 | 2581 | 32 | 30 | 30 | 30 |
|  | 132 | 2412 | 1293 | 34 | 31 | 32 | 34 |
| 60, FR2 | 24 | 1687 | 1878 | 70 | 90 | 73 | 58 |
|  | 64 | 1237 | 70 | 57 | 76 | 61 | 48 |
|  | 132 | 1844 | 79 | 64 | 81 | 76 | 59 |
| 120 | 32 |  |  |  |  |  |  |
|  | 64 |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |

Table 3: RSTD performance for TDL-D channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 1 | 2 | 4 |
|  |  | Es/Iot = (-6, -13, -13) dB |  |  | Es/Iot = (-6, -6, -6) dB |  |  |
| 15 | 24 | 2876 | 84 | 63 | 76 | 60 | 46 |
|  | 52 | 1003 | 34 | 29 | 32 | 28 | 26 |
|  | 104 | 1254 | 24 | 23 | 24 | 23 | 22 |
| 30 | 24 | 2843 | 34 | 29 | 32 | 30 | 26 |
|  | 48 | 1587 | 30 | 27 | 26 | 25 | 24 |
|  | 132 | 1156 | 21 | 21 | 20 | 21 | 20 |
| 60, FR1 | 24 | 1195 | 34 | 30 | 28 | 26 | 25 |
|  | 64 | 2509 | 22 | 21 | 21 | 21 | 21 |
|  | 132 | 1919 | 21 | 20 | 21 | 20 | 20 |
| 60, FR2 | 24 | 2106 | 34 | 30 | 28 | 26 | 25 |
|  | 64 | 1685 | 22 | 21 | 21 | 21 | 21 |
|  | 132 | 1563 | 21 | 20 | 21 | 20 | 20 |
| 120 | 32 |  |  |  |  |  |  |
|  | 64 |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |

Observation 1: For RSTD accuracy, with 1-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for some but not all cases under AWGN channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for all cases at -13dB Es/Iot under TDL-A/C or TDL-D channel

Observation 2: For RSTD accuracy, with 2-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for all cases under AWGN and TDL-D channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for most cases at -13dB Es/Iot under TDL-A/C channel

Simulation results for PRS-RSRP

The simulation results for PRS-RSRP are shown in Table 4-6, for AWGN, TDL-A/C and TDL-D channels. Same as for Rel-16, TDL-A channel is used for FR1 and TDL-C is used for FR2. The results are for 5% and 95% of PRS-RSRP error

Table 4: PRS-RSRP performance for AWGN channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 4 |  | 1 |  | 2 |  | 4 |  |
|  |  | Es/Iot = -13 dB |  |  |  |  |  | Es/Iot = -6 dB |  |  |  |  |  |
|  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 24 | -4.6 | 3.7 | -3.3 | 2.7 | -2.3 | 2.1 | -2.2 | 1.5 | -1.8 | 1.1 | -1.5 | 0.8 |
|  | 52 | -2.8 | 2.7 | -2 | 2.1 | -1.5 | 1.7 | -1.7 | 1.1 | -1.4 | 0.8 | -1.2 | 0.7 |
|  | 104 | -1.5 | 2.3 | -1.1 | 1.8 | -0.8 | 1.6 | -1.3 | 1 | -1.1 | 0.7 | -0.9 | 0.6 |
| 30 | 24 | -1.7 | 4.1 | -1.1 | 3.4 | -0.5 | 2.9 | 2.2 | 3.5 | 1.5 | 3.3 | 1 | 3.2 |
|  | 48 | -2.6 | 2.9 | -2 | 2.3 | -1.4 | 1.8 | -1.7 | 1.1 | -1.4 | 0.9 | -1.2 | 0.7 |
|  | 132 | -1.7 | 2.1 | -1.3 | 1.7 | -1 | 1.3 | -1.3 | 0.9 | -1.1 | 0.7 | -1 | 0.5 |
| 60, FR1 | 24 | -4 | 3.8 | -3.1 | 2.7 | -2.4 | 2.1 | -2.1 | 1.5 | -1.7 | 1.1 | -1.4 | 0.8 |
|  | 64 | -2.5 | 2.5 | -1.9 | 2 | -1.5 | 1.5 | -1.5 | 1 | -1.3 | 0.8 | -1.1 | 0.7 |
|  | 132 | -1.7 | 2.1 | -1.3 | 1.7 | -0.9 | 1.4 | -1.2 | 0.9 | -1.1 | 0.7 | -0.9 | 0.6 |
| 60, FR2 | 24 | -4 | 3.8 | -3.1 | 2.7 | -2.4 | 2.1 | -2.1 | 1.5 | -1.7 | 1.1 | -1.4 | 0.8 |
|  | 64 | -2.5 | 2.5 | -1.9 | 2 | -1.5 | 1.5 | -1.5 | 1 | -1.3 | 0.8 | -1.1 | 0.7 |
|  | 132 | -1.7 | 2.1 | -1.3 | 1.7 | -0.9 | 1.4 | -1.2 | 0.9 | -1.1 | 0.7 | -0.9 | 0.6 |
| 120 | 32 | 0.7 | 3.4 | 0.8 | 2.9 | 0.7 | 2.7 | 2.1 | 3.5 | 1.4 | 3.3 | 1 | 3.1 |
|  | 64 | -2.5 | 2.5 | -1.8 | 1.9 | -1.4 | 1.5 | -1.5 | 1 | -1.2 | 0.8 | -1.1 | 0.7 |
|  | 128 | -1.7 | 2 | -1.2 | 1.7 | -1 | 1.4 | -1.3 | 0.9 | -1.1 | 0.7 | -0.9 | 0.6 |

Table 5: PRS-RSRP performance for TDL-A/C channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 4 |  | 1 |  | 2 |  | 4 |  |
|  |  | Es/Iot = -13 dB |  |  |  |  |  | Es/Iot = -6 dB |  |  |  |  |  |
|  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 24 | -3.5 | 6.9 | -2.5 | 6.7 | -2 | 5.9 | -3.1 | 3 | -2.3 | 2.6 | -1.7 | 2.1 |
|  | 52 | -3.1 | 5 | -2.1 | 4.4 | -1.6 | 3.8 | -2.1 | 1.8 | -1.8 | 1.5 | -1.3 | 1.2 |
|  | 104 | -1.7 | 3.9 | -1.1 | 3.9 | -0.8 | 3.4 | -1.4 | 1.5 | -1.1 | 1.3 | -1 | 1.1 |
| 30 | 24 | -2.8 | 3.8 | -2.1 | 3.5 | -1.4 | 3.3 | -1.9 | 3.6 | -1.6 | 3.3 | -0.9 | 3.2 |
|  | 48 | -3.3 | 3.9 | -2.5 | 3.6 | -1.8 | 3.3 | -2.1 | 1.6 | -1.6 | 1.2 | -1.5 | 1 |
|  | 132 | -1.8 | 2.6 | -1.4 | 2.4 | -1 | 2.5 | -1.4 | 1 | -1.2 | 0.9 | -1 | 0.8 |
| 60, FR1 | 24 | -4.4 | 5.5 | -3.6 | 4.5 | -2.7 | 4.5 | -3.2 | 2.3 | -2.7 | 1.5 | -2.1 | 1.2 |
|  | 64 | -3.6 | 3 | -2.6 | 2.7 | -2.2 | 2.4 | -2.1 | 1.2 | -1.6 | 0.9 | -1.5 | 0.7 |
|  | 132 | -2 | 2.6 | -1.6 | 2 | -1.4 | 1.9 | -1.4 | 1 | -1.2 | 0.7 | -1.1 | 0.6 |
| 60, FR2 | 24 | -4.3 | 4.4 | -4.1 | 4.1 | -3.1 | 3 | -3.8 | 2.1 | -3 | 1.5 | -2.6 | 1 |
|  | 64 | -3.5 | 2.6 | -2.9 | 2.3 | -2.2 | 1.9 | -2 | 1 | -1.7 | 0.8 | -1.4 | 0.6 |
|  | 132 | -2.1 | 2.1 | -1.8 | 2 | -1.4 | 1.6 | -1.5 | 0.9 | -1.3 | 0.7 | -1.1 | 0.5 |
| 120 | 32 | -3.8 | 3.3 | -3.2 | 3 | -2.6 | 2.7 | -0.6 | 3.2 | 0.7 | 3 | 0.7 | 2.9 |
|  | 64 | -4.3 | 2.3 | -3.3 | 1.9 | -2.7 | 1.5 | -2.1 | 0.9 | -1.8 | 0.6 | -1.6 | -0.5 |
|  | 128 | -2.4 | 2 | -2.1 | 1.8 | -1.7 | 1.3 | -1.6 | 0.8 | -1.4 | 0.6 | -1.2 | -0.5 |

Table 6: PRS-RSRP performance for TDL-D channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 |  | 4 |  | 1 |  | 2 |  | 4 |  |
|  |  | Es/Iot = -13 dB |  |  |  |  |  | Es/Iot = -6 dB |  |  |  |  |  |
|  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| 15 | 24 | -5.5 | 4 | -3.7 | 3.1 | -2.6 | 2.3 | -3.2 | 2 | -2.4 | 1.3 | -2 | 1 |
|  | 52 | -2.9 | 2.9 | -2.1 | 2.1 | -1.6 | 1.8 | -1.8 | 1.1 | -1.4 | 0.9 | -1.2 | 0.7 |
|  | 104 | -1.9 | 2.4 | -1.3 | 1.8 | -1 | 1.7 | -1.4 | 1 | -1.2 | 0.7 | -1 | 0.6 |
| 30 | 24 | -2 | 3.8 | -1.4 | 3.2 | -0.9 | 2.7 | 2 | 3.4 | 1.3 | 3.2 | 1 | 3.1 |
|  | 48 | -3.2 | 2.9 | -2.4 | 2.2 | -1.6 | 1.8 | -1.9 | 1.1 | -1.6 | 0.8 | -1.3 | 0.7 |
|  | 132 | -1.9 | 2.1 | -1.5 | 1.7 | -1.1 | 1.3 | -1.4 | 0.8 | -1.2 | 0.6 | -1.1 | 0.5 |
| 60, FR1 | 24 | -4.7 | 3.7 | -3.4 | 3 | -2.5 | 2.3 | -2.4 | 1.5 | -1.9 | 1 | -1.6 | 0.8 |
|  | 64 | -2.8 | 2.4 | -2.2 | 2 | -1.6 | 1.5 | -1.8 | 0.9 | -1.5 | 0.7 | -1.3 | 0.6 |
|  | 132 | -1.9 | 2 | -1.4 | 1.7 | -1.1 | 1.3 | -1.4 | 0.8 | -1.2 | 0.6 | -1.1 | 0.5 |
| 60, FR2 | 24 | -4.7 | 3.7 | -3.4 | 3 | -2.5 | 2.3 | -2.4 | 1.5 | -1.9 | 1 | -1.6 | 0.8 |
|  | 64 | -2.8 | 2.4 | -2.2 | 2 | -1.6 | 1.5 | -1.8 | 0.9 | -1.5 | 0.7 | -1.3 | 0.6 |
|  | 132 | -1.9 | 2 | -1.4 | 1.7 | -1.1 | 1.3 | -1.4 | 0.8 | -1.2 | 0.6 | -1.1 | 0.5 |
| 120 | 32 | -0.8 | 3.2 | 0.6 | 2.8 | 0.6 | 2.5 | 1.5 | 3.2 | 1.1 | 3 | 0.9 | 2.9 |
|  | 64 | -2.9 | 2.6 | -2.2 | 2 | -1.7 | 1.6 | -1.8 | 1 | -1.6 | 0.7 | -1.3 | 0.6 |
|  | 128 | -1.8 | 2.1 | -1.4 | 1.7 | -1.2 | 1.4 | -1.4 | 0.8 | -1.2 | 0.6 | -1.1 | 0.5 |

Observation 3: For PRS-RSRP accuracy,

- there is non-negligible performance degradation when going from 4-sample to 2-sample and to 1-sample, under all propagation conditions and at both -13dB and -6dB Es/Iot.

- the performance degradation is smaller at -6dB Es/Iot compared to -13dB Es/Iot.

Simulation results for UE Rx-Tx

The simulation results for RSTD are shown in Table 7-9, for AWGN, TDL-A/C and TDL-D channels. Same as for Rel-16, TDL-A channel is used for FR1 and TDL-C is used for FR2. The results are for the 90% of Rx-Tx error.

Note: the results for 120kHz are currently missing, and we will add them when results are ready.

Table 7: Rx-Tx performance for AWGN channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 1 | 2 | 4 |
|  |  | Es/Iot = (-6, -13, -13) dB |  |  | Es/Iot = (-6, -6, -6) dB |  |  |
| 15 | 24 | 76 | 56 | 48 | 44 | 36 | 32 |
|  | 52 | 36 | 32 | 27 | 24 | 24 | 23 |
|  | 104 | 24 | 23 | 23 | 22 | 21 | 21 |
| 30 | 24 | 32 | 28 | 26 | 24 | 24 | 23 |
|  | 48 | 77 | 29 | 27 | 24 | 23 | 22 |
|  | 132 | 61 | 21 | 21 | 20 | 20 | 20 |
| 60, FR1 | 24 | 73 | 29 | 27 | 26 | 24 | 23 |
|  | 64 | 81 | 21 | 21 | 21 | 21 | 20 |
|  | 132 | 68 | 20 | 20 | 20 | 20 | 20 |
| 60, FR2 | 24 | 84 | 29 | 27 | 26 | 24 | 23 |
|  | 64 | 71 | 21 | 21 | 21 | 21 | 20 |
|  | 132 | 60 | 20 | 20 | 20 | 20 | 20 |
| 120 | 32 |  |  |  |  |  |  |
|  | 64 |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |

Table 8: Rx-Tx performance for TDL-A/C channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 1 | 2 | 4 |
|  |  | Es/Iot = (-6, -13, -13) dB |  |  | Es/Iot = (-6, -6, -6) dB |  |  |
| 15 | 24 | 1260 | 1844 | 135 | 148 | 140 | 132 |
|  | 52 | 1949 | 1593 | 107 | 100 | 102 | 101 |
|  | 104 | 2837 | 1832 | 89 | 86 | 89 | 93 |
| 30 | 24 | 1609 | 2017 | 99 | 100 | 98 | 107 |
|  | 48 | 1284 | 1985 | 80 | 72 | 72 | 71 |
|  | 132 | 2581 | 2309 | 54 | 53 | 55 | 54 |
| 60, FR1 | 24 | 2654 | 2873 | 98 | 90 | 85 | 90 |
|  | 64 | 2013 | 2653 | 57 | 54 | 54 | 55 |
|  | 132 | 2644 | 2328 | 58 | 57 | 54 | 58 |
| 60, FR2 | 24 | 2697 | 2357 | 110 | 132 | 112 | 102 |
|  | 64 | 1796 | 100 | 90 | 102 | 89 | 79 |
|  | 132 | 1441 | 103 | 102 | 104 | 104 | 101 |
| 120 | 32 |  |  |  |  |  |  |
|  | 64 |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |

Table 9: Rx-Tx performance for TDL-D channel

| SCS (kHz) | RB | Sample number |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 4 | 1 | 2 | 4 |
|  |  | Es/Iot = (-6, -13, -13) dB |  |  | Es/Iot = (-6, -6, -6) dB |  |  |
| 15 | 24 | 2830 | 72 | 57 | 68 | 48 | 40 |
|  | 52 | 2484 | 34 | 28 | 28 | 26 | 25 |
|  | 104 | 1275 | 24 | 23 | 22 | 22 | 22 |
| 30 | 24 | 2298 | 32 | 28 | 28 | 28 | 25 |
|  | 48 | 2993 | 29 | 27 | 24 | 23 | 23 |
|  | 132 | 2405 | 21 | 21 | 20 | 21 | 20 |
| 60, FR1 | 24 | 1222 | 31 | 28 | 26 | 25 | 24 |
|  | 64 | 1936 | 22 | 21 | 21 | 21 | 21 |
|  | 132 | 2163 | 21 | 20 | 21 | 20 | 20 |
| 60, FR2 | 24 | 1946 | 31 | 28 | 26 | 25 | 24 |
|  | 64 | 1966 | 22 | 21 | 21 | 21 | 21 |
|  | 132 | 1874 | 21 | 20 | 21 | 20 | 20 |
| 120 | 32 |  |  |  |  |  |  |
|  | 64 |  |  |  |  |  |  |
|  | 128 |  |  |  |  |  |  |

Observation 4: The performance trend for UE Rx-Tx is similar to RSTD.

Conclusions

In this paper we provided our views on reduced number of samples for positioning measurement.

Observation 1: For RSTD accuracy, with 1-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for some but not all cases under AWGN channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for all cases at -13dB Es/Iot under TDL-A/C or TDL-D channel

Observation 2: For RSTD accuracy, with 2-sample

- it is feasible to achieve similar accuracy as Rel-16 requirements at -6dB Es/Iot under all propagation conditions assumed in Rel-16

- it is feasible to achieve similar accuracy as Rel-16 requirements at -13dB Es/Iot for all cases under AWGN and TDL-D channel

- it is NOT feasible to achieve similar accuracy as Rel-16 requirements for most cases at -13dB Es/Iot under TDL-A/C channel

Observation 3: For PRS-RSRP accuracy,

- there is non-negligible performance degradation when going from 4-sample to 2-sample and to 1-sample, under all propagation conditions and at both -13dB and -6dB Es/Iot.

- the performance degradation is smaller at -6dB Es/Iot compared to -13dB Es/Iot.

Observation 4: The performance trend for UE Rx-Tx is similar to RSTD.

Reference

- R1-2106185, LS on PRS processing samples, RAN1

- R4-2115365, Way forward document for [100-e][236] NR_pos_enh_RRM_1, Moderator (Ericsson)