TSG-RAN Working Group 4 (Radio) meeting #101-e	R4-2119376

Electronic Meeting, 1th – 12th November 2021

Source:	T-Mobile USA

Title:	TP for TR 37.828: Filter and PA data for n71, n26 and n12

Agenda item:	7.28.2.3

Document for:	Approval

# 1	Introduction

The WID includes the following objective: 	Investigate the feasibility of filter with small duplex for B13, n13 and n71. This Text Proposal provides filter and PA data for n71, n26 and n12

# 2	Text Proposal

<First changed section>

# 7 	Band specific requirements for a UE

Editor note: This section relates to the Band Specific objectives of the WI.

## 7.x	Band n71

Editor note: All sub-clauses are not necessary for all bands.

### 7.x.1	REFSENS exception

### 7.x.2	A-MPR

### 7.x.3	Feasibility of the filter

A filter example is presented below with input power capabilities of 43 dBm which obviously in more than needed for PC1 operation even considering the post PA-losses but this demonstrates that capable technology is available.

Band 71/n71 duplex-filter UMD071A (ctscorp.com)

In addition to the above filter with 43 dBm capability, there are also filters with input power capabilities of 38 dBm, that are smaller and may be more appropriate for FWA devices. Nonetheless, the filters are still very large and may only be suitable for large form factor FWA.  Not all FWA devices may be able to accommodate these filters.

Band 71/n71 and Band 85 UL duplex-filter USD7185A (ctscorp.com)

### 7.x.4		Feasibility of the PA

Here are some commercial data sheets for PAs that appear to be compatible with n71 PC1 FWA devices.

| Device | Fmin (MHz) | Fmax (MHz) | Avg. power |  | OBO | Peak power |  | Voltage | Technology | Device type | Gain | Key parameter | Parameter description | Package | Package type | Matching | Configuration | Recommended driver |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3G26D055N | 100 | 2690 | 8 W | 39.0 dBm | 8.2 dB | 53 W | 47.2 dBm | 48 V | GaN | Discrete | 18 dB | 54% | drain efficiency (Doherty) at 8.2 dB OBO | DFN 7 x 6.5 | over-molded plastic | Input pre-matched, output unmatched | Dual path | A3V26S004N |
| A2I09VD030N | 575 | 960 | 4 W | 36.0 dBm | 10.1 dB | 41 W | 46.1 dBm | 48 V | LDMOS | IC | 34 dB | 20% | PAE (Class AB) at 10.1 dB OBO | TO-270WB-15 | over-molded plastic | 50-ohm input, output pre-matched | Dual path | MMG38151B |
| MW7IC915N | 600 | 900 | 2 W | 32.0 dBm | 9.9 dB | 16 W | 41.9 dBm | 28 V | LDMOS | IC | 38 dB | 17% | PAE (Class AB) at 9.9 dB OBO | PQFN 8 x 8 | over-molded plastic | Input pre-matched, output unmatched | Single path | MMG3014N |
| A2T27S020N | 400 | 2700 | 3 W | 34.0 dBm | 9.0 dB | 20 W | 43.0 dBm | 28 V | LDMOS | Discrete | 21 dB | 21% | drain efficiency (Class AB) at 9.0 dB OBO | TO-270-2 | over-molded plastic | Unmatched | Single path | A3M40PD012 |
| AFT09MS031N | 1.8 | 941 | 4 W | 36.4 dBm | 8.5 dB | 31 W | 44.9 dBm | 12.5 V | LDMOS | Discrete | 17 dB | 71% | drain efficiency (CW) | TO-270-2 | over-molded plastic | Unmatched | Single path | AFT05MS004N |

### 7.x.4	Other

## 7.y	Band n26

### 7.y.1	REFSENS exception

### 7.y.2	A-MPR

### 7.y.3	Feasibility of the filter

A filter example is presented below with input power capabilities of 43 dBm which obviously in more than needed for PC1 operation even considering the post PA-losses but this demonstrates that capable technology is available.

Band 26/n26 duplex-filter (subset of n26 for theUS market) UMD026B (ctscorp.com)

In addition to the above filter with 43 dBm capability, there are also filters with input power capabilities of 38 dBm, that are smaller and may be more appropriate for FWA devices. Nonetheless, the filters are still very large and may only be suitable for large form factor FWA.  Not all FWA devices may be able to accommodate these filters.

Band 26/n26 duplex-filter USD005A (ctscorp.com)

### 7.y.4		Feasibility of the PA

Here are some commercial data sheets for PAs that appear to be compatible with n26 PC1 FWA devices.

| Device | Fmin (MHz) | Fmax (MHz) | Avg. power |  | OBO | Peak power |  | Voltage | Technology | Device type | Gain | Key parameter | Parameter description | Package | Package type | Matching | Configuration | Recommended driver |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3G26D055N | 100 | 2690 | 8 W | 39.0 dBm | 8.2 dB | 53 W | 47.2 dBm | 48 V | GaN | Discrete | 18 dB | 54% | drain efficiency (Doherty) at 8.2 dB OBO | DFN 7 x 6.5 | over-molded plastic | Input pre-matched, output unmatched | Dual path | A3V26S004N |
| A2I09VD030N | 575 | 960 | 4 W | 36.0 dBm | 10.1 dB | 41 W | 46.1 dBm | 48 V | LDMOS | IC | 34 dB | 20% | PAE (Class AB) at 10.1 dB OBO | TO-270WB-15 | over-molded plastic | 50-ohm input, output pre-matched | Dual path | MMG38151B |
| MW7IC915N | 600 | 900 | 2 W | 32.0 dBm | 9.9 dB | 16 W | 41.9 dBm | 28 V | LDMOS | IC | 38 dB | 17% | PAE (Class AB) at 9.9 dB OBO | PQFN 8 x 8 | over-molded plastic | Input pre-matched, output unmatched | Single path | MMG3014N |
| A2T27S020N | 400 | 2700 | 3 W | 34.0 dBm | 9.0 dB | 20 W | 43.0 dBm | 28 V | LDMOS | Discrete | 21 dB | 21% | drain efficiency (Class AB) at 9.0 dB OBO | TO-270-2 | over-molded plastic | Unmatched | Single path | A3M40PD012 |
| AFT09MS031N | 1.8 | 941 | 4 W | 36.4 dBm | 8.5 dB | 31 W | 44.9 dBm | 12.5 V | LDMOS | Discrete | 17 dB | 71% | drain efficiency (CW) | TO-270-2 | over-molded plastic | Unmatched | Single path | AFT05MS004N |

### 7.y.4	Other

### 7.x.4	Other

## 7.z	Band n12

### 7.z.1	REFSENS exception

### 7.z.2	A-MPR

### 7.z.3	Feasibility of the filter

A filter example is presented below with input power capabilities of 43 dBm which obviously in more than needed for PC1 operation even considering the post PA-losses but this demonstrates that capable technology is available.

Band 12 duplex-filter  UMD012A (ctscorp.com)

In addition to the above filter with 43 dBm capability, there are also filters with input power capabilities of 38 dBm, that are smaller and may be more appropriate for FWA devices. Nonetheless, the filters are still very large and may only be suitable for large form factor FWA.  Not all FWA devices may be able to accommodate these filters.

Band 12 duplex-filter USD012A (ctscorp.com)

### 7.z.4		Feasibility of the PA

Here are some commercial data sheets for PAs that appear to be compatible with n26 PC1 FWA devices.

| Device | Fmin (MHz) | Fmax (MHz) | Avg. power |  | OBO | Peak power |  | Voltage | Technology | Device type | Gain | Key parameter | Parameter description | Package | Package type | Matching | Configuration | Recommended driver |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3G26D055N | 100 | 2690 | 8 W | 39.0 dBm | 8.2 dB | 53 W | 47.2 dBm | 48 V | GaN | Discrete | 18 dB | 54% | drain efficiency (Doherty) at 8.2 dB OBO | DFN 7 x 6.5 | over-molded plastic | Input pre-matched, output unmatched | Dual path | A3V26S004N |
| A2I09VD030N | 575 | 960 | 4 W | 36.0 dBm | 10.1 dB | 41 W | 46.1 dBm | 48 V | LDMOS | IC | 34 dB | 20% | PAE (Class AB) at 10.1 dB OBO | TO-270WB-15 | over-molded plastic | 50-ohm input, output pre-matched | Dual path | MMG38151B |
| MW7IC915N | 600 | 900 | 2 W | 32.0 dBm | 9.9 dB | 16 W | 41.9 dBm | 28 V | LDMOS | IC | 38 dB | 17% | PAE (Class AB) at 9.9 dB OBO | PQFN 8 x 8 | over-molded plastic | Input pre-matched, output unmatched | Single path | MMG3014N |
| A2T27S020N | 400 | 2700 | 3 W | 34.0 dBm | 9.0 dB | 20 W | 43.0 dBm | 28 V | LDMOS | Discrete | 21 dB | 21% | drain efficiency (Class AB) at 9.0 dB OBO | TO-270-2 | over-molded plastic | Unmatched | Single path | A3M40PD012 |
| AFT09MS031N | 1.8 | 941 | 4 W | 36.4 dBm | 8.5 dB | 31 W | 44.9 dBm | 12.5 V | LDMOS | Discrete | 17 dB | 71% | drain efficiency (CW) | TO-270-2 | over-molded plastic | Unmatched | Single path | AFT05MS004N |

### 7.z.4	Other

<End of changes>