3GPP TSG-RAN WG4 Meeting#101-e	R4-2119513

Online, 1st Nov – 12th Nov, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on PDSCH grant on TRS slots for CSI Reporting Tests

Agenda item:	4.2.4.1

Document for:	Discussion

- Introduction

In RAN4#100-e, an issue was brought up on changing the PDSCH grant pattern for CQI and RI reporting tests. In this paper, we provide our views on that issue.

- PDSCH grant on TRS slots

In [1], it was agreed to not schedule PDSCH on slots containing CSI-RS. This was done to align with reference resource setting in clause 5.2.2.5 in TS 38.214 because RAN4 is defining minimum performance requirements to verify CQI reporting requirements to verify if UE is properly calculating CQI and UE is supposed to calculate CQI based on reference resource setting. If we don’t align the test configuration with reference resource setting and schedule grant on slots containing TRS (which is also CSI-RS and should not have PDSCH grant as per agreement in [1]), we will see a change in code rate on TRS slots compared to other slots and that will impact CQI computation.

If other companies want to align the PDSCH grant configuration with other CSI reporting tests, it will be better to not schedule any grant on TRS slots in PMI reporting tests as well since PMI reporting is not impacted by observed code rate.

Therefore, we have following observations and proposals.

Observation 1: It was agreed in Rel-15 not to schedule PDSCH on slots containing CSI-RS for CSI reporting tests as captured in [1].

Observation 2: TRS is also a type of CSI-RS.

Observation 3: Scheduling grant on slots containing TRS will increase the code rate on those slots and will impact CQI reporting computation.

Observation 4: Whether we PDSCH is scheduled on slots containing TRS or not, will not impact PMI reporting.

Proposal 1: Do not schedule PDSCH grant on slots containing TRS for CSI reporting tests.

- Conclusions

This paper provides our views on PDSCH grant pattern in CSI reporting tests in RAN4. Following has been proposed.

Observation 1: It was agreed in Rel-15 not to schedule PDSCH on slots containing CSI-RS for CSI reporting tests as captured in [1].

Observation 2: TRS is also a type of CSI-RS.

Observation 3: Scheduling grant on slots containing TRS will increase the code rate on those slots and will impact CQI reporting computation.

Observation 4: Whether we PDSCH is scheduled on slots containing TRS or not, will not impact PMI reporting.

Proposal 1: Do not schedule PDSCH grant on slots containing TRS for CSI reporting tests.

# References

[1] R4-1814235, “Way forward on NR UE CSI requirements”, 3GPP TSG RAN WG4 Meeting #88-bis, Chengdu, China, Oct 8 - 12, 2018.