3GPP TSG-RAN WG4 Meeting #101-e                                         R4-2119515

Electronic Meeting, November 01-12, 2021

Source: 	Huawei, HiSilicon

Title: 	RF requirements for PC2 intra-band UL CA with UL MIMO

Agenda Item:	8.3.2.5

Document for:	Approval

# Introduction

CR for PC3 intra-band UL CA with UL MIMO has been agreed in last RAN4 meeting [1]. The remaining issue is the PC2 MPR requirement for intra-band CA with MIMO [2].

This contribution provides our consideration on completion of the remaining requirement.

# Discussion

The WF in the GTW meeting is copied as below:

GTW Agreement:

  - Additional delta MPR is needed for UL contiguous CA+MIMO with 2 PC3 PA+1LO compared to 1Tx PC2

    - Additional 0.5dB MPR is needed for outer allocation

    - FFS for inner PRB allocation

  - Re-check the agreement above after finalizing the work for 1CC with 2Tx UL-MIMO for PC2

Based on the discussion also considering the progress on TxD WI, the only remaining issue is whether the PC2 MPR requirement should be defined separately for different PA configurations. As we discussed in [3], it is proposed to specify same set MPR requirement without discriminating the 2Tx PA configurations. Some changes are needed to be reflected in the specification to enable PC2 intra-band CA with UL MIMO, i.e.

1. Update the MPR requirements for intra-band UL CA with 2Tx

2. Update the referred MPR table for intra-band UL CA with UL MIMO

In addition, considering intra-band contiguous CA with UL MIMO under suffix H agreed in last RAN4 meeting may need to be extended to other scenarios, e.g. intra-band non-contiguous CA with MIMO, some adjustments of the clauses with suffix H are provided in draft CR in [5]. The changes for PC2 intra-band UL CA with UL MIMO should also consider the possible adjustment.

Proposal 1: It is proposed to adopt the 2Tx PC2 intra-band contiguous CA MPR requirements for UL intra-band contiguous CA with UL MIMO.

Proposal 2: One set of MPR requirements for different PC2 CA with UL MIMO PA configurations.

A CR on PC2 intra-band UL CA with UL MIMO is provided in [5].

# Conclusion

This contribution provides consideration to complete the remaining requirements for PC2 intra-band UL CA with UL MIMO.

Proposal 1: It is proposed to adopt the 2Tx PC2 intra-band contiguous CA MPR requirements for UL intra-band contiguous CA with UL MIMO.

Proposal 2: One set of MPR requirements for different PC2 CA with UL MIMO PA configurations.

# References

[1] R4-2114953, CR on contiguous CA with UL MIMO for power class 3Huawei, HiSilicon

[2] R4-2114951, WF_CA_UL-MIMO, vivo

[3] R4-2119514, MPR for PC2 intra-band contiguious CA with 2Tx, Huawei, HiSilicon

[4] R4-2119516, draft CR for TS 38.101-1 correction of clauses of suffix H for CA with UL MIMO (R17), Huawei, HiSilicon

[5] R4-2119517, CR on contiguous CA with UL MIMO for power class 2, Huawei, HiSilicon