3GPP TSG-RAN WG4 Meeting #101-e                                         R4-2119529

Electronic Meeting, August 16-27, 2021

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.839: Principles for simultaneous Rx/Tx capability

Agenda Item:	7.39.2

Document for:	Approval

# Background

This contribution provides the TP for TR of simultaneous Rx/Tx capability based on [1-2].

# References

[1] R4-2114946, WF on Simultaneous Rx/Tx, Huawei, HiSilicon.

[2] R4-2107311, TR skeleton for TR38.839 Simultaneous RxTx band combinations

# Text Proposal

<Start of Text Poposal>

## 5.2	Rules to decide simultaneous Rx/Tx capability for a band combination

### 5.2.1	FR1+FR1 FDD-TDD band combination

Editor note: This band combination category also includes SUL band combination where SUL band is derived from FDD band.

For FR1+FR1 FDD-TDD band combination,

- Simultaneous Rx/Tx is the default capability in FDD-TDD FR1 band combinations

- For TDD-FDD combinations, the capability is mandatory without checking if there are no MSD issues and frequency range is not adjacent to each other for the band combinations

- For TDD-FDD combinations with REFSENS degradation due to self-interference, MSD caused by harmonics/IMD, etc. should be defined. For some band combinations that are close in frequency, e.g. bands n7 and n38, other methods to mitigate interference can be considered as well. Simultaneous Rx/Tx capability for this kind of special cases could be handled separately from those having MSD issues.

- For band combinations whose MSD is larger than a threshold (value FFS), further discuss whether simultaneous Rx/Tx can be changed to optional, otherwise, the simultaneous Rx/Tx capability is mandatory support.

### 5.2.2	FR1+FR1 TDD-TDD band combination

Editor note: This band combination category also includes SUL band combination where SUL band is derived from TDD band.

For FR1+FR1 TDD-TDD band combination, the simultaneous Rx/Tx capability should be studied case by case.

### 5.2.3	FR1+FR2 FDD-TDD band combination

For FR1+FR2 FDD-TDD band combination, the simultaneous Rx/Tx capability shall be mandatory if FR1 FDD band (<4GHz) is aggregated with FR2 TDD bands.

### 5.2.4	FR2+FR2 TDD-TDD band combination

For FR2+FR2 TDD-TDD band combination, the simultaneous Rx/Tx capability should be studied case by case.

<End of Text Poposal>