3GPP TSG-RAN WG4 Meeting #101-e                         	         R4-2119541

# Electronic meeting, 1st November – 12th November, 2021

Agenda item:	8.1.3.3

Source:	Huawei, HiSilicon

Title:	Initial results on NR UE FR1 MIMO OTA channel model validation

Document for:	Approval

# Introduction

For the purpose of lab alignment and requirements development, channel model validation has been discussed for several meetings for NR UE FR1 MIMO OTA. Reference validation targets were agreed in last RAN4 #100e meeting [3], with the refined time plan [4]. According to the agreed Way forward (cited as below for convenience), this document presents some initial analysis and measurement results to facilitate the meeting discussion.

Reference validation targets for FR1 [1]

- For CDL-C UMa and CDL-C UMi, reference channel emulation curves in R4-2115759 is endorsed;

- Companies are encouraged to bring proposals and analysis for pass limit of channel validation and target to conclude in Nov 2021 RAN4 meeting.

    - A check point for offline alignment among interested parties before November is encouraged

    - Given the reference emulation curves are based on the average approach from several companies, the pass/fail limit should consider different implementations.

gNB Beams Usage Criteria for Channel model validation [2]

- Keep both beam specific and beam simultaneous approach for validation procedure in the TR at this stage.

- Further down selection may be needed after we make decision on how to define the pass/fail limit of channel model validation.

# Discussion

Table 1: General parameters

| Channel model | FR1 CDL-C UMa |
| --- | --- |
| Polarization | X2V |
| Measurements | Doppler/Temporal correlation Spatial correlation |
| Frequencies | 2450MHz, 3600MHz, 4700MHz |
| Beams | Beam1, Beam2, Beam combined |

  - Power Delay Profile (PDP)

According to TR38.827, for PDP measurement, Vector Network Analyzer (VNA) transmits frequency sweeping signal of 200MHz bandwidth (as shown in below table from TR), which translated to 5ns time resolution. Such resolution is not able to distinguish the clusters with similar delay value e.g. two groups of cluster indicated with red and blue in below table cited from R4-2115759. In other words, the measurement setting of PDP is not consistent with the reference. Clarification is needed.

From TR 38.827 Table 7.4.1.1-1: VNA settings for PDP measurements

| Item | Unit | Value |
| --- | --- | --- |
| Center frequency | MHz | Downlink center frequency in Table 7.4.1-1 |
| Span | MHz | 200 |
| Number of traces |  | 1000 |
| Number of points |  | 1101 |
| Averaging |  | 1 |

From R4-2115759, Table 1. Reference PDP of CDL-C UMa with combined beams and ≤ 2.5 GHz

| Delay [ns] | Power [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | X2V | H11 | H21 | H12 | H22 |
| 0.0 | -25.9 | -27.6 | -24.9 | -25.0 | -27.7 |
| 76.6 | -2.5 | -2.5 | -2.5 | -2.8 | -2.9 |
| 79.4 | -13.0 | -13.8 | -12.5 | -12.7 | -14.0 |
| 81.0 | -5.0 | -6.1 | -4.3 | -4.5 | -6.3 |
| 85.0 | -6.2 | -6.2 | -5.9 | -6.6 | -6.9 |
| 232.4 | 0.0 | -0.1 | 0.0 | -0.3 | -0.4 |
| 235.4 | -2.7 | -3.5 | -2.0 | -2.3 | -4.0 |
| 239.4 | -3.9 | -4.0 | -3.7 | -4.3 | -4.6 |
| 240.3 | -inf | -inf | -39.7 | -inf | -inf |
| 289.6 | -25.8 | -26.2 | -25.5 | -25.8 | -26.6 |
| 299.8 | -36.2 | -36.9 | -35.9 | -36.0 | -37.0 |
| 448.4 | -26.3 | -26.3 | -26.1 | -26.7 | -26.8 |
| 477.5 | -25.8 | -26.2 | -25.5 | -25.9 | -26.6 |
| 1679.1 | -37.8 | -37.8 | -37.7 | -38.1 | -38.4 |

Observation 1: for PDP measurement, the given reference in R4-2115759 is not consistent with the measurement setting of 200MHz frequency sweeping signal (5ns time resolution).

Proposal 1: for PDP measurement, propose to clarify the inconsistence between given reference in R4-2115759 and the measurement setting of 200MHz frequency sweeping signal (5ns time resolution) in TR 38.827.

  - Doppler/Temporal correlation

## Measurement results from Huawei’s Lab

Figure 1a 2.45GHz Beam1 		     Figure 1b 2.45GHz Beam1		  Figure 1c 2.45GHz Beam combined

Figure 2a 3.6GHz Beam1 		      Figure 2b 3.6GHz Beam2

Figure 3a 4.7GHz Beam1 		      Figure 3b 4.7GHz Beam2

Table 2: Maximum deviation of above measured Doppler/Temporal correlation (CDL-C Uma)

| Frequency(MHz) | Beam1 | Beam2 | Beam Combined |
| --- | --- | --- | --- |
| 2450 | 0.01 | 0.02 | 0.05 |
| 3600 | 0.01 | 0.04 | NA |
| 4700 | 0.02 | 0.02 | NA |

  - Spatial correlation

## Measurement results from Huawei’s Lab

Figure 4a 2.45GHz Beam1 		     Figure 4b 2.45GHz Beam1		  Figure 4c 2.45GHz Beam combined

Table 3: Deviation of above measured spatial correlation (CDL-C Uma, 2450MHz)

| Angle | Beam1 | Beam2 | Beam Combined |
| --- | --- | --- | --- |
| 270 | 0.0000 | 0.0000 | 0.0000 |
| 263 | -0.0002 | 0.0010 | -0.0044 |
| 256 | 0.0000 | 0.0042 | 0.0133 |
| 249 | 0.0012 | 0.0069 | 0.0193 |
| 242 | 0.0025 | 0.0065 | 0.0241 |
| 234.9 | 0.0035 | 0.0024 | 0.0098 |
| 227.9 | 0.0048 | -0.0066 | 0.0118 |
| 220.9 | 0.0070 | -0.0198 | 0.0102 |
| 213.9 | 0.0098 | -0.0317 | 0.0003 |
| 206.9 | 0.0138 | -0.0329 | 0.0085 |
| 199.9 | 0.0230 | -0.0204 | 0.0236 |
| 192.9 | 0.0384 | -0.0059 | 0.0424 |
| 185.9 | 0.0564 | -0.0040 | 0.0750 |
| 144.9 | 0.0700 | 0.0031 | -0.0705 |
| 109.9 | -0.0205 | -0.0188 | -0.0054 |
| 74.8 | -0.0312 | -0.0506 | 0.0215 |
| 39.8 | -0.0516 | -0.0230 | 0.0106 |
| 4.7 | 0.0565 | -0.0376 | 0.1376 |
| 329.7 | 0.0157 | -0.0194 | 0.0278 |
| 294.6 | -0.0020 | -0.0061 | 0.0302 |

## To deduce the workload, it is proposed that

Proposal 2: select either beam specific or beam simultaneous approach for each measurement of Power Delay Profile (PDP), Temporal Correlation Function (TCF), Spatial Correlation Function (SCF) and V/H Ratio, i.e. only one approach for each measurement

Figure 4c 3.6GHz Beam combined

Table 3: Deviation of above measured spatial correlation (CDL-C Uma, 2450MHz)

| Angle | Beam Combined |
| --- | --- |
| 270 | 0.0000 |
| 265.2 | 0.0048 |
| 260.5 | -0.0036 |
| 255.7 | -0.0029 |
| 250.9 | 0.0066 |
| 246.1 | -0.0012 |
| 241.4 | -0.0140 |
| 236.6 | -0.0270 |
| 231.8 | -0.0265 |
| 227.1 | -0.0256 |
| 222.3 | -0.0210 |
| 217.5 | -0.0203 |
| 212.7 | -0.0225 |
| 208 | -0.0143 |
| 203.2 | -0.0022 |
| 198.4 | -0.0003 |
| 193.7 | 0.0072 |
| 188.9 | 0.0135 |
| 184.1 | 0.0073 |
| 156.1 | -0.0955 |
| 132.3 | 0.0451 |
| 108.4 | 0.0204 |
| 84.6 | 0.0548 |
| 60.7 | -0.0752 |
| 36.9 | 0.0224 |
| 13 | 0.1105 |
| 349.1 | -0.0714 |
| 325.3 | 0.0702 |
| 301.4 | 0.0026 |
| 277.6 | 0.0061 |

# Conclusions

This document presents some initial measurement results on NR UE FR1 MIMO OTA channel model validation, it is proposed to take these measurement results into consideration in the discussion of pass limit of channel validation. More details include:

Observation 1: for PDP measurement, the given reference in R4-2115759 is not consistent with the measurement setting of 200MHz frequency sweeping signal (5ns time resolution).

Proposal 1: for PDP measurement, propose to clarify the inconsistence between given reference in R4-2115759 and the measurement setting of 200MHz frequency sweeping signal (5ns time resolution) in TR 38.827.

Proposal 2: select either beam specific or beam simultaneous approach for each measurement of Power Delay Profile (PDP), Temporal Correlation Function (TCF), Spatial Correlation Function (SCF) and V/H Ratio, i.e. only one approach for each measurement

# Reference

- R4-2115756 WF on NR MIMO OTA, RAN4 100-e, vivo, CAICT

- R4-2108085 WF on NR MIMO OTA, RAN4 99-e, vivo, CAICT

- R4-2115759 Reference Channel Emulation Curves for Validation Purposes, RAN4 100-e, Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT

- R4-2115758 Time plan for FR1 lab alignment and requirement development, RAN4 100-e, CAICT, OPPO, vivo

- TR 38.827 V16.1.0