3GPP TSG-RAN WG4 Meeting #101-e  		                                  R4-2119553

Electronic Meeting, 1st-12th of November, 2021

Source:	THALES

Title:	NTN NR UE Technical Specification Discussion

Type:	discussion

Document for:	discussion

Agenda Item:	8.13.1.4

Release:	Rel-17

# Introduction

During RAN4#100-e, the following discussion took place in R4-2115784 (Email discussion summary for [100-e][312] NTN_Solutions_Part1):

Issue 4-3-1: Introduction of New Specific UE TS for UE NTN NR

- Moderator Note: The goal of the table below is to gather information. Any potential decision will be taken in RAN-P meeting, by proposing a WID update.

Issue 4-3-1: Introduction of New Specific UE TS for UE NTN NR

- Moderator Note: The goal of the table below is to gather information. Any potential decision will be taken in RAN-P meeting, by proposing a WID update.

|  | Pros | Cons |
| --- | --- | --- |
| Creating a new VSAT and handheld NTN UE specification | Huawei: Antenna gain will be assumed for VSAT, so a new UE specification can be created considering the different requirements between VSAT and traditional TN UE. Given we have to create a new UE specification, all the NTN UE requirements can be specified in one specification for tracking. RAN4 can avoid FR1, FR2 or partial FR2 discussion on how to organize UE specification. Ericsson: NTN UE requirements will be in the same specification, not spread between 2 TSs (depending on frequency range, type of UE, …). It’s still very easy to refer to 38.101-1 subclauses when the requirements are the same for NTN and TN. CATT: we prefer new specification, not only handheld UE but also VSAT can be included in such specification. ZTE: we also support to have new spec for NTN UE. For VSAT UE, this is not considered in the coexistence study, not sure whether this should also been captured. | Qualcomm: No need to create a new UE specification. As discuss in NTN UE RF (thread [314]), most of TN UE RF requirements can be reused for NTN UE. Even we have a new specification for NTN UE, we could not solve the Ka band issues since for FR1 we will specify conductive requirements but for Ka band radiated requirements will be defined. THALES: We currently do not perform any coexistence simulations for VSAT in FR1. We proposed this, but it has not been accepted by other companies. VSAT may refer in the future to both FR1 and FR2, and represents a different class of UE, and therefore it makes sense to introduce them together in a separate specification, starting from Rel-18. In Rel-17 we only have handheld devices. Please see the following agreements: - RAN4#98-e meeting, WF for NTN general part, R4-2103877: For RAN4 NTN coexistence studies in FR1, handheld devices could be prioritized (to be further discussed in the NTN coexistence analysis). - RAN4#98-e meeting, WF for NTN co-existence study, R4-2103878: NTN UE parameters: Handheld UEs for FR1. Propose to continue discussion in March 2022, when VSAT options (FR2 and FR1) work could continue. (see e.g. decision Topic#6) |
| The satellite handheld UE RF requirements can be specified in TS 38.101-1 together with other UE features | Qualcomm: Most of TN UE RF requirements can be used for NTN handheld UE RF. The framework is the same. THALES: Agee with Qualcomm. For handheld (NTN) UE the current requirements are not different from existent ones. Why we want to make a new specification for this? Please also note that we recommend not to impact any ACLR and ACS values of handheld UE for NTN. | Huawei: Ka band will be discussed after Rel-17, but neither TS 38.101-1 nor TS 38.101-2 is suitable to specify Ka band’s UE RF requirements. The requirements for VSAT is different from 38.101-1. A separate VSAT requirements can be foreseen. Ericsson: 38.101-1 is already too big document, very painful to navigate in it. Other type of NTN UEs won’t be specified in this TS 38.101-1, being possibly a source of confusion. |

Issue 4-3-1: Introduction of New Specific UE TS for UE NTN NR

- 2nd round candidate proposals:

- Moderator Note: Moderator proposes to continue discussion in RAN4#101-e, currently companies disagree if to introduce or not New Specific UE TS for UE NTN NR (see discussion in the Annex).

- Moderator Note: According to RAN-P guidance (Proposal 1 and proposal 2 in RP-211596), RAN4 work on FR2 postponed until March 2022. No discussion on RAN4 FR2 from now on till March 2022.

- Moderator Note: According to the previous RAN-P guidance and Chairman Guidance for Topic#6, UE VSAT discussion and related decision with respect to introduction of New Specific UE TS (for both FR1 and FR2 UE NTN NR) should probably continue in March 2022.

Issue 4-3-1: Introduction of New Specific UE TS for UE NTN NR

- 2nd round candidate proposals:

- Moderator Note: Moderator proposes to continue discussion in RAN4#101-e, currently companies disagree if to introduce or not New Specific UE TS for UE NTN NR (see discussion in the Annex).

- Moderator Note: According to RAN-P guidance (Proposal 1 and proposal 2 in RP-211596), RAN4 work on FR2 postponed until March 2022. No discussion on RAN4 FR2 from now on till March 2022.

- Moderator Note: According to the previous RAN-P guidance and Chairman Guidance for Topic#6, UE VSAT discussion and related decision with respect to introduction of New Specific UE TS (for both FR1 and FR2 UE NTN NR) should probably continue in March 2022.

Please also see moderator notes from R4-2115640 (Way Forward on NTN_solutions_Part1):

# NTN UE Technical Specification Discussion and Purpose

As already mentioned in RAN4#100-e, in Rel-17 there are (currently) no coexistence simulations for VSAT in FR1. The coexistence analysis is performed only for handheld FR1 NTN UE, and not for VSAT UE, ESIM, or for FR2 NTN UE.

Moreover, VSAT may refer in the future to both FR1 and FR2, and represents a different class of UE, and therefore it makes sense to introduce them together in a separate specification, starting from e.g. Rel-18.

In Rel-17 we only have NTN handheld devices to be considered. Please see the following agreements:

- RAN4#98-e meeting, WF for NTN general part, R4-2103877: For RAN4 NTN coexistence studies in FR1, handheld devices could be prioritized (to be further discussed in the NTN coexistence analysis).

- RAN4#98-e meeting, WF for NTN co-existence study, R4-2103878: NTN UE parameters: Handheld UEs for FR1.

In this contribution it is therefore proposed to continue discussion in March 2022, when VSAT options (FR2 and FR1) work could continue. For information, please see e.g. decision Topic#6, from R4-2115640:

- “RAN4 work on FR2 postponed until March 2022. No discussion on RAN4 FR2 from now on till March 2022.“

Proposal 1: UE VSAT discussion and related decision with respect to introduction of New Specific UE TS (for both FR1 and FR2 UE NTN NR) should be deferred in March 2022.

For handheld (NTN) UE the current requirements are not different from existent ones of TN UE. It does not make sense to consider a new specification for NTN UE if the requirements are the same as for TN UE.

Proposal 2: RAN4 should not consider a new specification for NTN UE, since most requirements are the same as for TN UE.

Please also note that we recommend not to impact any ACLR and ACS values of handheld UE for NTN. The same TN UE can be potentially used as NTN UE if no coverage (or poor coverage) from TN.

Proposal 3: RAN4 should use for NTN UE the same ACLR and ACS values as for handheld TN UE.

Observation 1: The NTN UE FR1 requirements can be considered in separate sections of TN UE FR1 specification (i.e. 38.101-1) and treated similarly as for other use cases, e.g. V2X. For example, V2X does not require any separate specification (different from 38.101-1), and is not clear why handheld NTN UE FR1 should have different treatment.

Proposal 4: The NTN UE FR1 RF requirements should be considered in TS 38.101-1.

# Conclusions

Proposal 1: UE VSAT discussion and related decision with respect to introduction of New Specific UE TS (for both FR1 and FR2 UE NTN NR) should be deferred in March 2022.

Proposal 2: RAN4 should not consider a new specification for NTN UE, since most requirements are the same as for TN UE.

Proposal 3: RAN4 should use for NTN UE the same ACLR and ACS values as for handheld TN UE.

Observation 1: The NTN UE FR1 requirements can be considered in separate sections of TN UE FR1 specification (i.e. 38.101-1) and treated similarly as for other use cases, e.g. V2X. For example, V2X does not require any separate specification (different from 38.101-1), and is not clear why handheld NTN UE FR1 should have different treatment.

Proposal 4: The NTN UE FR1 RF requirements should be considered in TS 38.101-1.

# References

- R4-2115784, Email discussion summary for [100-e][312] NTN_Solutions_Part1, THALES.

- R4-2115640, Way Forward on NTN_solutions_Part1, THALES.

END