3GPP TSG-RAN WG4 Meeting #101-e	R4- 2119561

Electronic Meeting, 01 – 12 November, 2021

Agenda item:	9.20.3.1.5

Source: 	Qualcomm Incorporated

Title: 	UE complexity reduction impact on measurement procedures

Document for:	Discussion

# Introduction

- Reduced minimum number of Rx branches:

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

  - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

  - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

In RAN#90e meeting, a Rel-17 work item for support of reduced capability NR devices was approved and the WID was further updated in RAN#92e [1]. One of the key objectives of the work-item is to provide support for UE complexity reduction features. In the previous RAN4 meetings, it was agreed that reduced minimum number of Rx branches is one such UE complexity reduction feature which is likely to impact RRM requirements such as RLM/BFD evaluation periods, cell search and measurement delays and accuracies etc.

Simulation assumptions to evaluate cell detection and PBCH detection performance for RedCap UEs with 1 Rx were agreed in RAN4#100e [2, 3]. In this contribution, we provide the simulation results based on the agreed simulation assumptions and present our views on the impact of 1Rx on cell detection and PBCH detection periods.

# Simulation results for cell detection

In this section, we present the simulation results for cell detection performance as per the simulation assumptions agreed in [2].

Simulation settings

| Cell ID | PSS ID | SSS ID |
| --- | --- | --- |
| 337 | 1 | 112 |
| 673 | 1 | 224 |
| 674 | 2 | 224 |

- SCS – 30kHz, Carrier Frequency – 4 GHz, Bandwidth – 20 MHz

- Serving cell ID (Cell 1) – 337

- Target cell IDs (Cell 2) – 673, 674

CDF curves for PSS/SSS detection

Observation 1: For the two desired cell-IDs considered, the cell detection delays are the same.

Observation 2: Number of attempts needed for 90% detection rate

- Synchronous mode – 2

- Asynchronous mode – 3

Proposal 1: At least for FR1, increase the cell detection period by three times for 1 Rx RedCap UEs as described below:

Table 9.2.5.1-1: Time period for PSS/SSS detection for 1Rx RedCap UEs, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 15 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 15 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(15 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

# Simulation results for PBCH acquisition time

In this section, we present the simulation results for PBCH acquisition time as per the simulation assumptions agreed in [3]. The following cdf curves, show the number of SS-blocks required for 99% to successfully decode the PBCH.

Simulation settings

- SCS – 15kHz, Carrier Frequency – 4 GHz

Figure 1: Number of attempts needed for 99% PBCH decoding rate for 1 Rx

The above CDF plot shows that around 10 attempts are needed at -8 db SNR, 8 attempts at -6 db SNR and around 4 attempts at -3 db SNR. We think specifying the requirements at -8 db SNR will cause long delays, so we can focus on defining PBCH decoding requirements at -6db or -3db SNR.

Observation 3: At -8db SNR, 1 Rx UE needs around 10 attempts to achieve 99% PBCH decoding rate.

Observation 4: At -6db SNR, 1 Rx UE needs around 8 attempts to achieve 99% PBCH decoding rate.

Observation 5: At -3db SNR, 1 Rx UE needs around 4 attempts to achieve 99% PBCH decoding rate.

Proposal 2: Propose RAN4 to consider the following options for the number of attempts to achieve 99% PBCH decoding rate for 1 Rx UE

- 8 attempts at -6db SNR

- 4 attempts at -3db SNR

# Conclusion

Observation 1: For the two desired cell-IDs considered, the cell detection delays are the same.

Observation 2: Number of attempts needed for 90% detection rate

- Synchronous mode – 2

- Asynchronous mode – 3

Proposal 1: At least for FR1, increase the cell detection period by three times for 1 Rx RedCap UEs as described below:

Table 9.2.5.1-1: Time period for PSS/SSS detection for 1Rx RedCap UEs, (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 15 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 15 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(15 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Observation 3: At -8db SNR, 1 Rx UE needs around 10 attempts to achieve 99% PBCH decoding rate.

Observation 4: At -6db SNR, 1 Rx UE needs around 8 attempts to achieve 99% PBCH decoding rate.

Observation 5: At -3db SNR, 1 Rx UE needs around 4 attempts to achieve 99% PBCH decoding rate.

Proposal 2: Propose RAN4 to consider the following options for the number of attempts to achieve 99% PBCH decoding rate for 1 Rx UE

- 8 attempts at -6db SNR

- 4 attempts at -3db SNR

# References

[1] RP-211574, “Revised WID on support of reduced capability NR devices”, Ericsson, RAN#92e

[2] RP-2115359, “Simulation assumptions for RedCap cell detection performance”, Vivo, RAN4#100e

[3] RP-2115363, “Simulation assumptions for RedCap PBCH detection”, Vivo, RAN4#100e