3GPP TSG RAN WG4 Meeting #101-e	R4-2119595

E-meeting, November 1 – November 12, 2021

Agenda Item:	9.9.2.4

Source:	Guangdong Communications and Networks Institute

Title:	Potential High speed train scenarios in FR2

Document for:	Discussion

Introduction

The recent development of high-speed trains (HSTs) as an emerging high mobility transportation system, and the growing demands of broadband services for HST users, introduce new challenges to wireless communication systems for HSTs. Accurate and efficient channel models considering both large-scale and non-stationary small-scale fading characteristics are crucial for the design, performance evaluation, and parameter optimization of HST wireless communication systems. However, the characteristics of the underlying HST channels have not yet been sufficiently investigated.

In the chairman’s note of RAN4#100, “9.9 NR support for high speed train scenario in FR2” had discussed the necessary of channel models for HST scenario. In this proposal we discuss the potential scenarios of HST in FR2.

Discussion

HST environments can be roughly classified into the following 6 scenarios: open space, viaduct, cutting, hilly terrain, tunnels, and stations[1].

- In the open space scenario , also called plain scenario [2], the Rx is moving at a very high speed in a rural area where the BS antenna is much higher than the surroundings. This environment focuses on large cells and continuous coverage where the link between the fixed Tx and moving Rx normally has a dominant line-of-sight (LoS) component. Based on the geographic nature and the distribution/height of the surrounding scatters, the open scenarios can be further classified into rural [3], urban, and suburban scenarios as illustrated in Fig. 1.

Fig.1. Classification of HST scenarios.

- The viaduct scenario is very common for HSTs. The main purpose of viaducts is to ensure the smoothness of the rail and the high speed of the train. In this scenario, the radio reflection, scattering, and diffraction caused by nearby scatters, e.g., trees and buildings, can also be reduced significantly.

- The cutting scenario is another common scenario for HST wireless communications. It represents an environment where the HST passes a U-shaped geographical cut surface between the hills. The cutting is widely used for HST construction to ensure the smoothness of the rail and help to achieve a high speed of the train when passing through hills. The propagation of radio wave forms in this scenario is significantly affected by the steep walls on both sides.

- In the hilly terrain scenario, the surrounding environment is densely scattered with objects distributed irregularly and non-uniformly. With high-altitude transmit antennas and low-altitude obstacles, the LoS component is observable and can be detected along the entire railway.

- The tunnel scenario represents an environment where HST passes through tunnels [81], [82] with different lengths ranging from hundreds of meters to several kilometers.

- The stations scenario represents the railway facility where HSTs stop regularly to load/unload passengers. HST stations can be classified according to their size or architecture.

The aforementioned scenarios are the most encountered ones in HST environments. The frequent and fast transition between these scenarios can degrade the quality of the communication link and makes signal prediction quite challenging.

Table 1 HST Scenarios

Proposal : HST environments should be clearly classified.

Conclusions

In summary, we discuss on roughly classification of HST scenarios, and made the proposal below:

Proposal : HST environments should be clearly classified.

- References

[1]Wang, Cheng Xiang , et al. "Channel Measurements and Models for High-Speed Train Communication Systems: A Survey." IEEE Communications Surveys & Tutorials 18.2(2017):974-987.

[2] M. Zhao et al., “Analysis and modeling for train-ground wireless wideband channel of LTE on high-speed  railway,”  in  Proc.  IEEE Veh. Technol. Conf. (VTC’13-Spring), Dresden, Germany, Jun. 2013, pp. 1–5.

- [3]K. Pekka, “WINNER II channel models—Part II radio channel mea- surement and analysis results,” IST-4-027756, WINNER II D1.1.2, v1.0, Sep. 2007.

[4]RAN1#104-bis-e Chairman's Notes.

[5]3GPP TS 22.261, “Service requirements for the 5G system”.

[6]R1-2104258, “Application scenarios of IoT in NTN”, Huawei, HiSilicon

[7]R1-2105404,“Link budget evaluations and deployment for NB-IoT/eMTC over NTN”