3GPP TSG-RAN WG4 Meeting #101-e                                                         R4-2119756

# xElectronic Meeting, Nov. 1-12, 2021

Source:	ZTE Corporation,CMCC

Title:	TP for TR38.717-03-01_CA_n28A-n40A-n79A

Agenda Item:	7.8.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to introduce 3DL/1UL CA_n28A-n40A-n79A in TR38.717-03-01.

- Reference

- TR38.717-03-01,Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.6.0

- Text Proposal

----- Start of TP -----

## 6.X	CA_n28-n40-n79

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28A-n40A-n79A3 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n40A-n79A | - | n28 | 5 | 10 | 15 | 20 |  | 30 |  |  |  |  |  |  |  | 0 |
|  |  | n40 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 6.X.3	Co-existence studies

The coexistence studies of harmonic interference have been captured in the constituent fallback modes in TR 38.716-02-00, where no harmonic issues for the 1UL/3DL NR CA CA_n28A-n40A-n79A.

    - 6.X.4	∆TIB and ∆RIB values

For CA_n28A-n40A-n79A,  TIB,c and RIB,c values are given in table 6.x.4.1 and 6.x.4.2

6.x.4.1 ΔTIB,c

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n28-n40-n79 | n28 | 0.5 |
|  | n40 | 0.3 |
|  | n79 | 0.8 |

6.x.4.2 ΔRIB,c

| Inter-band CA combination | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n28-n40-n79 | n28 | 0.2 |
|  | n40 | 0 |
|  | n79 | 0.5 |

### 6.X.5	MSD

No additional MSD is needed for 1UL/3DL NR CA CA_n28A-n40A-n79A since there are no additional harmonic issues. Also there are no cross band isolation MSD issues.

----- End of TP -----