3GPP TSG-RAN WG4 Meeting#101     	R4-2119824

E-meeting, 1st – 12th November 2021

Title:	TP to TR 36.717-01-01 Addition of CA_n1(2A)

Source:	Nokia

Agenda item:	7.6.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 36.717-01-01 for addition of CA_n1(2A).

************************************* Start of TP*****************************************

## 6.x	CA_2DL_n1(2A)_1UL_n1A

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: Supported bandwidth combinations

| NR CA Configuration | Uplink Configurations | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |

### 6.x.2	Co-existence studies

### There are no additional co-existence issues for this combination since it is single UL.

### 6.x.3	REFSENS

This combination requires MSD.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1(2A) | 15/15 | 5MHz + 5MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |

************************************* End of TP*****************************************