3GPP TSG-RAN WG4 Meeting # 101-e 													R4-2119918

Electronic Meeting, 1 – 12 November 2021

Agenda item:			7.37, 7.38, 7.39, 10.7, 10.9

Source:	Moderator (Huawei)

Title:	Email discussion summary for [101-e][118] LTE_NR_Other_WI

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion (e.g. list of treated agenda items) and provide some guidelines for email discussion if necessary.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

In this email thread, the following agenda items are discussed.

7.37 Additional NR bands for UL-MIMO
7.38 DL interruption for band combo dynamic Tx Switching
7.39 Simultaneous Rx/Tx band combination
10.7 Additional LTE bands for UE Cat M1/2, NB1/2
10.9 R17 Additional enhancements for NB-IoT and LTE-MTC

# Topic #1: Additional NR bands for UL-MIMO

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117308 | Verizon, Qualcomm | This contribution provides our request to the RAN4 Rel-17 basket work item [1] to support NR band n77/n78 UL MIMO for PC1.5 HPUE. |
| R4-2119530 | Huawei, HiSilicon | To introduce PC2 UL MIMO for n40. |
| R4-2119531 | Huawei, HiSilicon | Add UL MIMO for band n24 and n99 PC3. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Open issues and candidate options before e-meeting:

Issue 1-1: Add band n77/n78 PC1.5 for UL MIMO

Issue 1-2: WID revision

## Companies views’ collection for 1st round

### Open issues

Issue 1-1: Add band n77/n78 PC1.5 for UL MIMO

- Proposals

  - Option 1: Endorse the request

  - Option 2: Others (please specify)

- Recommended WF

  - Discuss the above options. If option 1 is agreed, prepare the corresponding (draft) CR.

| Company | Comments |
| --- | --- |
| AT&T | Option 1. |
| Skyworks | PC1.5 always had the intention to support UL MIMO. For MPR it would be sufficient that the 2Tx CP-OFDM values are pointed to. |

Issue 1-2: WID Revision

Add UL MIMO for band n24 and n99 PC3 as in R4-2119531.

- Proposals

  - Option 1: Endorse the revision

  - Option 2: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2119530 Introduce UL MIMO configurations for band n40 | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic# 1 | The request in R4-2117308 can be endorsed, the revised WID of R4-2119531 can be endorsed and the draft CR of R4-2119530 can be endorsed. Tentative agreements: Candidate options: Recommendations for 2nd round: The topic is closed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Downlink interruption for band combinations to conduct dynamic Tx Switching

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2118322 | China Telecom | Draft CR to 38.101-1 Correction on the DL interruption clarification table |

## Companies views’ collection for 1st round

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2118322 | Apple: CR updates DL interruption as shown in the image below (highlighted Line) The highlighted combination seems to have a typo and should be CA_n8-n41-79. Would it be possible to correct the typo with this CR? Is the change to ‘No’ still intended for CA_n8-n41-79? To our understanding if a higher order combination has ‘No’ for DL interruption then all lower order combinations should as well. This is not true for CA_n8-n39-n41 as ‘No’ is not specified for CA_n8-n39. Please consider explicitly naming the lower order combinations for CA_n8-n39-n41 instead of general ‘No’. |
|  | Company B |

## Summary for 1st round

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2118322 | to be revised |

## Discussion on 2nd round (if applicable)

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2118322 | China Telecom: The CR is revised according to comments from Apple and uploaded in draft folder [CR revision for DL interruption] of this thread. The revision include: typo for CA_n8-n41-n79 is corrected, for CA_n8-n39-n41, lower order combos except of CA_n8-n39 are explicitly indicated with mandatory no DL interruption. However, this CR is targeted for spec implementation correction, not intended to touch any issue for specific combo, which has been approved in the previous meetings. As we know the CA_n8-n39-n41 is requested by CMCC, maybe they can give more clarification? CMCC: CA_n8-n39-n41 was requested by CMCC and TP was approved in R4-2103223 in RAN4#98e. During the discussion, no company raised concern on “No interruption” for CA_n8-n39. So the TP was approved. If “NO interruption” for CA_n8-n39 was missing from the current spec, we should fix that with a new CR for inter-band with 2 bands table. We should follow previous agreement on CA_n8-n39-n41. We support to approve this TP. Apple: Thank you for the revision. It looks good. And also thanks to CMCC for the clarification on CA_n8-n39. |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Draft CR R4-2119949 (revised from R4-2118322) is agreeable.

# Topic #3: Simultaneous Rx/Tx band combination

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117665 | SoftBank Corp. | Observation: Some band combinations have already specified as the simultaneous Rx/Tx capability is mandatory support. The maximum MSD value in those band combinations is 29.8(2 antenna ports) / 32.5(4 antenna ports) dBm. Proposal: The threshold should be higher than 29.8(2 antenna ports) / 32.5(4 antenna ports) dBm. |
| R4-2118881 | OPPO | Observation 1: RAN4 has used the 2nd and 3rd order IMD interference as the criteria for single UL transmission which is 14dB MSD. Proposal 1: 14dB MSD is taken as the criteria for optional simultaneous RxTx capability. Proposal 2: NW controlled approached can be further considered if single MSD criteria is difficult to be aligned. |
| R4-2119215 | ShenZhen Zhongxing Shitong | Observation 1: For a FDD-TDD inter-band NR CA band combination, inter-band NR CA operation can not workable if simultaneous Rx/Tx operation is not supported. Observation 2: Due to actual MSD can be 20 dB or more better than the MSD in the specs, actual MSD could be smaller than the threshold in case of the MSD defined in the spec larger than the MSD threshold. Observation 3: The performance would be affected if changing the simultaneous Rx/Tx capability from mandatory to optional. Proposal: The threshold value should be higher than 32.5dBm. |
| R4-2119528 | Huawei, HiSilicon | TR 38.839 V0.1.0 |
| R4-2119529 | Huawei, HiSilicon | TP for TR 38.839: Principles for simultaneous Rx/Tx capability |
| R4-2117681 | MediaTek Beijing Inc. | Proposal: Add NOTE1 in Table 5.2A.2-1 (Inter-band CA operating bands in FR2) of TS38.101-2 as below: NOTE 1: For UEs supporting inter-band CA operation in FR2, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in FR2 in the current version of this specification. |
| R4-2118179 | ZTE Corporation | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA A note is added in table 5.2A.2-1 to indicate the information of simultaneous Rx/Tx capability for the existing FR2 band combinations |
| R4-2118180 | ZTE Corporation | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA A note is added in table 5.2A.2-1 to indicate the information of simultaneous Rx/Tx capability for the existing FR2 band combinations |
| R4-2118887 | OPPO | Observation 1: Simultaneous RxTx has two different meaning, one is about the UE capability, and the other is about the requirements whether RAN4 spec supports the simultaneous RxTx operation. Observation 2: It needs to clear whether FR2 combinations in RAN4 support simultaneous RxTx implementation or not. Proposal 1: It is proposed to confirm that support for simultaneous RxTx should be indicated when band combination is introduced. Proposal 2: It is proposed to only define FR2 band combinations based on non-simultaneous RxTx, and postpone the simultaneous RxTx operation to Rel-18 if there is interest. |
| R4-2119519 | Huawei, HiSilicon | Observation 1: There is a dedicated WI on simultaneous Rx/Tx capability for band combinations, but the FR2 discussion in last meeting is not under the right agenda. Proposal 1: The simultaneous Rx/Tx related discussion should be under the dedicated WI agenda, otherwise people focusing on the discussion may miss the sporadic discussion in other places. Observation 2: A band combination which cannot mandatorily support simultaneous Rx/Tx capability does not mean it has to not support simultaneous Rx/Tx capability, and it depends on UE implementation capability. Proposal 2: More analysis is needed for the case by case study for the FR2 band combinations on the simultaneous Rx/Tx capability. |
| R4-2117991 | Apple | Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification Add a note in Table 7.3A.2.3-1 and Table 7.3A.3.3-1 to clarify that the inter-band DL CA REFSENS and EIS spherical coverage requirement is only applicable for non-simultaneous Tx/Rx between all carriers. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1 Simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination

Sub-topic description: According to the agreed WF, two options are going to be explored for the feasibility study.

Open issues and candidate options before e-meeting:

Issue 3-1-1: MSD threshold

### Sub-topic 3-2 Simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD band combination

Sub-topic description: According to the agreed WF, two options are going to be explored for the feasibility study.

Open issues and candidate options before e-meeting:

Issue 3-2-1: Confirmation of Simultaneous Rx/Tx Support for FR2 band combinations

## Companies views’ collection for 1st round

### Open issues

Issue 3-1-1: MSD threshold

For band combinations whose MSD is larger than a threshold (value FFS), further discuss whether simultaneous Rx/Tx can be changed to optional, otherwise, the simultaneous Rx/Tx capability is mandatory support.

- Proposals

  - Option 1: The threshold should be higher than 29.8(2 antenna ports) / 32.5(4 antenna ports)

  - Option 2: 14dB MSD is taken as the criteria for optional simultaneous RxTx capability

  - Option 3: NW controlled approached can be further considered if single MSD criteria is difficult to be aligned

  - Option 4: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | Option 2 or 3. Option 2 is based on the observation that RAN4 has used the 2nd and 3rd order IMD interference as the criteria for single UL transmission which is 14dB MSD in Rel-15. This can be reused here to decide the threshold of optional simultaneousRxTx capability. If Option 2 is not agreeable, Option 3 is also acceptable, i.e. define two options 14dB and 30dB thresholds and leave it to NW decide which MSD threshold is configured. Real MSD below this NW configured threshold UE shall support simultaneousRxTx, otherwise optionally support. |
| SoftBank | Support Option 1considering the current specs. |
| MTK | Option 2. If option 2 is not agreeable, then option 4 is preferred. Mandatory simultaneous RX/TX shall be specified case by case manner. |
| ZTE | Option 1. Similar view with softbank. The performance would be affected if changing the simultaneous Rx/Tx capability from mandatory to optional. It seems 14dB comes from the average value from all the existing MSD values, but it may unfair for all the combs. |
| CHTTL | Support Option 1. share the view as Softbank. |
| Qualcomm | For clarification, how would Option 3 work? in our understanding, we are discussing whether to mandate a feature or not, how could this be network controlled? We believe there should be some system level consideration for this, for example beyond what level of MSD the feature is not really usable in practice. The argument that we have something in the current spec and we should this as a threshold (whether it is 14dB or 29dB) does not make much sense to us as it is not related to anything practical. If the MSD is very high, there doesn’t seem to be much point in mandating the feature. Concrete input from infra vendors or operators with some analysis showing network benefits depending on MSD would be very useful. In conclusion, neither options is useful. It should be Option 4 with a decision based on some system level data. |
| Samsung | Option4. I wondering if it very necessary to define a MSD threshold for the time being? Considering together with “Low MSD” discussion, actual deployment MSD could be better. If the threshold works and the actual scenario MSD of a combo is very large, then it can not support simultanous Rx/Tx, but how it works? (as ZTE mentioned in their contribution, inter-band NR CA can not workable if simultaneous Rx/Tx operation is not supported) I suppose the combo should not be deployed in this case rather than change simultaneous Rx/Tx to optional. Also very interested in the NW controlled approach mentioned in Option3. Does it mean to explore the actual MSD when the UE is working? If it could works, as aforementioned, the band combo should not be deployed if actual MSD exceeds some threshold rather than change simultaneous Rx/Tx to optional. |
| Huawei | Option 1 is ok for us. We think that the threshold decision should be left to operators. Even with large MSD, operators still have the choice to decide whether to schedule UL configurations based on the deployment consideration. |
| Skyworks | Option 1 would make almost any combination mandatory Rx/Tx we do not think it is agreeable as a threshold. Also there different 30dB MSD. Some are related to IMDs for example MB/MB quadplexers where the filter assumption are already tight where some other that are harmonic related have a lot of variation in filter/diplexer…and even which harmonic/victim bands are optimized, also receive harmonic response…. So threshold may depend on the type of combinations, harmonic/IMD order. |
| Apple | Option 2 would be preferred to have a clear increase in overall throughput and coverage when simultaneous Rx/Tx is mandatory. |

Issue 3-2-1: Confirmation of Simultaneous Rx/Tx Support for FR2 band combinations

Simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD band combination should be studied case by case.

- Proposals

  - Option 1: It is proposed to only define FR2 band combinations based on non-simultaneous RxTx, and postpone the simultaneous RxTx operation to Rel-18 if there is interest.

  - Option 2: More analysis is needed for the case by case study for the FR2 band combinations on the simultaneous Rx/Tx capability.

  - Option 3: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| OPPO | Option 1. Up to now there is no study of simultaneous RxTx for FR2, and it is expected there might be complex and lengthy discussion and cannot be closed in Rel-17 leftover meetings. Leave to Rel-18 for a thorough study is better. |
| Qualcomm | Option 1 |
| ZTE | Option 1. RAN4 have discussed in the other thread in last meeting, and the common understaind was that it was the current R17 status that no studies for the simultaneous RxTx operation. It would be discussed in future if there are request. |
| MediaTek | Option 1. |
| Huawei | Option 2. According to the principle we discussed in previous RAN4 meeting, whether a band combination for FR2 can mandatorily support simultaneous Rx/Tx should be studied case by case, that is the basic rule agreed. In Rel-17, if there are no specific values of MSD for a band combination, it means that either UE support simultaneous Rx/Tx without Rx degradation or UE can report that it does not support simultaneous Rx/Tx via capability reporting. The current signaling mechanism works. Without analysis is not a reason that all the band combinations for FR2 must not support simultaneous Rx/Tx operation. |
| Apple | Option 1 |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2119529 TP for TR 38.839: Principles for simultaneous Rx/Tx capability | Company A |
|  | Company B |
|  | Qualcomm: This TP depends on issue 3-2-1. Can revisit after first round discussion. ZTE: The agreements for FR1+FR2 TDD-TDD are missing in the TR, new sub-clause may be needed. Huawei: the TP is based on previous agreed WFs for all the cases. For issue 3-2-1, so far the spec does not mandate the UE support simultaneous Rx/Tx capability, which is optional. Whether support simultaneous Rx/Tx can still be reported by UE via capability. |
| R4-2118179 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | Company A |
|  | Company B |
|  | Qualcomm: This TP depends on issue 3-2-1. Can revisit after first round discussion. MediaTek: We support this TP. Huawei: depends on 1st round discussion for issue 3-2-1. Similar to FR1, if some band combinations are identified not supporting simultaneous Rx/Tx, a note is used for such indication, the current wording is not well reflect the principles we discussed in the WI. |
| R4-2118180 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | Moderator: Should this CR be Cat A? |
|  | ZTE: For the Category in the CR cover, due to the corrections in 8180 (R17) and 8179 (R16) are not exactly the same, i.e. more corrections in 8180 (R17) for additional two combs which are not existed in the 8179 (R16), so Cat F was used in 8180, just like other companies’ CR did in last meeting. |
|  | MediaTek: We basically support this TP. However, third band combination should be “CA_n257-n259”. Could ZTE double-check whether my understanding is correct or not? ZTE Reply: Thanks MediaTek for the carefully check. There was copy&paster mistakes for the combinations, sorry for the mistake. After checking the spec, the first band combination should be “CA_n257-n259”. we can revised it. Thanks. Huawei: depends on 1st round discussion for issue 3-2-1. |
| R4-2117991 Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | Company A |
|  | Company B |
|  | Qualcomm: While we agree with the contents, this CR is not necessary if 8179 or similar gets agreed. ZTE: It was agreed in R4-2114959, where Method 1 is agreeable, i.e.: - Method 1: Note for the combs (i.e. a note to be added in Table 5.2A.2-1: Inter-band CA operating bands in FR2) So the note should be added in Table 5.2A.2-1. MediaTek: We support this TP. Huawei: also related to the discussion for the CR in 8179. Apple: We are fine to note this draft CR if 8179 can be endorsed. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 3-1 & 3-2 | Issue 3-1-1: MSD threshold The views are diverged between network operators and UE vendors. Issue 3-2-1: Confirmation of Simultaneous Rx/Tx Support for FR2 band combinations Five companies support option 1 while one company supports option 2. Tentative agreements: Candidate options: Recommendations for 2nd round: Assign a WF to the rapporteur company to drive the progress |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2119529 | return to |
| R4-2118179 | return to |
| R4-2118180 | revised |
| R4-2117991 | return to |

## Discussion on 2nd round (if applicable)

WF on simultaneous Rx/Tx capability

| Company | Comments |
| --- | --- |
| Skyworks | Issue 3-1-1: As a prime input we do not support option 1 and in absence of NW help, option 2 is our preference but as threshold to discuss if simultaneous Tx/Rx is optionally supported as it really depends on the type of MSD. Independently we think it would be worth discussing NW help at least in R18 |
| MTK | Issue 3-1-1: There may be NBC issue for the band combination if it was specified without simultaneous RX/TX but required to support the feature at a later release. There would be UE implementation impact that needs further study. |

| CR/TP number | Comments collection |
| --- | --- |
| R4-2119529 TP for TR 38.839: Principles for simultaneous Rx/Tx capability | Huawei: this TP just captured the previous agreement in WF R4-2114946, R4-2107841, R4-2105377. ZTE: As we commented in the 1st round, the agreements for FR1+FR2 TDD-TDD are missing in the TR, new sub-clause may be needed, i.e. (in R4-2114946) Proposed WF Simultaneous Rx/Tx capability is mandatory for FR1+FR2 TDD-TDD band combination with FR1 bands up to 5GHz and FR2 bands above 24GHz, whether the FR1 bands can be extended to 7.125GHz is FFS until there are such FR1+FR2 band combinations available in RAN4. |
| R4-2118179 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE: Reply huawei’ comments in 1st round. First, 8179 is for Rel-16 spec, and only CA_n260-n261 was incorporated. In terms of the requirements defined in the Rel-16 spec, there were no requirements defined for CA_n260-n261 supporting simultaneous Rx/Tx. Second, it was agreed in WF R4-2114946 that Simultaneous Rx/Tx capability for FR2+FR2 TDD-TDD band combination should be studied case by case. So, for CA_n260-n261 in Rel-16 spec, there is no choise except supporting non-Simultaneous Rx/Tx, since it is impossible to add new requirements to Rel-16 spec to support simultaneous Rx/Tx. If supporting simultaneous Rx/Tx, new requirements (if any) should be added in later release. Last, together with R4-2118180 below, which is also for anther two FR2 band combination i.e. CA_n257-n259 and CA_n258-n260. It was agreed in last WF R4-2114959, which is: - Agree to mark current CA configurations as no support for SimultaneousRxTx operation. - Note for the combs (i.e. a note to be added in Table 5.2A.2-1: Inter-band CA operating bands in FR2) to implement it in the TS38.101-2. Therefore, “Option 1: It is proposed to only define FR2 band combinations based on non-simultaneous RxTx, and postpone the simultaneous RxTx operation to Rel-18 if there is interest.” is aligned with the current agreements in WF R4-2114959. Also the corrections in 8179 and 8180 are aligned with these agreements. Hope it is clarified. Huawei: Though the CR is for Rel-16, the proposed changes should be aligned with the principle agreed under the dedicated WI, as the principles are general for the band combinations. For FR2 band combinations, yes, those should be studied case by case. In our understanding, it is not mandatory for the band combination to support simultaneous Rx/Tx if there is no mandatory indication in the spec. UE just uses the existing signaling mechanism, without capability reported, NW will not force UE to mandatorily support simultaneous Rx/Tx operation. Considering the practice, a note could be considered for some band combinations which cannot support the capability, but the wording should be revised. Similar to FR1, just clarify the applicable minimum requirements under non-simultaneous Rx/Tx operation. ZTE: The discussion for FR2 TDD-TDD are similar with FR1 TDD-TDD, i.e. whether or not supporting simultaneous Rx/Tx. For FR1 TDD-TDD combs, several combs support mandatory simultaneous Rx/Tx, and some of others do not support mandatory simultaneous Rx/Tx (see the notes in Table 5.2A.2.1-1 in TS38.101-1), which depends on operator’s demands. I agree with ‘ a note could be considered for some band combinations which cannot support the capability’, as i said above, it depends on operator’s demands, and i have already discussed these combs with related operator, they are ok to mark them as ‘..non-simultaneous Rx/Tx’ in current version of this specification. We are not against to support simultaneous Rx/Tx for these combs in the future. Also we have discussed the wordings in last meeting, where ‘in current version of this specification’ was proposed by operators to solve their concern. The revision of R4-2118179 is available in the folder. https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_101-e/Inbox/Drafts/%5B101-e%5D%5B118%5D%20LTE_NR_Other_WI/Round%202/draft%20R4-2118179_draft%20CR%20to%20TS38.101-2%5BR16%5D%20On%20Simultaneous%20RxTx%20capability%20for%20FR2%20inter-band%20CA.docx |
| R4-2118180 DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE: The revision is available in the folder, where only the first band combination is repalced with “CA_n257-n259”, i.e. based on latest Rel-17 spec. https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_101-e/Inbox/Drafts/%5B101-e%5D%5B118%5D%20LTE_NR_Other_WI/Round%202/revision%20of%20R4-2118180_draft%20CR%20to%20TS38.101-2%5BR17%5D%20On%20Simultaneous%20RxTx%20capability%20for%20FR2%20inter-band%20CA.docx Huawei: See comments above, and the proposed Note to align with FR1 is: Note: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in the current version of this specification. ZTE: Although i didn’t see there are different meanings between the two notes in TS38.101-2, superscript for the combs are used to apply this notes. If deleting the wordings highlighted in yellow below will solve Huawei’s concern, we are ok to remove it. Then i will ask for a new number for revision in #100. NOTE 1: For UEs supporting inter-band CA operation in FR2, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between inter-band NR carriers in FR2 in the current version of this specification. As for the specific band combinations, we are ok for the same band combination for Rel-16, i.e. CA_n260-n261, but for the Rel-17 combinations, we didn't see technical analysis for the potential degradation due to simultaneous Rx/Tx operation, which should be discussed further. Even though no enough time to complete the analysis, it does not preclude that UE not indicate simultaneous Rx/Tx capability, and consequently, no simultaneous Rx/Tx operation. For Rel-17, cat-A CR can be considered firstly for CA_n260-n261. For the Rel-17 introduced band combinations, some analysis is needed in next meeting following the previous agreement under simultaneous Rx/Tx WI. Qualcomm: FR2 bands are TDD without appreciable filtering protection in the front end. Simultaneous Rx/Tx is the exception rather than the rule in FR2-FR2 inter-band. Not sure what ‘analysis’ is required. Instead we may need analysis to determine what RF requirements should apply if simultRxTx were allowed. We have proposed (see CR) moving the table note to the body of the text so the note can be reserved for something that only applies to some entries of the table. Currently in our view, the note applies to all entries in the table. If not agreeable, we can go back to the ZTE agreement, and the note would apply to all combos. ZTE: We agree with Qualcomm’s view. It is hard for FR2 TDD-TDD combs to support simultaneous Rx/Tx due to no enough filter attenuation between two bands, which means severe performance degradation would be seen. Unlike FR1 TDD-TDD, we even don’t know what RF requirements should be defined so far (whether or not MSD approach in FR1 can be applied?), so further discussion would be needed for supporting simultaneous Rx/Tx. I don’t think this can be discussed in simultaneous Rx/Tx WI, it is out of the scope of the WID, instead RF_FR2_enh WID would be a suitable WID to discuss. Also in last meeting, we bring contributions to RF_FR2_enh WID thread, and the WF R4-2114959 was agreed, show below: Issue 1: Whether or not mark current CA configurations as no support for SimultaneousRxTx operation. - Option 1: Agree to mark current CA configurations as no support for SimultaneousRxTx operation. - Option 2: Not agree to mark current CA configurations as no support for SimultaneousRxTx operation. Agreements: Option 1 is agreeable. Issue 2: If Option 1 above is accept, how to implement it in the TS38.101-2? - Method 1: Note for the combs (i.e. a note to be added in Table 5.2A.2-1: Inter-band CA operating bands in FR2) - Method 2: Descriptions for the requirements - Method 3: Other solutions are not precluded. Agreements: Method 1 is agreeable. Huawei: We understand that for TDD-TDD FR2 band combination, there could be performance degradation if no filtering is considered, but so far, we didn’t see the analysis how much the degradation could be. UE implementation is one perspective, network deployment scenario should also be considered. For two large separated bands, e.g. 26GHz and 42GHz, how to guarantee the two bands are co-located and synchronized? To guarantee the simultaneous Rx/Tx operation for TDD bands, UL/DL slots configuration should be the same to avoid the interference. If that is the real deployment scenario, how can the requirements to preclude the possible deployment scenario? We disagree with restriction for all FR2 band combinations blindly without technical analysis and inputs from operators. For Rel-16 combination, we are ok to make a note to indicate the condition for the applicable requirements, as that is an early release issue. To make change for the Rel-16 combo, the Rel-17 CR should be a category A CR. We are open for further discussion for other band combinations introduced in Rel-17. |
| R4-2117991 Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | ZTE: We reiterate the 1st comments here: It was agreed in R4-2114959, where Method 1 is agreeable, i.e.: - Method 1: Note for the combs (i.e. a note to be added in Table 5.2A.2-1: Inter-band CA operating bands in FR2) So the note should be added in Table 5.2A.2-1, rather than the table for the requirements. The change can be reflected in the table of band combinations, i.e. Table 5.2A.2-1. ZTE: To respect the agreements in 4959, changes will be added in Table 5.2A.2-1, and for all current CA configurations. Apple: As we commented in the first-round discussions, if ZTE’s draft CRs R4-2118179 and R4-2118180 can be endorsed, our draft CRs R4-2117991 and R4-2117992 can be noted. We just want to make sure that the current specified inter-band DL CA requirements are only applicable under non-simultaneous Rx/Tx operation between all carriers. Please note that there is slight fundamental difference between our draft CR and ZTE’s draft CR. ZTE’s draft CR would limit the combination to only non-simultaneous operation. Our draft CR ties the current specified requirements to non-simultaneous operation, but does not preclude simultaneous Rx/Tx operation if there would be demand from operators and different requirements under simultaneous Rx/Tx operation would be defined. ZTE: reply to Apple, we also does not preclude simultaneous Rx/Tx operation if there would be demand from operators and different requirements under simultaneous Rx/Tx operation would be defined. |

## Summary for 2nd round (if applicable)

The WF on simultaneous Rx/Tx is not agreed.

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2119529 | To be noted Please add the missing agreement and bring back next time. |
| R4-2118179 | The revision in R4-2120052 is agreeable. |
| R4-2118180 | Please change the revised CR (R4-2119950) to cat-A and mirror the changes in R4-2120052. |
| R4-2117991 | To be noted |

# Topic #4: Additional LTE bands for UE Cat M1/2, NB1/2

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117043 | Ligado Networks | Adding A-MPR requirement for LTE Band 24 for CAT-M1/M2 |
| R4-2119006 | Ericsson | This paper presents results of A-MPR simulations for LTE Band 24 CAT-M1 UE with full-RB and sub-PRB allocation. |
| R4-2119007 | Ericsson | This paper presents initial analysis and work plan on adding the B48 on CAT-M1/M2 and NB1/NB2. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 4-1 A-MPR for LTE Band 24

Sub-topic description: According to the agreed WF, two options are going to be explored for the feasibility study.

Open issues and candidate options before e-meeting:

Issue 4-1-1: A-MPR for NS_56

### Sub-topic 4-2 Add B48 for Cat-M1/M2 and NB1/NB2

Sub-topic description: According to the agreed WF, two options are going to be explored for the feasibility study.

Open issues and candidate options before e-meeting:

Issue 4-2-1: A-MPR for NS_27

Issue 4-2-1: Work plan

## Companies views’ collection for 1st round

### Open issues

Issue 4-1-1: A-MPR for NS_56

Discuss the A-MPR values as proposed in R4-2119006

- Proposals

  - Option 1: Agree with the proposed values

  - Option 2: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |

Issue 4-2-1: A-MPR for NS_27

Discuss whether A-MPR is needed for NS_27 for Cat-M1/M2

- Proposals

  - Option 1: A-MPR is needed for sub-PRB allocation, but not for full-RB allocation

  - Option 2: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | Could proponent explain more why the full-RB allocation is not needed for NS_27? Don’t we need to consider the CAT M1/M2 with 1.4MHz channel bandwidth? |
| Ericsson | For NS 27, there is no A-MPR needed for 5MHz and 10MHz, the A-MPR is needed for 15MHz and 20MHz. Cat M1 is 1.4MHz and CAT-M2 is 5MHz, so we think there is no A-MPR needed for these two channel BW. |

Issue 4-2-2: Work plan

Discuss the proposed work plan as below.

1.	RAN4#101e: Discuss and agree with the work plan.

2.	RAN4#101e-b: CR to the UE and BS spec and subPRB simulation results can be provided.

3.	RAN4#102e: CR to the UE and BS spec.

- Proposals

  - Option 1: Approve the above work plan.

  - Option 2: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | Could proponent explain more why the full-RB allocation is not needed for NS_27? Don’t we need to consider the CAT M1/M2 with 1.4MHz channel bandwidth? |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117043 CR for adding A-MPR for LTE Band 24 for UE categories M1 and M2 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 4-1 | Issue 4-1-1: A-MPR for NS_56 No objections received, the A-MPR values are agreed. Issue 4-2-1: A-MPR for NS_27 After proponent’s clarification, no further questions are raised. The proposal is agreed, i.e., A-MPR is needed for sub-PRB allocation, but not for full-RB allocation Issue 4-2-2: Work plan The proposed work plan can be approved. Tentative agreements: Candidate options: Recommendations for 2nd round: The topic is closed. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2117043 | agreeable |

# Topic #5: NB-IoT 16QAM

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117192 | Nokia, Nokia Shanghai Bell | Adding 16-QAM for NB-IoT into the specification |
| R4-2117250 | Nokia, Nokia Shanghai Bell | Proposal 1: The same rated output power as declared for QPSK should be used for 16QAM, i.e., not to allow different rated output power for 16QAM. Proposal 2: To define at least one of the new uplink FRCs for 16QAM as shown in table 2 above. |
| R4-2118544 | Huawei, HiSilicon | Proposal 1: Allow manufacturers to declare different rated output power for NB-IoT 16QAM transmission in standalone mode. Proposal 2: Consider whether 16QAM FRCs provides extra value in addition to the existing QPSK FRCs for NB-IoT BS dynamic range tests. If so, consider whether 16QAM FRCs can replace QPSK FRCs in conformance tests for base stations that support 16QAM. |
| R4-2118545 | Huawei, HiSilicon | Proposal 1: Take the above simulation results into account when deciding the PC6 MPR for 16QAM. |
| R4-2118996 | Ericsson | Observation#1: There should be no coverage impact on legacy NB-IoT device due to the 16QAM introduction. Observation#2: new feature will be specified starting from Rel-17 and there is no NBC (non-backward compatibility issue for it. Observation#3: For new equipment, it is better to have one NB-IoT declared power to support both 16QMA and QPSK. Observation#4: Declaration on the NB-IoT carrier to support 16QAM would be fine for legacy equipment. Proposal-1 :Allow the power boosting of 6dB for NB-IoT carrier signal mixing the QPSK tone and 16QAM tone wih with in-band and guard band operation for a LTE and NR carrier. Proposal-2: Considering 16QAM is supported starting from 3 tones, existing dynamic range requirement for QPSK and 1 tone is stringent enough and there is no need to introduce 16 QAM FRC for BS dynamic range. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 5-1 BS RF Requirements

Sub-topic description: The impact to BS RF requirements in support of 16QAM are discussed here.

Open issues and candidate options before e-meeting:

Issue 5-1-1: Declaration of rated output power for 16QAM DL

Issue 5-1-2: 16QAM FRCs for BS Rx dynamic range tests

### Sub-topic 5-2 UE RF Requirements

Sub-topic description: The impact to UE RF requirements in support of 16QAM are discussed here.

Open issues and candidate options before e-meeting:

Issue 5-2-1: MPR for 16QAM PC6

## Companies views’ collection for 1st round

### Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

Issue 5-1-1: Declaration of rated output power for 16QAM DL

- Discuss whether to allow manufacturers to declare different rated output power for NB-IoT 16QAM transmission in standalone mode

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: Others (to be proposed)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Propose option 2; for R4-2118544, the 16QAM declaration is applicable when configured for 16QAM transmissions, so would apply also to BCH and QPSK PDSCH transmission when configured for 16QAM transmissions even though 16QAM PDSCH is not being transmitted. |
| Huawei | Option 1. In standalone mode, the power of the NB-IoT carrier could be as high as that of a LTE carrier. But with a CBW of approximately 1 RB, the PSD of the NB-IoT carrier is much higher than that of LTE, like a single-tone in the spectrum, which makes it very challenging for the BS power amplifiers in today’s multi-carrier, multi-RAT implementations. With increased PAPR and tightened EVM limit, 16QAM is even harder than QPSK. Hence we believe a different rated output power should be allowed as an option. Regarding Nokia’s comment above, we agree with the analysis from the perspective of minimum requirements. However, for NB-IoT DL, the modulation is applied to the whole RB and whole sub-frame, which makes it relatively easy to scale the power of different modulations in the baseband. Therefore, in practical implementations, the impact to QPSK and other channels can be minimized if not eliminated. Besides, the declaration is optional and it wouldn’t bring any harm to implementations that do not use it. |
| Ericsson | Option 2. The RF impact of DL 16QAM is not backward compatible and thus is NBC issue for legacy BS. 3GPP will not mandate the legacy BS to support this feature. But if it could support this with different NB-IoT rated power because backoff would be needed, operator needs to be aware about this and plan it in advance. This means that declaration will be different w/wo the 16QAM support in practice. |

Issue 5-1-2: 16QAM FRCs for BS Rx dynamic range tests

- Discuss whether to add 16QAM FRCs for BS Rx dynamic range tests

- Proposals

  - Option 1: Yes

  - Option 2: No

  - Option 3: FFS

  - Option 4: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Propose option 1, but OK with option 2 provided that 16QAM FRCs will be added for demodulation tests; for R4-2118996 proposal 1, there can be NB-IoT positions near the edges of the transmission BW where only a power boosting of 3 dB is required, and guard band operation is implementation specific; for R4-2118996 proposal 2, the Rx dynamic range test is not the most stringent test for spur at the receiver caused by design imperfection (clock harmonic or other spur), a test with a spectrally more concentrated interference than AWGN should be used instead. |
| Huawei | Either option 2 or 3 is ok. |
| Ericsson | Option 2. The definition of the single tone as stringent case was agreed when NB-IoT is introduced. Thus the 1 tone for REFSENS. BS usually define full RB for REFSENS and not define single RB as worst case. |

Issue 5-2-1: MPR for 16QAM PC6

- The MPR simulation results are summarized as follows.

Table 5.2.1-1: MPR for 16QAM (power class 6)

| Tone spacing | Number of tones | Allocated tones | MPR [dB] R4-2118545 | MPR [dB] R4-2113620:B | Average [dB] |
| --- | --- | --- | --- | --- | --- |
| 15 kHz | 3 | 0-2 | 2.1 | 1.3 | 1.7 |
|  |  | 3-5 | 2.1 | 1.2 | 1.7 |
|  |  | 6-8 | 2.1 | 1.2 | 1.7 |
|  |  | 9-11 | 2.1 | 1.3 | 1.7 |
|  | 6 | 0-5 | 2.1 | 1.1 | 1.6 |
|  |  | 6-11 | 2.1 | 1.1 | 1.6 |
|  | 12 | 0-11 | 2.9 | 2.3 | 2.6 |

- Proposals

  - Option 1: Agree with the average values

  - Option 2: Others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Nokia | Option 1: Agree with the average values |
| Huawei | For simplicity, propose to add 16QAM MPR as following: Modulation; QPSK; 16QAM. MPR for 3 Tones allocation; ≤ 0.5 dB; ≤ 1.7 dB. MPR for 6 Tones allocation; ≤ 1 dB; ≤ 1.7 dB. MPR for 12 Tones allocation; ≤ 1.5 dB; ≤ 2.6 dB. |
| Nokia | Reply to Huawei comment: Table 5.2.1-1: MPR for 16QAM (power class 6) Tone spacing; Number of tones; Allocated tones; MPR [dB] R4-2118545; MPR [dB] R4-2113620:B; Average [dB]. 15 kHz; 3; 0-2; 2.1; 1.3; 1.7. 3-5; 2.1; 1.2; 1.7. 6-8; 2.1; 1.2; 1.7. 9-11; 2.1; 1.3; 1.7. 6; 0-5; 2.1; 1.1; 1.6. 6-11; 2.1; 1.1; 1.6. 12; 0-11; 2.9; 2.3; 2.6. Average for 6-toned should be 1.6 not 1.7 dB. Otherwise ok for the proposal. |
| Huawei | To Nokia: no strong view on the 0.1 dB difference. However, wouldn’t it be a bit strange if MPR for 6-tone is smaller than that for 3-tone? |
| Nokia | Reply to Huawei comment: It is not surprising that the MPR for 6 tones is lower than that for 3 tones because for both, the in-band emission acts as the gating factor. The same phenomenon appears in all the three power classes. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117192 CR IBE mask and MPR for NB-IoT 16-QAM | Nokia: Can be revised to take the average values |
|  | Huawei: Please include MPR for PC6. Also it’s better to postpone the CR until the CR for the BS side is ready, so that 16QAM can be added for both sides in a complete package. |
|  | Nokia reply to Huawei comment: Can include PC6 in the revised CR, which can then be endorsed in this meeting to reflect the progress of the WI in the SR. |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #5-1 | Issue 5-1-1: Declaration of rated output power for 16QAM DL Views are still diverged. Issue 5-1-2: 16QAM FRCs for BS Rx dynamic range tests It seems companies are willing to compromise and the consensus is not to add 16QAM FRCs. Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss the remaining open issues in a WF. |
| Sub-topic #5-2 | Issue 5-2-1: MPR for 16QAM PC6 Views are close. Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss the revised CR in the 2nd round. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

### Open issues

WF on remaining issues for NB-IoT 16QAM

| Company | Comments |
| --- | --- |
| Nokia | We are ok with the Ericsson version. |

### CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| Revision of R4-2117192 CR IBE mask and MPR for NB-IoT 16-QAM | Huawei: The CR looks agreeable, but as commented in the 1st round, prefer to postpone it till the BS side CR is ready. |
|  | Nokia reply to Huawei comment: As the CR is technically agreeable, we should endorse the CR in this meeting to reflect the progress of the WI in the SR. |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

The CR R4-2119952 (revised from R4-2117192) can be endorsed, which concludes the UE side RF requirements for 16QAM. For the BS side, there’s one pending issue. The WF in R4-2119948 is agreeable.

# Topic #6: LTE-MTC Additional Enhancements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117620 | Sony | Observation 1 For CE Mode B, PUSCH would be the limiting channel for coverage if coverage were defined at a PUSCH data rate of 3kbps. Observation 2 For CE Mode A, PUSCH is expected to be the coverage limiting channel. Observation 3 Transmission of sub-PRB PUSCH at a higher transmit power can improve the overall cell coverage in both CE Mode B and CE Mode A. Observation 4 Transmission of sub-PRB PUSCH at a higher transmit power improves spectral efficiency. Observation 5 Transmission of sub-PRB PUSCH at a higher transmit power improves UE battery lifetime. Observation 6 Transmission of sub-PRB PUSCH at a higher transmit power reduces latency. Observation 7 Transmission of sub-PRB PUSCH at a higher transmit power benefits both the network and UE. Proposal 1 RAN4 to study how to specify the increased relative power for sub-PRB PUSCH transmission. At least the following approaches can be considered: • power boosting for sub-PRB PUSCH from a lower PC; or • power reduction for full-PRB PUSCH, PRACH and PUCCH from a higher PC. |
| R4-2118997 | Ericsson | Observation#1: Reducing the full-PRB transmission power generally is against the UE rated power definition. Proposal-1: Follow the framework of NR pi/2 BPSK power boosting if RAN4 decides that there is an overall gain from the subPRB boosting. Observation#2: If RAN4 decided for the subPRB power boosting, it will be possible to boost power for 2 out 3 tone subPRB transmission thanks to low PAPR characteristic. Proposal-2: Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission. Proposal-3: RAN4 need make work plan to add sub-PRB boosting and decide whether to continue on subPRB power boosting work or not. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 6-1 Feasibility study on max power reduction for PRACH, PUCCH, and full-PRB PUSCH

Sub-topic description: According to the agreed WF, two options are going to be explored for the feasibility study.

Open issues and candidate options before e-meeting:

Issue 6-1-1: Feasibility study on max power reduction

## Companies views’ collection for 1st round

### Open issues

Issue 6-1-1: Feasibility study on max power reduction

- Proposals

  - Option 1: power boosting for sub-PRB PUSCH from a lower PC

    - Focus on PC5 CAT-M1 device for the potential power boosting to PC3 on subPRB transmission

  - Option 2: power reduction for full-PRB PUSCH, PRACH and PUCCH from a higher PC

  - Option 3: others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Option 1 seems reasonable. The power boosting only applies to 2-out-of-3 subPRB allocation as the PAPR is 1.1 dB compared with 8 dB PAPR for full-RB allocation. Option 2 will make control channel 3 dB lower than rated power and lead to lower mobility performance compared with the same rated power UE. |
| Qualcomm | Option 3. We would rather not pursue this study as there hasn’t be any data submitted after multiple meetings. |
| Sony | We think that the outcome of either option is the same: subPRB PUSCH can be transmitted at a higher power than full-PRB PUSCH, PRACH and PUCCH. The approach that is more in line with the WID objective is option 2: - For UEs supporting PUSCH sub-PRB resource allocation, study and if found feasible, specify support power reduction for PRACH, PUCCH, and full-PRB PUSCH, with a maximum reduction of e.g. 3 dB below sub-PRB PUSCH power. [LTE-MTC] [RAN4] Hence, our preference would be to go with option 2, where the “higher PC” is PC3. We could thus be more focused with option 2 and state: Option 2: power reduction for full-PRB PUSCH, PRACH and PUCCH from PC3. |

Issue 6-1-2: Work plan

- Proposals

  - Option 1: RAN4 needs to decide whether to continue on subPRB power boosting work or not

    - If yes, make a work plan to add sub-PRB boosting

  - Option 2: others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Ericsson | Option 1. As subPRB MPR/A-MPR not specified for PC5, evaluation of the MPR/A-MPR of PC5 Cat-M device is needed. subPRB power boosting is possible when there is no MPR/A-MPR needed. As there are only two meetings left so work plan would be needed. The detail can be discussed in 2nd round. |
| Qualcomm | Option 2. As commented in 6-1-1, we think this part of the work should be dropped. |
| Sony | Option 1: RAN4 should continue the work on subPRB boosting. This is an important feature, and our studies show that there are clear benefits in terms of spectral efficiency, coverage, battery lifetime and latency. The work plan should be fairly straightforward. The relative power boosting / power reduction between sub-PRB PUSCH and full-PRB PUSCH transmissions has already been defined in 36.101 section 6.2.3F. Since there are many implementations where NB-IoT and eMTC share the same radio, it seems evident that the MPR in Table 6.2.3F-1 should be the baseline for the eMTC definition of power reduction of full-PRB channels relative to sub-PRB PUSCH. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic# 6-1 | Issue 6-1-1: Feasibility study on max power reduction Views are much split, three companies support three different options. There’s little progress on this issue. Issue 6-1-2: Work plan Two companies outline their work plan proposals, while one company suggests to drop the work. As per the schedule of the WI, the core part requirements have to be completed by RAN4#102, i.e. only two meetings left for the work if RAN4 decide to continue. Otherwise, it’s better to remove this objective in the coming RAN#94. Tentative agreements: Candidate options: Recommendations for 2nd round: Discuss and decide whether to drop the work. If not, discuss how to complete the work in two more meetings. |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

### Open issues

As per the schedule of the WI, the core part requirements have to be completed by RAN4#102, i.e. only two meetings left for the work if RAN4 decide to continue. Otherwise, it’s better to remove this objective from the WID in the coming RAN#94.

Issue 6-1-3: How to proceed with the WID objective

- Proposals

  - Option 1: Remove the objective from the WID

  - Option 2: Continue and complete the work in two meetings

  - Option 3: others (please specify)

- Recommended WF

  - TBA

| Company | Comments |
| --- | --- |
| Qualcomm | We support to remove the objective from the WID. There is a clear lack of interest for this work. |
| Huawei | Share similar view with Qualcomm. Only two companies provided contributions, but even they have different views on the objective as seen from the 1st round. Were the work to continue, there’d be little chance for it to be completed in two meetings. |
| Sony | Option 2. We prefer to move this objective on. We shouldn’t be spending all of our time on working out a work plan / time plan. We need to work on the technical issues. Our concern with the “power boosting for sub-PRB PUSCH from a lower PC” approach is that it would require specification of MPR / A-MPR for PC5, which would increase the workload. We also think that power-boosting of the sub-PRB is less in-line with the WID objective, which talks about power reduction of “other channels” rather than power boosting of sub-PRB. In summary, there seems to be less work involved with power reduction of non-sub-PRB channels (and the solution has already been specified for NB-IOT, so we know it works) and this is in line with the WID. |

## Summary for 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Two companies support option 1 while one company supports option 2. Anyway, changing WID objectives is RAN plenary business. As long as the objective exists, interested companies are encouraged to bring contributions before the end of the WI.

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on simultaneous Rx/Tx capability | Huawei, HiSilicon |  |
| WF on remaining issues for NB-IoT 16QAM | Huawei, HiSilicon |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2117308 | Request adding support NR band n77 with UL-MIMO for PC1.5 UPUE | Verizon Denmark | Endorsed |  |
| R4-2119530 | draft CR for TS 38.101-1 Introduce UL MIMO configurations for band n40 | Huawei, HiSilicon | Endorsed |  |
| R4-2119531 | revised WID Rel-17 Basket UL MIMO bands | Huawei, HiSilicon | Endorsed |  |
| R4-2117323 | TR 37.867 v0.4.0 | CATT | Withdrawn |  |
| R4-2118321 | CR to 38.101-1 Introduce DL interruption clarification for CA conduting Tx Switching | China Telecom | Email approval |  |
| R4-2118322 | Draft CR to 38.101-1 Correction on the DL interruption clarification table | China Telecom | Revised |  |
| R4-2117665 | Discussion on the simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination | SoftBank Corp. | Noted |  |
| R4-2118881 | R17 simultaneous RxTx | OPPO | Noted |  |
| R4-2119215 | Simultaneous Rx/Tx capability for FR1+FR1 FDD-TDD band combination | ShenZhen Zhongxing Shitong | Noted |  |
| R4-2119528 | TR 38.839 v0.1.0 | Huawei, HiSilicon | Approved |  |
| R4-2119529 | TP for TR 38.839: Principles for simultaneous RxTx capability | Huawei, HiSilicon | Return to |  |
| R4-2117681 | Wording proposal for note of simultaneous Tx/Rx | MediaTek Beijing Inc. | Noted |  |
| R4-2118179 | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation | Return to |  |
| R4-2118180 | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation | Revised |  |
| R4-2118887 | R17 FR2 simultaneous RxTx | OPPO | Noted |  |
| R4-2119519 | On simultaneous RxTx for FR2 | Huawei, HiSilicon | Noted |  |
| R4-2117991 | Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | Apple | Return to |  |
| R4-2117992 | Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | Apple | Return to |  |
| R4-2117043 | CR for adding A-MPR for LTE Band 24 for UE categories M1 and M2 | Ligado Networks | Endorsed |  |
| R4-2119006 | AMPR simulation results for Cat-M1 for B24 | Ericsson | Noted |  |
| R4-2119007 | RF impact and work plan for B48 on CAT-M1/M2 and NB1/NB2 | Ericsson | Approved |  |
| R4-2117192 | CR IBE mask and MPR for NB-IoT 16-QAM | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2117250 | Proposals on BS RF requirements for support of 16QAM in NB-IoT | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2117620 | On max power reduction for PRACH, PUCCH, and full-PRB PUSCH | Sony | Noted |  |
| R4-2118544 | Remaining issues for NB-IoT 16QAM BS RF requirements | Huawei,HiSilicon | Noted |  |
| R4-2118545 | PC6 MPR for NB-IoT 16QAM | Huawei,HiSilicon | Noted |  |
| R4-2118996 | BS RF impact analysis on R17 NB_IoT | Ericsson | Noted |  |
| R4-2118997 | RF impact analysis on R17 eMTC WID | Ericsson | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2119949 | Draft CR to 38.101-1 Correction on the DL interruption clarification table | China Telecom | Agreeable |  |
| R4-2119947 | WF on simultaneous Rx/Tx capability | Huawei, HiSilicon | To be noted |  |
| R4-2119529 | TP for TR 38.839: Principles for simultaneous RxTx capability | Huawei, HiSilicon | To be noted |  |
| R4-2120052 | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation | Agreeable |  |
| R4-2119950 | DRAFT CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation | Agreeable |  |
| R4-2117991 | Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | Apple | Noted |  |
| R4-2119952 | CR IBE mask and MPR for NB-IoT 16-QAM | Nokia, Nokia Shanghai Bell | To be endorsed | Do not merge into R17 big CR. |
| R4-2119948 | WF on remaining issues for NB-IoT 16QAM | Huawei, HiSilicon | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents