3GPP TSG-RAN WG4 Meeting # 101-e 	R4-2119942

# Electronic Meeting, 1st – 12th November 2021

Source:		Verizon, Samsung

Title:			TP for TR 37.826 for DC_66_n77

Agenda item:		7.30.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.826 to include additional DC_66_n77 configuration according to the request in [1] and the approved text proposal [2].

Reference

[1] RP-211760: 	Revised WID: Rel-17 Power Class 2 for EN-DC with x LTE bands + y	NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2)

[2] R4-2103178: 	TP for TR 37.826 for DC_66_n77

# Text Proposal

<Start of Text Proposal>

## 6.6	DC_66_n77

### 6.6.1	Transmitter Characteristics

#### 6.6.1.1	Maximum Output Power

Table 6.6.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

6.6.1.2	Configurations for EN-DC

Table 6.6.1.2-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | DC_66A_n77A |

#### 6.6.1.3	Co-existence study

According to the PC3 DC_66A_n77A study, the 2nd and 5th order IMD products generated from dual uplinks of band 66 and n77 may fall into the band 66 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for the PC2 DC_66A_n77A combination.

### 6.6.2	Receiver Characteristics

#### 6.6.2.1		MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 6.6.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_66_n77 are defined in table 6.6.2.1.1-1.

Table 6.6.2.1.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD for PC2 | Duplex mode | Source of IMD |
| Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | 1775 | 5 | 25 | 2175 | 34.33 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |

#### 6.6.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_66_n77 are same as the Case A defined in table 6.6.2.1.1-1.

<End of Text Proposal>