3GPP TSG-RAN4 Meeting #101-e	R4-2119943

# Online, , 1st November 2021 - 12th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR 38.101-3 Corrections for EN-DC PC2 MOP Band-edge Relaxations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ENDC_UE_PC2_R17_NR_TDD-Core |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Maximum Output Power for DC tables contain incorrect superscript references to the table note that refers to the band-edge relaxation in NOTE 1 in Table 6.2B.1.3-1 for PC2 combinations as it should be based on the requirement that the band-edge relaxation applies for at least one of the bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Maximum Output Power for DC table to remove the superscript references to the table note that refers to the band-edge relaxation in NOTE 1 in Table 6.2B.1.3-1 for PC2 combinations and for underlying PC3 combinations. Updated table note to follow same approach utilized for CA case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements would contain incorrect band-edge relaxation references for certain PC2 and PC3 combinations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-3 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged content omitted >>

#### 6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration |  | Power class 2 (dBm) |  | Tolerance (dB) |  | Power class 3 (dBm) |  | Tolerance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n8A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n20A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n50A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n51A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_1A_n80A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n12A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n30A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n41A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_2A_n46A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n48A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_2A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n5A DC_3C_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n8A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n20A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n34A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n41A, DC_3C_n41A, DC_3C_n41A, |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_3A_n50A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n51A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n77A DC_3C_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n78A DC_3C_n78A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_3A_n79A DC_3C_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n80A_ULSUP-TDM_n41 DC_3C_n80A_ULSUP-TDM_n41 |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n80A_ULSUP-TDM_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n82A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_3A_n84A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_4A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n12A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n30A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n48A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_5A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_5A_n78A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_5A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n5A DC_7C_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n8A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n20A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n25A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n51A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_7A_n78A DC_7C_n78A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_7A_n80A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n20A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n34A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n39A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n41A, |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n78A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_8A_n79A DC_8A_n79C |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n80A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n81A_ULSUP-TDM_n41 |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_11A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_11A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_11A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_11A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_11A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_11A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n25A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n30A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n71A7 |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_12A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_12A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n25A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n48A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_13A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_13A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_14A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_14A_n30A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_14A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_14A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_18A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_18A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_18A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_18A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_18A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_18A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_19A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_19A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_19A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_19A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n8A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n50A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n51A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n80A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_20A_n83A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_21A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_21A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_21A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_21A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_21A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_25A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_25A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_25A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_26A_n25A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_26A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_26A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_26A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_26A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n7A DC_28A_n7B |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n8A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n50A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n51A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n83A_ULSUP-TDM_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_30A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_30A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_30A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_30A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_38A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_38A_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_38A_n8A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_38A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_38A_n78A |  |  |  |  |  | N/A |  | N/A |  |
| DC_39A_n40A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_39A_n41A DC_39C_n41A |  | 265 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_39A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_39A_n79A |  | 265 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_40A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_40A_n41A DC_40C_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_40A_n77A |  |  |  |  |  | N/A |  | N/A |  |
| DC_40A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_40C_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_40A_n79A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_41A_n3A DC_41C_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_41A_n28A DC_41C_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_41A_n77A DC_41C_n77A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_41A_n78A DC_41C_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_41A_n79A DC_41C_n79A |  | 265 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_42A_n1A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_42A_n3A DC_42C_n3A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_42A_n28A DC_42C_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_42A_n51A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_42A_n77A |  |  |  |  |  | N/A |  | N/A |  |
| DC_42A_n78A |  |  |  |  |  | N/A |  | N/A |  |
| DC_42A_n79A |  |  |  |  |  | N/A |  | N/A |  |
| DC_48A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_48A_n12A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_48A_n25A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_2A_n46A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_48A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_48A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n7A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n12A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n25A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n28A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n30A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n41A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_66A_n46A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n48A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n71A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n77A |  | 266 |  | +2/-3 |  | 23 |  | +2/-3 |  |
| DC_66A_n78A DC_66A-66A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n2A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n5A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n38A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n41A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n48A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n66A |  |  |  |  |  | 23 |  | +2/-3 |  |
| DC_71A_n78A |  |  |  |  |  | 23 |  | +2/-3 |  |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

## << End of changes >>