3GPP TSG-RAN WG4 Meeting # 101-e 												R4-2120352

Electronic Meeting, 1st – 12th November 2021

Agenda item:			5.1.4.2.1. & 5.1.4.2.2.

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [101-e][209] Maintenance_NR_pos_2

Document for:	Information

# Introduction

The scope of this email discussion is UE RRM requirements for NR positioning from the following agenda items:

- AI 5.1.4.2.1. UE requirements and test cases

- & 5.1.4.2.2. gNB requirements

In providing comments, companies are encouraged to:

- Be concise

- Provide comments on all topics/sub-topics of interest

- Ensure that comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

# Topic #1: General performance requirements for NR Positioning (AI 5.1.4.2.1.1)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Open issues summary and companies views’ collection for 1st round

N.A.

### Open issues

N.A.

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

Sub-topic#1-1

## Summary on 2nd round

# Topic #2: UE Measurement Accuracy Requirements (AI5.1.4.2.1.2)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2117346 | CATT | Proposal 1: The PRS RSRP accuracy requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. Proposal 2: For the PRS RSRP accuracy requirements in extreme condition, define a single value for each SINR side condition. |
| R4-2117347 | CATT | DraftCR on UE positioning accuracy requirements |
| R4-2117348 | CATT | DraftCR on UE positioning accuracy requirements |
| R4-2117490 | Qualcomm Incorporated | Proposal 1: For UE Rx-Tx measurement accuracy requirements in FR1, add group delay calibration margins listed in the table below. Min(PRS BW, SRS BW) (MHz); Margin (Tc). ≥ 5; 160. ≥ 10; 80. ≥ 20; 40. ≥ 50; 16. ≥ 100; 8. Proposal 2: For UE Rx-Tx measurement accuracy requirements in FR2, add group delay calibration margins listed in the table below. FFS if additional margin should be added for OTA calibration. Min(PRS BW, SRS BW) (MHz); Margin (Tc). ≥ 20; 80. ≥ 50; 32. ≥ 100; 16. ≥ 200; 8. Proposal 3: For UE Rx-Tx measurement accuracy, discuss whether to add a constant margin to account for frequency error. Proposal 4: For RSTD measurement accuracy requirements in FR1 when the reference and target PRS resources are in the same PFL (single PFL case), add group delay calibration margins listed in the table below. PRS BW (MHz); Margin (Tc). ≥ 5; 160. ≥ 10; 80. ≥ 20; 40. ≥ 50; 16. ≥ 100; 8. Proposal 5: For RSTD measurement accuracy requirements in FR2 when the reference and target PRS resources are in the same PFL (single PFL case), add group delay calibration margins listed in the table below. FFS if additional margin should be added for OTA calibration. PRS BW (MHz); Margin (Tc). ≥ 20; 80. ≥ 50; 32. ≥ 100; 16. ≥ 200; 8. Proposal 6: For PRS-RSTD measurements in FR1 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS. Proposal 7: For PRS-RSTD measurements in FR2 with neighbor and reference PRS resources in different PFLs, the value of the group delay calibration margin is FFS. Proposal 8: Add a frequency margin of 50 Tc for RSTD that applies for a maximum time offset of 1 second between the PRS resource instances used to calculate the RSTD measurement. Proposal 9: No need to define PRS-RSRP accuracy requirements for extreme conditions. Observation 1: PRS resources in the same PRS resource set belong to a single positioning frequency layer and are transmitted by a single TRP. Proposal 10: The definition and applicability of PRS-RSRP relative accuracy in TS 38.133, section 10.1.24.2.2 should be edited and clarified to reflect prior RAN4 agreement as follows: - The relative accuracy of PRS-RSRP is defined as the accuracy of the difference between two PRS-RSRP measurements. - The relative PRS-RSRP accuracy requirements apply for the cases when PRS-RSRP is measured from PRS resources in the same PRS resource set in FR1 or FR2 and measured with same Rx beam in case of FR2. NOTE: The UE is assumed to use one Rx beam in FR1. Proposal 11: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| R4-2117500 | Qualcomm Incorporated | Draft CR: Corrections to NR positioning performance requirements |
| R4-2117501 | Qualcomm Incorporated | Draft CR: Corrections to NR positioning performance requirements |
| R4-2117755 | vivo | Proposal 1: Group delay calibration margin for the RSTD accuracy requirements is 1Ts (64Tc) in FR1 and 0.5Ts (32Tc) in FR2. Proposal 2: Frequency drift margin for the RSTD accuracy requirements is 0.5Ts (32Tc) in both FR1 and FR2. Proposal 3: The margin for PRS-RSRP accuracy requirements under extreme conditions are - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. Proposal 4: FFS applicability of UE Rx-Tx measurement accuracy requirements if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. Proposal 5: Group delay calibration margin for the UE Rx-Tx time difference accuracy requirements is 0.75Ts (48Tc) in FR1 and 0.5Ts (32Tc) in FR2. |
| R4-2117758 | vivo | Draft CR to 38.133 correction to NR positioning accuracy requirements |
| R4-2117759 | vivo | Draft CR to 38.133 correction to NR positioning accuracy requirements |
| R4-2118016 | Intel Corporation | Proposal 1: The RF margin for RSTD/UE Rx-Tx time difference measurement accuracy requirements can be differentiated by FR and PRS BW. Proposal 2: The maximum frequency drift between the measured TRPs can be dependent the maximum time offsets among the measured TRPs/cells (e.g. 2* expectedRSTD) Proposal 3: Applicability of accuracy requirements under TA adjustment if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment can be: - UE Rx-Tx measurement accuracy requirements shall apply for a serving cell even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a neighbor cell if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment |
| R4-2118380 | OPPO | Draft CR to TS 38.133 Rel-16 on measurement accuracy for UE Rx-Tx time difference |
| R4-2118381 | OPPO | Draft CR to TS 38.133 Rel-17 on measurement accuracy for UE Rx-Tx time difference |
| R4-2118939 | ZTE Corporation | - Proposal 1: PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2. Proposal 2: UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| R4-2118940 | ZTE Corporation | [draft CR] accuracy requirements for PRS-RSRP |
| R4-2118941 | ZTE Corporation | [draft CR] accuracy requirements for PRS-RSRP (Cat A) |
| R4-2119332 | Huawei, Hisilicon | Proposal 1: Add the following group delay calibration margin for RSTD accuracy. - For FR1, when the reference cell and neighbour cell are on same PFL: ±8ns for PRS BW ≥ 50MHz and ±16ns for PRS BW < 50MHz - For FR1, when the reference cell and neighbour cell are on different PFLs: ±16ns for PRS BW ≥ 50MHz and ±32ns for PRS BW < 50MHz - For FR2: ±16ns for PRS BW >= 50MHz and ±32ns for PRS BW < 50MHz Proposal 2a: For RSTD accuracy requirements, define the frequency drift margin as +/-32Tc based on maximum offset of 160ms. Proposal 2b: RAN4 to remove the test cases for RSTD accuracy with dual PFLs. [Moderator : remove to subtopic xxx] Proposal 4: Add the following group delay calibration margin for UE Rx-Tx accuracy. - ±24ns for PRS BW ≥ 50MHz - ±48ns for PRS BW < 50MHz Proposal 5: Applicability of Rx-Tx accuracy requirements with autonomous timing adjustment is defined as: - UE Rx-Tx measurement accuracy requirements shall apply for a cell on which SRS is transmitted, even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - UE Rx-Tx measurement accuracy requirements shall not apply for a cell on which SRS is not transmitted, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| R4-2119333 | Huawei, Hisilicon | CR on accuracy requirements for positioning measurement |
| R4-2119334 | Huawei, Hisilicon | CR on accuracy requirements for positioning measurement R17 |
| R4-2119399 | Nokia, Nokia Shanghai Bell | - a) UE Rx-Tx measurement accuracy requirements shall apply for a serving cell even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - b) UE Rx-Tx measurement accuracy requirements shall not apply for a neighbour cell if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. Absolute PRS-RSRP measurement requirements in extreme condition are specified and are X dB larger than that in normal condition, where X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - 1dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2 |
| R4-2119458 | Ericsson | PRS-RSRP accuracy requirements: - Observation 1: It may not be uncommon that the UE operates in extreme conditions e.g. at 14C or 36C. - Observation 2: PRS-RSRP may significantly drift or vary in the extreme conditions. - Proposal 1: Specify PRS-RSRP accuracies in extreme conditions as in Option 1 in the WF [1]. UE Rx-Tx accuracy under autonomous timing adjustment: - Observation 3: The UL autonomous timing adjustment, which is based on the DL timing, follows the DL timing change in the reference cell, resulting in the UE transmit timing changes in the same direction in which the DL timing changes in the reference cell. - Observation 4: The UE can be configured to perform UE Rx-Tx measurements on multiple cells (multi-RTT). - Observation 5: In one multi-RTT measurement scenario, the UE may transmit SRS on reference cell (e.g. PCell) while perform PRS also on the reference cell (e.g. PCell). In this case the changes in SRS timing due to autonomous timing adjustment follows the PRS timing change in the reference cell. - Observation 6: In another multi-RTT measurement scenario, the UE may transmit SRS on reference cell (e.g. PCell) while perform PRS on another serving cell (e.g. SCell). In this case the changes in SRS timing due to autonomous timing adjustment is not related to PRS timing in the SCell. - Observation 7: In another multi-RTT measurement scenario, the UE may transmit SRS on reference cell (e.g. PCell) while perform PRS on a neighbor cell (non-serving cell). In this case as well the changes in SRS timing due to autonomous timing adjustment is not related to PRS timing in the neighbor cell. - Proposal 2: Applicability of accuracy requirements under TA adjustment if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment is specified to cover all supported single RTT and multi-RTT measurement scenarios. - Proposal 3: Applicability of accuracy requirements under TA adjustment if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment is specified based on Option 3 in the WF [1]: - If SRS is transmitted on a cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall apply even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - If SRS is transmitted on a cell different than the cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment |
| R4-2119459 | Ericsson | Draft CR |
| R4-2119460 | Ericsson | Draft CR |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 2-1 Group delay calibration margin for RSTD requirements

[Moderator Notes: in R4#100e the agreements for this are captured below:

- Group delay calibration margin for the RSTD accuracy requirements is FFS

  - Different values for the group delay calibration margin for the RSTD accuracy requirements can be defined depending on PRS  parameters combination

    - FR(frequency range)

    - PRS BW (absolute BW)

    - Number of PFLs and whether reference/neighbor cells belong to same PFL

  - Companies are encouraged to bring analysis for dependency of margin on these parameters

]

- Option 1 (Qualcomm, Huawei, E///,Intel, vivo) : defined depending on PRS BW, FR1/FR2, PFL

- Option 1a(Qualcomm, Huawei,vivo):

  - in FR1 when the reference and target PRS resources are in the same PFL (single PFL case)

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | 160 |
| ≥ 10 | 80 |
| ≥ 20 | 40 |
| ≥ 50 | 16 |
| ≥ 100 | 8 |

  - in FR2 when the reference and target PRS resources are in the same PFL (single PFL case). FFS if additional margin should be added for OTA calibration.

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | 80 |
| ≥ 50 | 32 |
| ≥ 100 | 16 |
| ≥ 200 | 8 |

- Option1b (Huawei, Intel):

  - For FR1, when the reference cell and neighbour cell are on same PFL: ±8ns for PRS BW ≥ 50MHz and ±16ns for PRS BW < 50MHz

  - For FR1, when the reference cell and neighbour cell are on different PFLs: ±16ns for PRS BW ≥ 50MHz and ±32ns for PRS BW < 50MHz

  - For FR2: ±16ns for PRS BW >= 50MHz and ±32ns for PRS BW < 50MHz

- Option 2(Intel): defined depending on PRS BW, FR1/FR2.

- Option 3 (vivo) : defined depending on FR1/FR2:

  - 1Ts (64Tc) in FR1 and 0.5Ts (32Tc) in FR2

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| E/// | We are fine with the principle in option 1 |
| Intel | We are fine the principle of option 1 also. And how to group these parameter can be take Option 1b as the start point (the exact value can be bracketed ) |
| Qualcomm | Options 1 and 1a. For reference and target PRS resources in two different PFLs we prefer to keep the margin values FFS. |
| Huawei | We also support the principle in option 1. On the number of BW ranges (how to group the parameters), we are also fine with the split in option 1a, but we cannot agree on all the exact values in option 1a. On the exact values, we support option 1b for the corresponding BW ranges. |
| vivo | In principle, we can consider option 1 if such detailed categorization is necessary. Option 1b seems a balanced trade-off between simplified requirements and necessary differentiation of margins. The value can be further decided. |
| CATT | Fine with the principle in option 1. But the PRS BW in accuracy requirements is based on PRB number, should we follow the same unit for margin definition? |

### Sub-topic 2-2 Group delay calibration margin for UE Rx-Tx time difference requirements

- Option 1 (Qualcomm, Intel) : defined depending on PRS BW, FR1/FR2

- Option 1a(Qualcomm):

  - in FR1

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 5 | 160 |
| ≥ 10 | 80 |
| ≥ 20 | 40 |
| ≥ 50 | 16 |
| ≥ 100 | 8 |

  - in FR2 . FFS if additional margin should be added for OTA calibration.

| PRS BW (MHz) | Margin (Tc) |
| --- | --- |
| ≥ 20 | 80 |
| ≥ 50 | 32 |
| ≥ 100 | 16 |
| ≥ 200 | 8 |

- Option2 (Huawei): defined depending on PRS BW

  - ±24ns for PRS BW ≥ 50MHz

  - ±48ns for PRS BW < 50MHz

- Option 3 (vivo) : defined depending on FR1/FR2:

  - 0.75Ts (48Tc) in FR1 and 0.5Ts (32Tc) in FR2.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| E/// | We are fine with the principle in option 1 |
| Intel | Same as 2-1 |
| Qualcomm | Options 1 and 1a. |
| Huawei | On the general principle, we need more time to check the dependency on FR1/FR2. On the number of BW ranges (how to group the parameters), we are also fine with the split in option 1a, but we cannot agree on the exact values in option 1a. On the exact values, we support option 2 for the corresponding BW ranges at least for FR1. |
| vivo | We are fine also to consider PRS BW as one condition to define margin. One thing we would like to highlight is that the margin for UE Rx-Tx should not be larger than that for RSTD. |
| CATT | Same as sup-topic 2-1. |

### Sub-topic 2-3 Frequency drift margin for RSTD

[Moderator notes:

- The frequency drift margin in RSTD accuracy is FFS

  - Frequency drift margin will be derived based on the maximum time offsets between the two PRS resource instances from the reference cell and neighbour cells which are used for a single RSTD measurement.

    - The maximum time offset for margin definition can be FFS

  - A single fixed margin will be used for requirements definition

]

- Option 1 (vivo): fixed value 0.5Ts (32Tc) in both FR1 and FR2

- Option 2(Intel, Huawei, Qualcomm): The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells.

- Option 2a (Qualcomm) A frequency margin is 50 Tc for RSTD if a maximum time offset of 1 second between the PRS resource instances used to calculate the RSTD measurement.

- Option 2b(Huawei): the frequency drift margin is +/-32Tc based on maximum offset of 160ms

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | The principle of Option 2 can be agreed. |
| Qualcomm | Options 2 and 2a. Regarding option 2a, we tried to propose reasonable margin that does not severely limit applicability of the requirements. |
| Huawei | The principle for defining the frequency drift margin was already agreed in last meeting as shown in the Moderator’s note, and we should stick to it. On the exact value, we have some concern on option 2a. It is based on the assumption of frequency error as 0.025ppm while the RAN4 requirements in 38101 is 0.1ppm. This means to meet the RSTD accuracy requirements, UE has to meet a more stringent frequency error requirements than what is required by RAN4 and what is demanded for data communication. Although we understand that UE can do better than the requirements, we are not sure if this is a proper way to pursue (implicitly tightening a core requirement). Therefore, we still prefer to go with option 2b. |
| vivo | Option 1. Option 2b is same as option 1 in our understanding. |

### Sub-topic 2-4 Frequency drift margin for UE Rx-Tx time difference

- Option 1 (Qualcomm): FFS on whether to add a constant margin to account for frequency error.

Recommended WF: Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| Intel | Same as 2-3 |
| Qualcomm | For option 1, we don’t think it is needed based on the measurement definition. We wanted to check other companies’ views. |
| Huawei | In our view, frequency drift margin for UE Rx-Tx is not needed based on the definition in 38.215, but we are open to discuss if other companies identified such needs. |
| Vivo | It may be further studied considering multiple samples are needed for the measurement. |

### Sub-topic 2-5 RSRP requirements under the extreme conditions

- Option 1 ( CATT, vivo, ZTE, Ericsson, Nokia, Huawei, Intel). The PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is:

  - 3dB for absolute accuracy for FR1.

  - 3dB for absolute accuracy for FR2.

  - 1dB for relative accuracy for FR1.

  - 3dB for relative accuracy for FR2

- Option 1a (CATT) For the PRS RSRP accuracy requirements in extreme condition, define a single value for each SINR side condition.

- Option 2 ( Qualcomm, Huawei): No need to define PRS RSRP accuracy requirements with the extreme conditions

Recommended WF:  Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| ZTE | Support Option 1 to add extreme conditions. Extreme conditions are quite commonly seen in other RSRP tables. |
| E/// | We support option 1 |
| Intel | Slightly prefer Option 2 |
| Qualcomm | Prefer option 2. Don’t see this as high priority. |
| Huawei | We support option 2, but we can compromise to option 1 except that we think X should be 1.5dB for relative accuracy for FR1. |
| Vivo | Support option 1. |
| CATT | Support option 1. |

### Sub-topic 2-6 Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment

- Option 1. (vivo,  Qualcomm,)

  - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

- Option 2 (Intel, Huawei, Nokia)

  - UE Rx-Tx measurement accuracy requirements shall apply for a serving cell even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

  - the UE Rx-Tx measurement accuracy requirements shall not apply for a neighbor cell if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment

- Option 3 (Ericsson)

  - If SRS is transmitted on a cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall apply even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

  - If SRS is transmitted on a cell different than the cell where UE Rx-Tx measurement is performed then the UE Rx-Tx measurement accuracy requirements shall not apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

Recommended WF:  Further discussion needed. Collect companies’ views.

| Company | Comments |
| --- | --- |
| E/// | We support option 3. - Issue with Option 2: Serving cell on which the UE Rx-Tx is measured can be SpCell or SCell or both SpCell and SCell. SRS are typically transmitted on SpCell, which is also typically the DL reference cell for UE timing. So based on option 2, if there is autonomous timing adjustment (on SpCell) then the UE Rx-Tx accuracy for UE Rx-Tx measurement will also apply on SCell. But UE Rx-Tx accuracy for UE Rx-Tx measurement only on SpCell should apply. Compromise option: UE UL timing follows the DL timing of the DL reference cell. So in summary UE Rx-Tx measured in reference cell is fine even under autonomous timing adjustment. For other cells there is misalignment between SRS and PRS timings and in those cells the accuracy should not apply. We therefore propose the following compromise option: - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1), if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| Intel | The compromised option from E/// can be fine for us. |
| Qualcomm | We still think option 1 is preferable. We will consider the other options but we are not convinced that the distinction made here between the serving cell and neighbor cells is valid. |
| Huawei | We agree with the issue identified by Ericsson on option 2, and we are fine to work based on the compromise option from Ericsson. We suggest to further add clarification that the reference cell is for SRS transmission: - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. |
| vivo | From UE Rx-Tx time difference measurement definition, it is not clear how the UE transmit timing of uplink subframe is assumed. The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. For neighbor cell measurements, it is not clear which serving cell’s uplink timing should be used to calculate UE Rx-Tx time difference if there are multiple serving cells with different uplink timing. One option would be based on uplink timing of the serving cell on which the SRS is transmitted. But how to decide if SRSs are also transmitted on multiple serving cells. What if there are multiple TAGs. We think this needs further study and definition in TS38.215 can be further clarified if necessary. |
| Ericsson2 | We are fine with the updated version from Huawei to include “SRS transmission”. To QC: With option 1, the UE measures also on a cell which is not the timing reference cell (Cell2) even if the UE’s SRS Tx timing changes due to autonomous adjustment in DL reference cell (Cell1). But in this case there is mismatch between SRS Tx timing and RX timing in Cell2. This timing mismatch leads to error in the UE Rx-Tx measurement. To Vivo: In one TAG, there can be multiple serving cells. But there is only ONE UL timing for all cells in one TAG. For UE Rx-Tx time difference measurement on a neighbor cell of a serving carrier (F1), the SRS Tx timing in a TAG to which F1 belongs will be used. I guess the scenario you refer to is UE Rx-Tx time difference measurement on a cell of a non-serving carrier (F2)? If F2 is FR1 then SRS timing of FR1 TAG will be used and F2 is FR2 then SRS timing of FR2 TAG will be used. In rare case there can be separate TAGs within the same FR e.g. pTAG on lower bands and sTAG on mid bands. In our understanding the PRS and SRS relation/association for UE Rx-Tx might be possible with LPP assistance signaling. We do not think any clarification is needed in the definition in TS 38.215. In any case, regardless of how the relation is set, there is no impact on the applicability of UE Rx-Tx time difference accuracy requirements under autonomous timing change. |

## CRs/TPs

[Moderator notes: suggest take one of these CR drafts as the baseline which can be revised in 2nd round discussion.]

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117347 CATT DraftCR for PRS RSRP accuracy requirements | Intel: up to 2-5. And can be merged with other draft CRs for RSRP accuracy (R4-2117347 , R4-2117500, R4-2117758, R4-2118940) |
|  | Qualcomm: Pending issue 2-5. |
| R4-2117500 Qualcomm Incorporated Draft CR for relative PRS RSRP | Intel: the correction is not exact aligned with the agreement before [R4-2108299]: - “Relative PRS-RSRP accuracy requirements apply for the cases when PRS-RSRP is measured from resources in the same resource set, and PRS-RSRP is measured with same Rx beam in case of FR2. “ And can be merged with draft CRs for RSRP accuracy (R4-2117347 , R4-2117500, R4-2117758, R4-2118940) |
|  | Qualcomm: To Intel: We would like to clarify the applicability of the requirements to FR1. Do you agree that the requirements apply to FR1 under the same assumptions? We are open to working on a different wording. |
|  | Huawei: We can support the clarification in this CR. |
|  | CATT: the following note need clarification, we don’t think we have this agreement before. - Note: The UE is assumed to use one Rx beam in FR1. |
| R4-2117758 Vivo DraftCR for PRS RSRP accuracy requirements | Intel: up to 2-5. And can be merged with other draft CRs for RSRP accuracy (R4-2117347 , R4-2117500, R4-2117758, R4-2118940) |
|  | Qualcomm: Requirements for extreme conditions pending issue 2-5. |
| R4-2118380 OPPO DraftCR for UE Rx-Tx time difference accuracy requirements | Intel: can be merged with R4-2119333 ,R4-2118380 , R4-2119459 |
| R4-2118940 ZTE [draft CR] accuracy requirements for PRS-RSRP | ZTE: depends on outcome of subtopic 2-5. If extreme conditions are agreed then the draft CR can be endorsed. |
|  | Intel: up to 2-5. And can be merged with other draft CRs for RSRP accuracy (R4-2117347 , R4-2117500, R4-2117758, R4-2118940) |
| R4-2119333 Huawei, Hisilicon draftCR for RSTD and UE Rx-Tx time difference | Intel: up to issue 2-1,2-2, 2-3. For UE Rx-Tx time part can be merged with R4-2119333 ,R4-2118380 , R4-2119459 |
|  | Qualcomm: Pending issues 2-1, 2-2, 2-3 and 2-6. |
| R4-2119459 Ericsson draftCR for UE Rx-Tx time difference | Intel: can be merged with R4-2119333 ,R4-2118380 , R4-2119459 |
|  | Qualcomm: Pending issue 2-6. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Group delay calibration margin for RSTD Tentative agreements: - in FR1 when the reference and target PRS resources are in the same PFL (single PFL case) PRS BW (MHz); Margin (Tc). ≥ 5; [TBD]. ≥ 10; [TBD]. ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. - in FR2 when the reference and target PRS resources are in the same PFL (single PFL case). FFS if additional margin should be added for OTA calibration. PRS BW (MHz); Margin (Tc). ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. ≥ 200; [TBD]. Candidate options: Recommendations for 2nd round: Check the tentative agreements above |
| Sub-topic#2-2 | Group delay calibration margin for UE Rx-Tx time difference Tentative agreements: - in FR1 PRS BW (MHz); Margin (Tc). ≥ 5; [TBD]. ≥ 10; [TBD]. ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. - in FR2 . FFS if additional margin should be added for OTA calibration. PRS BW (MHz); Margin (Tc). ≥ 20; [TBD]. ≥ 50; [TBD]. ≥ 100; [TBD]. ≥ 200; [TBD]. Candidate options: Recommendations for 2nd round: Check the tentative agreements above |
| Sub-topic#2-3 | Frequency drift margin for RSTD Tentative agreements: Candidate options: - Option 1 (vivo): fixed value 0.5Ts (32Tc) in both FR1 and FR2 - Option 2(Intel, Huawei, Qualcomm): The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells. - Option 2a (Qualcomm) A frequency margin is 50 Tc for RSTD if a maximum time offset of 1 second between the PRS resource instances used to calculate the RSTD measurement. - Option 2b(Huawei): the frequency drift margin is +/-32Tc based on maximum offset of 160ms Recommendations for 2nd round: Further discussion is needed. Could we agree the principle firstly? Please check the proposal below is acceptable? - The maximum frequency drift between the measured TRPs can be dependent with the maximum time offsets among the measured TRPs/cells. - FFS on the exact value |
| Sub-topic#2-4 | Frequency drift margin for RSTD Tentative agreements: Candidate options: - Option 1 (Qualcomm): FFS on whether to add a constant margin to account for frequency error. Recommendations for 2nd round: Further discussion is needed. Could we agree the principle firstly? Please check the proposal below is acceptable? “The frequency drift margin for UE Rx-Tx is NOT needed” |
| Sub-topic#2-5 | RSRP requirements under extreme conditions Tentative agreements: - The PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - [1.5]dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2 Candidate options: Recommendations for 2nd round: Check whether the tentative agreements are agreeable. |
| Sub-topic#2-6 | Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment Tentative agreements: - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. Candidate options: Recommendations for 2nd round: Check whether the tentative agreements are agreeable as the harmonization results.. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117347 CATT DraftCR for PRS RSRP accuracy requirements | Merged |
| R4-2117500 Qualcomm Incorporated Draft CR for relative PRS RSRP | Merged |
| R4-2117758 Vivo DraftCR for PRS RSRP accuracy requirements | Revised |
| R4-2118380 OPPO DraftCR for UE Rx-Tx time difference accuracy requirements | Merged |
| R4-2118940 ZTE [draft CR] accuracy requirements for PRS-RSRP | Merged |
| R4-2119333 Huawei, Hisilicon draftCR for RSTD and UE Rx-Tx time difference | Revised For UE Rx-Tx time part can be merged with R4-2119333 ,R4-2118380 , R4-2119459 |
| R4-2119459 Ericsson draftCR for UE Rx-Tx time difference | Revised |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

|  | Status summary |
| --- | --- |
| Sub-topic#2-5 | RSRP requirements under extreme conditions - FFS on The PRS RSRP measurement requirements in extreme condition are X dB larger than that in normal condition, and X is: - 3dB for absolute accuracy for FR1. - 3dB for absolute accuracy for FR2. - [1.5]dB for relative accuracy for FR1. - 3dB for relative accuracy for FR2 QC: Don’t see much value in defining these requirements. Low priority or FFS. |
| Sub-topic#2-6 | Applicability of UE Rx-Tx time difference accuracy requirements under TA adjustment FFS on : - Option 1. - UE Rx-Tx measurement accuracy requirements shall apply if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment - Option 2. - UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. - the UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment. QC: The proposed agreement invalidates measurement accuracy for all TRPs except for the serving cell. (which may or may not be a TRP in the assistance data). How is multi-RTT supposed to work under this proposal? We support option 1 added below. Moderator: As no consensus on this issue, we can FFS in the next meeting |

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2120271 | agreeable |
| R4-2120272 | agreeable |
| R4-2120273 | return |

# Topic #3: Test cases (AI5.1.4.2.1.3)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2119262 | Rohde & Schwarz | Proposal 1: RAN4 to add UE-based DL-TDOA reporting delay test case requirements based on the existing RSTD reporting delay test cases, but adding the needed pre-requisites. Proposal 2: RAN4 to consider the definition of DL-TDOA accuracy test case requirements for UEs supporting UE-based DL-TDOA based on one of the following alternatives: - Alternative 1: considering the worst case RSTD absolute accuracy from TS 38.133 section 10.1.23 (e.g. Expected RSTD - K for the serving cell, Expected RSTD + K for all neighbor cells, where K is the accuracy limit for the corresponding PRS configuration), calculate the total positioning error that results. - Alternative 2: consider the E911 requirements as basis and define a positioning accuracy error of +-50m |
| R4-2117349 | CATT | DraftCR on RSTD and UE Rx-Tx test cases |
| R4-2117350 | CATT | DraftCR on RSTD and UE Rx-Tx test cases |
| R4-2119335 | Huawei, Hisilicon | CR to update TCs for PRS measurement requirements |
| R4-2119336 | Huawei, Hisilicon | CR to update TCs for PRS measurement requirements R17 |
| R4-2119337 | Huawei, Hisilicon | CR to update TC for PRS measurement accuracy |
| R4-2119338 | Huawei, Hisilicon | CR to update TC for PRS measurement accuracy R17 |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 3-1 Test cases for the UE-based positioning measurement delay reporting

- Option 1 (R&S)

  - RAN4 to add UE-based DL-TDOA reporting delay test case requirements based on the existing RSTD reporting delay test cases, but adding the needed pre-requisites.

Recommended WF: Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| Intel | Can be FFS. But in our understanding, for the requirements testing itself the UE assistant method is more sufficient. |
| Qualcomm | In general, we would support testing for UE-based positioning. Here we understand that the UE would be reporting the calculated positioning fix. |
| Huawei | We do not support option 1. RAN4 has disused the requirements for UE based positioning, and there was no consensus to define them. Given that there is no requirements, we do not think test cases can be defined. |
| CATT | Firstly, we share the same view as Huawei that we don’t have UE based requirements. secondly, we would like to understand when we define measurement delay requirements test case, what’s the difference between UE-assistant positioning and UE based positioning. Because in our understanding, the difference between the two approaches is the calculation unit, but the measurement should be same. |
| R&S | Thanks for the feedback: To Intel and CATT: We agree that testing the performance once (assisted or based) per device is sufficient. But the problem is that both methods (assisted and based) are optional. This means that for UEs supporting only UE based positioning (and not UE assisted positioning) there is no requirement and test coverage and they will remain completely untested. Is this intended? What are then the requirements for these devices and how will they be certified? |

### Sub-topic 3-2 Test cases for the UE-based positioning measurement accuracy requirements

- Option 1 (R&S)

  - RAN4 to consider the definition of DL-TDOA accuracy test case requirements for UEs supporting UE-based DL-TDOA based on one of the following alternatives:

    - Alternative 1: considering the worst case RSTD absolute accuracy from TS 38.133 section 10.1.23 (e.g. Expected RSTD - K for the serving cell, Expected RSTD + K for all neighbor cells, where K is the accuracy limit for the corresponding PRS configuration), calculate the total positioning error that results based on multilateration algorithm.

    - Alternative 2: consider the E911 requirements as basis and define a positioning accuracy error of +-50m

Recommended WF: Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| Intel | Same vies as 3-1 |
| Qualcomm | See our response to 3-1. |
| Huawei | Same comment as for 3-1. |
| CATT | Same as 3-1. |
| R&S | Same as 3-1 |

### Sub-topic 3-3	 TC of RSTD accuracy with dual PFLs

- Option 1 (Huawei):

  - RAN4 to remove the test cases for RSTD accuracy with dual PFLs.

Recommended WF: Further discussion needed. Collect companies’ views

| Company | Comments |
| --- | --- |
| Intel | Option 1 is fine for us |
| Qualcomm | We prefer to wait until the conclusion of the applicability issues for RSTD. |
| Huawei | Support option 1 based on our proposal for 2-3. |

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2117349 CATT DraftCR on RSTD and UE Rx-Tx test cases | Intel: R4-2119337, R4-2117349 can be merged |
|  | Huawei: To moderator, we understand 7349 and 9335 are both for measurement delay TCs for RSTD and Rx-Tx, while 9337 is for accuracy TCs. Would it be possible to split the work between 7349 and 9335? For example, we can capture all agreeable changes to RSTD delay TCs in 9335 and all agreeable changes to Rx-Tx delay TCs in 7349. |
|  | CATT: fine with Huawei’s suggestion. |
| R4-2119335 Huawei, Hisilicon CR to update TCs for PRS measurement requirements | Intel: check whether need to combined with other draft CR or use the single CR to correct the common issues. |
|  | Huawei: please see our comments above to 7349. |
| R4-2119337 Huawei CR to update TC for PRS measurement accuracy | Intel: up to 3-3. |
|  | Qualcomm: Pending issue 3-3. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Test cases for the UE-based positioning measurement delay reporting Tentative agreements: Candidate options: - RAN4 to add UE-based DL-TDOA reporting delay test case requirements based on the existing RSTD reporting delay test cases, but adding the needed pre-requisites. Recommendations for 2nd round: Further discussion needed. The proponents can provide more inputs on the feasibility of testing without requirements for themselves. |
| Sub-topic #3-2 | Test cases for the UE-based positioning measurement accuracy requirements Tentative agreements: Candidate options: - RAN4 to consider the definition of DL-TDOA accuracy test case requirements for UEs supporting UE-based DL-TDOA based on one of the following alternatives: - Alternative 1: considering the worst case RSTD absolute accuracy from TS 38.133 section 10.1.23 (e.g. Expected RSTD - K for the serving cell, Expected RSTD + K for all neighbor cells, where K is the accuracy limit for the corresponding PRS configuration), calculate the total positioning error that results based on multilateration algorithm. - Alternative 2: consider the E911 requirements as basis and define a positioning accuracy error of +-50m Recommendations for 2nd round: Further discussion needed. The same conclusion can be leveraged from issue 3-1. |
| Sub-topic#3-3 | TC of RSTD accuracy with dual PFLs Tentative agreements: Candidate options: - Option 1 (Huawei): - RAN4 to remove the test cases for RSTD accuracy with dual PFLs Recommendations for 2nd round: Further discussion is needed. |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2117349 CATT | Revised (To capture the CR for UE Rx-Tx time difference delay TCs) |
| R4-2119335 Huawei, Hisilicon CR to update TCs for PRS measurement requirements | Revised (To capture the CR for RSTD delay TCs) |
| R4-2119337 Huawei CR to update TC for PRS measurement accuracy | Revised |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

Sub-topic#3-1 Test cases for the UE-based positioning measurement delay reporting

Recommendations for 2nd round: Further discussion needed.

The proponents can provide more inputs on the feasibility of testing without requirements for themselves.

| Company | Comments |
| --- | --- |
| Huawei | As we commented in the first round, there is no requirement for UE based positioning, so we do not see why RAN4 should define test cases. |
| R&S | If we understand correctly the purpose is here to provide feasibility of testing positioning fix (UE based reports) delay and accuracy requirements, by deriving the requirements from the RSTD (UE assisted reports) delay and accuracy requirements. If this understanding is correct, we will try to provide this in the next RAN4 meeting. [Moderator: Yes. We are open for this discussion with more analysis. ] |

Sub-topic#3-3 TC of RSTD accuracy with dual PFLs

Recommendations for 2nd round: Further discussion needed.

| Company | Comments |
| --- | --- |
| Qualcomm | Suggest to postpone discussion of this issue until the applicability of the RSTD frequency drift margin is finalized. |

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |
| R4-2120274 | agreeable |
| R4-2120275 | agreeable |
| R4-2120276 | agreeable |

# Topic #4: gNB Requirements (AI5.1.4.2.2.)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2119400 | Nokia, Nokia Shanghai Bell | Draft CR: Correction to Ês and Iot symbol definitions for gNB measurements |
| R4-2119401 | Nokia, Nokia Shanghai Bell | Correction to Ês and Iot symbol definitions for gNB measurements |

## Open issues summary and companies’ views collection for 1st round

### Sub-topic 4-1

### CRs/TPs

| CR/TP number | Comments collection |
| --- | --- |
| R4-2119400 Nokia, Nokia Shanghai Bell | Huawei: Fine with the CR. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 |  |

### CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2119400 Nokia, Nokia Shanghai Bell | Agreeable |

## Discussion on 2nd round

Please only comment on topics that are selected for discussion in 2nd round.

## Summary on 2nd round

| CR/TP/LS/WF number | T-doc status update recommendation |
| --- | --- |

# Recommendation for Tdocs

After first round:

| Tdoc No. | Source company |  | Recommendation | Remarks |  |
| --- | --- | --- | --- | --- | --- |
| CR for accuracy requirements |  |  |  |  |  |
| R4-2117347 | CATT |  | Merged |  |  |
| R4-2117500 | Qualcomm Incorporated |  | Merged |  |  |
| R4-2117758 | Vivo |  | Revised | To capture CR for PRS RSRP accuracy requirements |  |
| R4-2118380 | OPPO |  | Merged |  |  |
| R4-2118940 [ | ZTE |  | Merged |  |  |
| R4-2119333 | Huawei, Hisilicon |  | Revised | To capture CR for RSTD accuracy requirements |  |
| R4-2119459 | Ericsson |  | Revised | To capture CR for UE Rx-Tx time difference accuracy requirements |  |
| CR for TC |  |  |  |  |  |
| R4-2117349 | CATT |  | Revised | To capture the CR for UE Rx-Tx time difference delay TCs |  |
| R4-2119335 | Huawei, Hisilicon |  | Revised | To capture the CR for RSTD delay TCs |  |
| R4-2119337 | Huawei, Hisilicon |  | Revised | To capture the CR for accuracy TCs |  |
| CR for gNB TC |  |  |  |  |  |
| R4-2119400 | Nokia, Nokia Shanghai Bell |  | Agreeable |  |  |
| WF |  |  |  |  |  |
| New | Intel |  | Return |  |  |

After 2nd round:

| Tdoc number | Revision from | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2120271 | R4-2117758 | Draft CR to 38.133 correction to NR positioning accuracy requirements | Vivo | Agreeable |  |
| R4-2120272 | R4-2119333 | CR on accuracy requirements for positioning measurement | Huawei, Hisilicon | Agreeable |  |
| R4-2120273 | R4-2119459 | Accuracy requirements for UE positioning measurements in TS 38.133 | Ericsson | Return to | Had not consensus on the applicability (issue 2-6) |
| R4-2120274 | R4-2117349 | DraftCR on RSTD and UE Rx-Tx test cases | CATT | Agreeable |  |
| R4-2120275 | R4-2119335 | CR to update TCs for PRS measurement requirements | Huawei, Hisilicon | Agreeable |  |
| R4-2120276 | R4-2119337 | CR to update TC for PRS measurement accuracy | Huawei, Hisilicon | Agreeable |  |
| R4-2120270 |  | WF | Intel | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents