3GPP TSG-RAN WG4 Meeting #101-e		R4-2120707

E-meeting, 1 November – 12 November, 2021

Agenda item:	8.12.2.1.1

Source:	Intel Corporation

Title:	WF on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC

Document for:	Approval

# Introduction

This document provides the way forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC.

# Common test parameters

## Background (agreements from #98-bis-e – #100-e meetings)

- Physical cell ID of 0 for the serving cell, and cell ID i for interferer i (where i ≥ 1, maximum number of i is FFS)

- SCS: FDD 15kHz, TDD 30kHz (Same SCS is used for the target and the interference cells)

- TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Channel bandwidth: RAN4 introduce test case under 10MHz for FDD 15kHz and 40MHz for TDD 30kHz only with assumption that interference-plus-noise covariance estimation granularity does not exceed the PRB bundling size regardless the channel bandwidth from RAN4 requirements introduction perspective.

- Number of carriers: In Rel-17, MMSE-IRC receiver performance requirements with interference cell condition is defined only for single carrier scenario

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain: Full PRB

- DMRS configuration

  - For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interferer

- TRS/CSI-RS configuration

  - TRS/CSI-RS colliding with TRS/CSI-RS interference

- Antenna configuration

  - Tx as baseline for serving cell and interfering cells

  - UE with 2 and 4 RX

  - ULA low correlation as baseline

- Test setup methodology for signal powers: Use SNR and INR methodology for signal powers setup

## Network type

- Previous meeting status

  - Synchronized for FDD and TDD

  - FFS asynchronized for FDD

- Way forward

  - Synchronized network is baseline assumption, interested companies are encouraged to bring results for async scenario under the baseline assumption of MMSE-IRC receiver.

    - For asynchronized scenario, reusing LTE configuration of time/frequency offset as starting point.

## SSB configuration

- Previous meeting status

  - Option 1: All SSBs (serving cell and interference cell(s)) are in the same time/frequency resources

  - Option 2: Serving cell SSB and interference cell(s) SSB(s) are in the different time/frequency resources

  - Interested companies can check the PBCH performance for both options

    - Use simulation assumptions from Section 5.4 of TS 38.101-4 for serving cell

    - Use the following SSB configuration for Option 2

      - SSB of serving cell is transmitted in PRB 0~19 in first occasion

      - For case with 1 interference cell (if agreed), SSB of interference cell is transmitted in PRB 0~19 in second occasion.

      - For case with 2 interference cell, SSB of interference cell 1 is transmitted in PRB 20~39 in first occasion, SSB of interference cell 2 is transmitted in PRB 0~19 in second occasion.

    - Other simulation assumptions are FFS

- Way forward

  - Option 1: Use SSB Option 1 for all test

  - Option 2: Use SSB Option 2 for all test

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: SSB Option 1 for homogeneous deployment assumptions and SSB Option 2 for heterogeneous deployment assumptions

    - Option 3B: SSB Option 2 for homogeneous deployment assumptions and SSB Option 1 for heterogeneous deployment assumptions

## Propagation condition

- Previous meeting status

  - Consider TDLA30-10 and TDLC300-100 channel models for evaluation purpose and select only one for requirements definition

  - Interested companies are encourage to provide views on down-selection criteria

- Way forward

  - Use TDLC300-100 for homogeneous deployment assumptions and TDLA30-10 for heterogeneous deployment assumptions

# Target PDSCH parameters for scenario 1

## Background (agreements from #98-bis-e – #100-e meetings)

- Transmission rank: Use rank 1 as baseline

- PRB bundle size: Set PRB bundle size as 2 for target PDSCH

- HARQ process number: 4 for FDD 15kHz SCS and 8 for TDD 30kHz SCS as baseline

- Precoding model: Random precoder with Type I SP codebook

## MCS

- Previous meeting status

  - Down selection between MCS 4 and MCS 13 based on results for agreed INR values based on the following criteria

    - Option 1: Testable performance benefit (i.e. > 1 dB) of MMSE-IRC vs MMSE-MRC

    - Option 2: SINR is not lower than -6 dB

    - Option 3: Consider the difference between SNR and INR to avoid possible handover (SNR-INR > -3dB)

    - Option 4: SNR > INR

    - Other options are not precluded

    - Using of multiple options is not precluded

  - Interested companies are encouraged to provide views on criteria for MCS down selection

- Way forward

  - Use MCS 13 for homogeneous deployment assumptions

  - Use MCS 13 as baseline for heterogeneous deployment assumptions

    - It can be revised in the next RAN4 meeting in case any technical issue is raised

# Interference model for scenario 1

## Background (agreements from #98-bis-e – #100-e meetings)

- Transmission rank of interfering PDSCH

  - Use the following assumptions initial simulation purpose: random rank with 70% and 30% probability for rank 1 and rank 2 transmission in the interfering cell(s)

- Precoding of interfering PDSCH

  - Random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2.

- Modulation order of interfering PDSCH

  - Use 16QAM randomly modulated symbols for initial simulation assumption and other options not precluded.

## Deployment

- Previous meeting status

  - Consider Homogeneous deployment assumptions

  - FFS whether for consider HetNet deployment assumptions

- Way forward

  - Introducing test cases with different parameters for Homogenous scenario and HetNet scenario with minimized test cases:

    - One test case applied for Homogenous for each duplex mode and 2Rx/4Rx

    - One test case applied for HetNet for each duplex mode and 2Rx/4Rx

    - If UE supporting both TDD and FDD with same Rx number, UE will pass test case under homogenous scenario with FDD mode, and pass test case under HetNet scenario with TDD mode

## INR values for Homogeneous deployment assumptions

- Previous meeting status

  - Further discuss the following options for PDSCH requirements definition for synchronous network

    - Option 1: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

    - Option 2: INRs 7.77 and 2.29 dB in case of 2 interference cells and FFS in case of 1 interference cell

    - Option 3: INRs 13.91 and 3.34 dB in case of 2 interference cells and FFS in case of 1 interference cell

    - Other options are not precluded

  - FFS assumptions for asynchronous network

- Way forward

  - Further discuss the following options for PDSCH requirements definition for synchronous network

    - Option 1: INRs 7.77 and 2.29 dB in case of 2 interference cells and INR 5.49 dB in case of 1 interference cell

    - Option 2: INRs 5.43 and -1.50 dB in case of 2 interference cells and INR 3.1 dB in case of 1 interference cell

  - FFS assumptions for asynchronous network

## INR values for HetNet deployment assumptions

- Previous meeting status

  - Option 1: INRs 11.39 and 5.45 dB (DIPs -1.23 and -7.16 dB)

  - Other options are not precluded

- Way forward

  - Baseline option: INRs 11.39 and 5.45 dB in case of 2 interference cells and INR 4.84 dB in case of 1 interference cell.

  - Baseline option can be updated in case technical issue will be observed

## Number of explicitly modeled interference cells

- Previous meeting status

  - Companies are encouraged to check performance with 1 and 2 interference cells for initial simulations

  - Further discuss the assumptions for requirements definition

- Way forward

  - Option 1: 1 interference cell for all tests

  - Option 2: 2 interference cells for all tests

  - Option 3: Use different assumptions for different deployment scenarios:

    - Option 3A: 2 interference cell for homogeneous deployment assumptions and 1 interference for heterogeneous deployment assumptions

    - Option 3B: 1 interference cell for homogeneous deployment assumptions and 2 interference for heterogeneous deployment assumptions

## Time and frequency offsets for synchronized network

- Way forward

  - FDD 15 kHz

    - Time offset: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

    - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

  - TDD 30 kHz

    - Time offset:

      - Option 1: The serving cell is 3 us for interfering cell 1 and -1 us for interfering cell 2 (in case modeled)

      - Option 2: The serving cell is 1 us for interfering cell 1 and -0.25 us for interfering cell 2 (in case modeled)

      - Other options are not recluded

    - Frequency shift: The serving cell is 300 Hz for interfering cell 1 and -100 Hz for interfering cell 2 (in case modeled)

## Modelling assumptions

- Way forward

  - Interfering cell has 2Tx and random precoding with PRB granularity of 2.

  - Interfering cell has same channel model as for serving cell and it is independently generated for each cell.

  - Interfering cell has full PDSCH allocation on all slots where serving cell has PDSCH grant.

# Receiver assumptions

## TRS-IC/IM processing

- Previous meeting status

  - Further discuss whether to add in the simulation assumptions the clarification that no TRS interference cancellation/mitigation is considered for inter-cell MMSE-IRC requirements definition

  - No such clarification will be added in the TS 38.101-4

  - Interested companies are encouraged to check with performance for scenarios with and without TRS-IC/IM

- Way forward

  - Keep previous meeting agreement

# Release independence

- Way forward

  - RAN4 discuss whether the UE demodulation with inter-cell interference is released independent from Rel-15 or not, after RAN4 agree with the detailed simulation assumption

# CR work split

Agreement on CR work split

| Topic | Section | Company |
| --- | --- | --- |
| Demodulation requirements | General and applicability section | Apple |
|  | PDSCH requirements - FDD | Apple |
|  | PDSCH requirements - TDD | CMCC |
|  | Annex B: Interference modelling assumptions | Nokia |
| CSI requirements | General and applicability section | Huawei |
|  | CQI requirements - FDD | Qualcomm |
|  | CQI requirements - TDD | Ericsson |

# References

- R4-2115730 “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation, RAN4 #100-e, August 2021.

- R4-2108664 “Way Forward on general and PDSCH demodulation requirements for inter-cell interference MMSE-IRC”, Intel Corporation, RAN4 #99-e, May 2021.