3GPP TSG-RAN WG4 Meeting # 101-e                                                Rev of R4-2118552

Electronic Meeting, 1– 12 November, 2021

Source:	Huawei, HiSilicon, AT&T

Title:	TP for TR 36.717-02-01 CA_38A-66A and CA_38C-66A

Agenda item:	10.1.2

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_38A-66A and CA_38C-66A as defined in the revised WID [1].

Reference

[1]	RP-211859, “Revised WID: Rel17 LTE inter-band CA for 2 bands DL with 1 band UL”, Qualcomm Incorporated

Text Proposal

---Start of changes---

5.x	CA_38-66

5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_38A-66A | - | 38 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_38C-66A | - | 38 | See CA_38C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

5.x.2 	Co-existence studies

Table 5.x.2-1 and 5.x.2-2 summarize frequency ranges where harmonics and/or harmonics mixing occur for CA _ 2-8.

Table 5.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 |
| 66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 |

Table 5.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 |
| 66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 |

There is no harmonic issues for this band combinations.

5.x.3	∆TIB and ∆RIB values

Referring to DC_66_n38, the ∆TIB and ∆RIB values can be specified as below.

Table 5.x.3-1: IB,c

| CA_38-66 | 38 | 0.5 |
| --- | --- | --- |
|  | 66 | 0.5 |

Table 5.x.3-2: R IB,c

| CA_38-66 | 38 | 0.5 |
| --- | --- | --- |
|  | 66 | 0.5 |

5.x.4 	REFSENS

Referring to the DC_66_n38, the REFSENS exception for CA_38-66 can be specified as below.

Table 5.x.4-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |
| CA_38A-66A CA_38C-66A | 66 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD | 38 |

Table 5.x.4-2: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_38A-66A CA_38C-66A | 38 |  |  | 100 | 100 | 100 | 100 | TDD |

---End of changes---