3GPP TSG-RAN4 #102-e	R4-220xxxx

Electronic Meeting, 21st February – 3rd March 2022

Title:	WF on FR1 UL coherent MIMO

Source:			Anritsu Limited

Agenda Item:			4.1.1.1.

Document for:			Approval

# 1.	Introduction

This Way Forward is intended to facilitate the CR to 38.101-1[1] expected in the coming meetings and covers several points corresponding to the proposals introduced and discussed in the 1st round of RAN4 #102-e [2][3].

# 2.	Topics

# 2.1.	Location of information: Alphabet letter of the Annex

Candidate proposal:

- Proposal 1: Put details regarding UL coherent MIMO requirements in "Annex G (informative): Transmit signal quality".

[Possible] WF recommendation: Approve Proposal 1.

- The submitted CRs should cover details regarding UL coherent MIMO requirements in the currently blank and unnamed Annex G, naming it: "Annex G (informative): Transmit signal quality".

# 2.2.	Channel estimation

The “relative phase error” and “relative amplitude” shall be calculated in frequency domain based on channel estimation.

Candidate proposal:

- Proposal 2: Channel estimation should be used for determining the relative phase and amplitude errors.

[Possible] WF recommendation: Approve Proposal 2.

- The principle of using channel estimation for determining the relative phase and amplitude errors is agreed but details need to be given in submitted CRs.

# 2.3.	Symbols used for calculation

Either DMRS only symbols (DMRS resource elements i.e. pairs (DMRS symbol, DMRS subcarrier)) or DMRS + Data symbols can be used as the channel estimation between the two options should be relatively similar.

Candidate proposal:

- Proposal 3: Use DMRS resource elements (DMRS symbol, DMRS subcarrier).

[Possible] WF recommendation: In the CRs submitted one of the two options below should be selected, details should be provided. Choice justification could be either explained in CR “reason of change” or in an accompanying discussion paper particularly if it is not aligned with the related decisions taken in NR_cov_enh [4] and NR_RF_FR2_enh2_Part_2 [5] if any, where related discussions exist.

- Option 1: DMRS REs

- Option 2: DMRS + Data

# 2.4.	Frequency or Time domain type of time measurement (instantaneous, average)

The “relative phase error” and “relative amplitude” shall be calculated in frequency domain for each slot. There should not be then mention of “instantaneous” or “average over a slot”.

Candidate proposal:

- Proposal 4: The “relative phase error” and “relative amplitude” shall be calculated in frequency domain. There should not be then mention of “instantaneous” or “average over a slot” for their calculations.

[Possible] WF recommendation: Approve proposal 4. How from the different values obtained for each slot is a single value per slot determined should be detailed in the CR in agreement with the choice made for “symbols used calculation” (section 2.3. above).

# 2.5.	RF impairment compensation enabled in TE - CFO

The carrier frequency offset (CFO) correction should be enabled and corrected for each slot individually, as if uncorrected it would lead to phase rotation and so phase error.

Candidate proposal:

- Proposal 5: CFO should be corrected for each slot.

[Possible] WF recommendation: Approve Proposal 5. In CR “reason of change” or in an accompanying discussion paper, it should be demonstrated that CFO correction is needed as for JCE [4] even if UL coherent MIMO used 2 Tx chains.

# 2.6.	Equalization

Equalization is not required as the values for amplitude and phase can be determined from the channel estimation and so prior the equalization block.

Candidate proposal:

- Proposal 6: Equalization should not be used by the TE for performing the test.

[Possible] WF recommendation: Approve Proposal 6.

# 2.7.	Block diagram and measurement point

A block diagram inspired of the "Figure F.1-1: EVM measurement points" should be used. It should show the location at which the errors need to be determined either directly or not.

Candidate proposal:

- Proposal 7: A block diagram such as shown in Figure 2 of [2] should be added in Annex G to indicate the reference point.

[Possible] WF recommendation: Approve Proposal 7.

# 3.	References

[1] 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[2] R4-2205610, ‘FR1 UL coherent MIMO’, Anritsu, RAN WG4 #102-e, Feb-Mar 2022

[3] R4-220xxxx, ‘Email discussion summary for [102-e][101] R15_Maintenance’, RAN WG4 #102-e, Feb-Mar 2022

[4] R4-220xxxx, ‘Email discussion summary for [102-e][136] NR_cov_enh’, RAN WG4 #102-e, Feb-Mar 2022

[5] R4-220xxxx, ‘Email discussion summary for [102-e][126] NR_RF_FR2_enh2_Part_2’, RAN WG4 #102-e, Feb-Mar 2022